Jun. 27, 2024

Subject:  MW link installation at Jdeideh 2 MPT

Dear Sir,

Touch seeks your approval to relocate existing microwave link at Jdeideh 2 MPT. 
We will be relocating existing 1.2m MW antenna to the nearby tower to be installed on new H-frame facing Chaat MPT since the current link has been obstructed and affecting the availability of link.

Please find below the required MW link to be installed at Jdeideh 2 MPT site:

| MW link between S62 Jdeideh 2 MPT and S65 Chaat MPT | Jdeideh 2 MPT 34 14 33.30 N 036 22 54.90 E | Chaat MPT 34 08 30.42 N 036 14 06.48 E |
| --- | --- | --- |
| ###### Antenna size | 1.2m | 1.2m |
| Antenna Height (antenna center line) | 20m | 20m |
| Azimuth (DEG) | 230 | 50 |
| Frequency (MHz) | 11323.00 | 10793.00 |
| Polarization | Vertical |  |
| Bandwidth (MHz) | 56 |  |
| Distance (Km) | 17.55 |  |
| Unfaded received signal level (U.R.S.L) | -31.25 dBm |  |
| Channel | CH1 H | CH1 L |

We remain at your disposal for any further information needed.

- Photographs

Existing 1.2m MW antenna

to be relocated to the nearby tower and installed on new

H-frame at 20m height.