# INFRASTRUCTURE & IMPLEMENTATION FILE

#### FOR EXISTING SITE

SITE NAME                    Damour2

SITE NUMBER               S 1339

### Site File Prepared by : Jad Noueilaty

INFRASTRUCTURE OF   “Damour2” SITE

1.  GENERAL

Site Name:	Damour2

Site Number:	S 1339

Surveyed by:	Jad Noueilaty and Sami Fawaz

Location’s Coordinates: 33.718487° 35.452961°

Address: Mount Lebanon, Chouf, Damour

2.  CHANGES

- The site will be configured as 3 sectors instead of 2 sectors.

- The new Sector will be Sector A.

- The site will be relocated to alfa existing tower .

- The new sector will be sector A.

- The old existing Sector A  will become sector B.

- The old existing sector B will become sector C.

- Sector A will be mounted on the tower under Alfa’s antenna on 30 m height as per Fig.1.

- Sector B and C will be mounted on the tower under Alfa’s antenna on 30 m height as per Fig.2.

## 3.   RADIO ANTENNA DATA

New configuration:

| Sector | Antenna Type | Orientation | Mec tilt | G900 Elec tilt | U 900 Elec Tilt | 3G 2100 Elec Tilt | LTE 1800 Elec Tilt | LTE 800 Elec Tilt | Height |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Quad band | 0 | 0 | X | 5 | X | 4 | 8 | 30 m |
| B | Quad band | 150 | 0 | X | 4 | X | 2 | 5 | 30 m |
| C | Quad band | 220 | 0 | X | 6 | X | 4 | 8 | 30 m |

## 4-  Site layout

Sector A will be mounted here under Alfa’s antenna on 30 m height.

Fig.1

Sector B will be mounted here under Alfa’s antenna on 30 m height.

Sector C  will be mounted here under Alfa’s antenna on 30 m height.

Fig.2

Alfa Antenna

## 5.   MICROWAVE DATA

| MW link between S Damour 2 Relocation and S276 Naameh 2. | S Damour 2 Relocation 33 43 05.00 N 035 27 10.34 E |
| --- | --- |
| ###### Antenna size | 0.6m |
| Antenna Height (antenna center line) | 12m |
| Azimuth (DEG) | 13 |
| Band (GHz) | 32 |
| Polarization | Vertical |
| Bandwidth (MHz) | 56 |
| Distance (Km) | 1.4 |
| Unfaded received signal level (U.R.S.L) | -35 dBm |

# Photographs

0.6m MW antenna

installed at 12m height.