Pico repeaters Requirements

RAN – IBS&DAS Unit

Background

After the increment in the demand for low power repeaters to provide coverage in underground areas and in shops/offices where coverage gaps exist; MIC2 is requesting an end-to-end pico-repeater system solution that will repeat both MIC1 and MIC2 signal. On the other hand, MIC1 will order repeaters supporting both operators’ bands as well.

Introduction

This document provides the major requirements in relation to the pico-repeaters specifications and requirements.

It provides an explicit list of the main Radio technical requirements in relation to this order.

The addressed vendor needs to provide a detailed technical solution for each requirement, along with its detailed technical specifications.

Main Technical Requirements

Technical requirements applicable to all provided solutions by the vendor

- The vendor will provide and include in his offer a turnkey solution whereby the vendor will provide the antennas, the pico-repeaters, the cables along with the ancillaries.

- Pico-repeater:

- The repeater unit should be operational on the below bands:

TOUCH Bands

UMTS on 900 Band

UMTS on 2100 Band

GSM on 900 Band

LTE on 1800 Band

LTE on 800 Band

5G on 3500 Band

ALFA Bands

UMTS on 900 Band

GSM 900 on 900 Band

LTE on 1800 Band

UMTS on 2100 Band

5G on 3500 Band

- Repeater Gain (all bands): 60dB<Gain<70dB

- Repeater Downlink Output Power: 10dBm<Po<15dBm

- Repeater Noise Figure should be less than 6dB

- Spurious Emission should meet the ETSI standard

- Group Delay should be less than 7µs

- Repeater to support remote management/monitoring system in addition to hands on training on configuration and troubleshoot

- Repair and return service to be provided by vendor

- Repeater total failure time should be less than 0.005%

- Warranty period of 2 years after delivery

- Equipment to be provided through DDP service including shipment clearance and transportation to warehouse

- Yagi Antenna:

- Antenna Gain: 10dB<G<15dB

- 7/16 Connector

- Antenna should be operational on all previously mentioned bands.

- Indoor Antenna:

- Indoor antennas should support the following bands: 800 MHZ, 900 MHZ, 1800 MHZ, 2100 MHZ, 3500 MHZ.

- N-Type Connectors

- Two types of indoor antennas should be provided:

  - Omni-directional antenna (Ceiling mount)

  - Directional panel antenna with gain between 5dB and 7dB (Wall mount)

- Antenna Pattern should be provided as part of the datasheet

- Splitters:

- Splitters should support all abovementioned bands.

- Two way splitter

- Three way splitter

- Four way splitter

- N-Type Connectors

- Max power should be greater than 20dBm

- Insertion loss < 0.05dB

- Cables:

- ½” Flex cables with a maximum loss of 12dB/100m

- Connectors:

- 7/16 Connectors

- N-Type Connectors

| Item | Number |
| --- | --- |
| Pico-repeater | 40 unit |
| Yagi Antennas | 40 |
| Omni-directional Antenna | 80 |
| Panel Antenna | 25 |
| Two way splitter | 25 |
| Three way splitter | 15 |
| Four way splitter | 8 |
| ½” Cable | 3000 meters |
| 7/16 Connector | Needed QTY for the solution |
| N-Type Connector | Needed QTY for the solution |

- In addition to the above requirements, all vendors should provide:

- List of available references for commercial deployment of pico-repeaters (more than two years deployed).

- The source supplier’s company profile, including but not limited to:

  - Last five years financial growth

  - Product line

  - Number of employees

  - Customers List

- Repeater’s Filters Specifications

- Detailed repeater specifications list including but not limited to:

  - Power Consumption

  - Third order intercept point

  - Noise  Figure

  - Delay

  - Size and weight requirements

- Technical datasheets for all items including electrical and mechanical specifications.

Killing factors:

- Pico-repeaters and all devices should be operational on all provided bands

- Hands on training on configuration and troubleshoot in case features available

- Vendor should have at least 5 commercial references including Europe/usa/gulf countries

- Warranty should be not less than 2 years after delivery

- A demo unit of each of the proposed items is mandatory for testing purposes to be provided by winner (i.e. Pico-repeater, Yagi, indoor antennas,...). Demo units that fail MIC2 testing procedures and quality control will be disqualified in the evaluation phase.