MIC2

| GENERATOR OIL SPECIFICATION | ## Ref. Core, RSC | Page: 1 |
| --- | --- | --- |

DESCRIPTION FOR OIL 15/40 CI4

- Oil should be suitable for all diesel engines four stroke models

- Oil should be efficient to maintain prolong engines life in all areas of Lebanon

- Oil performance should be excellent to reduce engines maintenance costs

- Vendor must provide a new certificate of a test result for the oil brand from the “Industrial Research Institute” at the Lebanese University dated after the RFP issuance, where the certificate by the lab should also confirm that the sample received was sealed and opened during the test.

- Viscosity @ 40 degrees above 110, noting that the bidder will be disqualified if the value drops below 90

- Viscosity @ 100 degrees above 14 noting that the bidder will be disqualified if the value drops below 13

- Sulfated Ash below 1.0 noting that the bidder will be disqualified if the value increases above 1.5

- Total base number –TBN above 11 noting that the bidder will be disqualified if the value drops below 9

- Flash point above 230

- Pour point below -30

- Certificate of country of origin should be provided by Oil Manufacturer, duly signed and should be Original or a Certified Copy

- Certificate of country of packaging should be provided and should indicate that the packaging is done in the same country of origin. The Certificate of country of packaging could be a packing list showing the source and destination of the packages; however, it should be very recent and it should CLEARLY show the types/brands/capacity of the shipped goods (for e.g. capacity should be 20L, not 200L nor 2000L for each package)

- Oil provided should be API Licensed (EOLCS) and/or have a duly signed Letter of Conformance (LoC) indicating that the oil follows the requirements of the ACEA European Oil Sequences (SAIL-Europe). API and/or LoC should be recently renewed i.e. not expired (Websites of both EOLCS and SAIL will be checked by MIC2 for validation).

- The quantity is 15,000 liters in pails.

- Bidder should arrange the delivery for the complete quantity within 10 days from PO issuance noting that the bidder will be disqualified if they cannot deliver the complete quantity within 15 days max.  the bidder is responsible for the delivery of the pails to MIC2 warehouse