New fiber routers Technical Specification

- Introduction

In its current network, MIC2 has SPO equipment deployed at its core sites to interconnect with MoT/Ogero fiber network. These equipment main role is to aggregate multiple fiber connection from MIC2s side into few fiber uplinks towards MoT/Ogero network.

The purpose of this RFP is to replace these equipment with new L3 routers.

- Main technical requirement

In total, 12 new routers are needed.

Two routers to be deployed in each of the following sites:

- Bir Hassan

- Justice

- Jdeideh

- Saida

- Tripoli

- Zahle

- Routers main technical requirement:

Each new router should be equipped with:

- Redundant Route Processors

- Redundant Fabrics

- Redundantly Power Supply

- A minimum of 2 Line cards to accommodate for the below ports :

|  | Bir Hassan | Jdeideh | Justice | Saida | Tripoli | Zahle |
| --- | --- | --- | --- | --- | --- | --- |
| 10G (MM) | 18 | 18 | 10 | 18 | 10 | 10 |
| 10G (SM) | 2 | 2 | 2 | 2 | 2 | 2 |
| 1G (SM) | 25 | 25 | 25 | 25 | 25 | 25 |
| 1G (MM) | 4 | 4 | 4 | 4 | 4 | 4 |
| 1G (Copper) | 4 | 4 | 4 | 4 | 4 | 4 |

- General requirement

- All product and Software versions to be the latest.

- All routers to be part of the vendor 5G portfolio and future ready for 5G networks.

- Slots occupation should not exceed 50% of the total available slots per chassis.

At least 50% of the usable slots per router should remain free for future needs.

- Switching capacity not less than 600Gbps (one direction)

- Packet forwarding per second not less then 600Mpps

- Should support, for future need, the following interfaces:

  - 100 GE

  - 50 GE

  - 40 GE

  - 25 GE

- Supported synchronization:

  - Synch E

  - IEEE 1588v2

  - G8275.1

  - Class A, B & C

- Reliability:

- NSR (Nonstop routing)for OSPF/RIP/LDP/RSVP-TE/PIM/MSDP/IGMP/ARP/L3VPN/ ISIS/BGP/VLL/VRRP

- Graceful restart: BGP/OSPF/ISIS/LDP/VLL/VPLS/RSVP/L3VPN/PIM

- Bidirectional Forwarding Detection (BFD)

- Fast convergence of BGP/IGP/multicast routes

- Ethernet OAM, Y.1731

- Security:

- ACL-based packet filtering, SNMPv3 encryption and authentication, URPF, GTSM, DHCP snooping, ARP attack defense, and SSHv2

- Security on management layer (Access Lists for controlling Access to the Appliance, Authentication to console via TACACS+ or RADIUS,…)

- AAA integration (Centralized management, Multiple Privilege Levels,…)

- Provided features/Licenses:

  - Layer 2 features:

IEEE 802.1q QinQ, VLAN mapping, IEEE 802.1p, IEEE 802.3ad, IEEE 802.1ab, and STP/RSTP/MSTP, VLAN switching

- IPv4/IPv6:

Static routes and IPv4 dynamic routing protocols, such as RIP, OSPF, BGP, and IS-IS IPv4/IPv6 dual stack

IPv6 static routes and dynamic routing protocols, such as BGP4/BGP4+, RIPng, OSPFv3, and IS-ISv6 IPv6 neighbor discovery, PMTU discovery, TCP6, ping IPv6, tracert IPv6, socket IPv6, and IPv6 policy-based routing

- L2VPN/L3 VPN:

LDP over TE, VPLS/H-VPLS, and VPN policy-based routing, MPLS/BGP L3VPN

- Multicast:

IGMPv1/v2/v3, IGMP snooping, IPv6 multicast, static multicast routing

- QoS:

Multiple service traffic classification methods and identification policies to flexibly differentiate services and ensure fair scheduling

WRED, multi-level HQoS

Support for both DiffServ and IntServ models

- Features and Licenses that are needed to be supported for future 5G readiness but not necessary provided at the time being:

  - Telecom Boundary Clock

  - Segment Routing

SR IPv4, SRV6, SR-TE, SR-BE

  - EVPN

  - Flex E, network slicing

  - IPsec

  - MACsec

  - SDN NetConf YANG and Telemetry for future evolution

- Device latency not to exceed 15 microseconds

- New HW/SW that follows a license subscription model should be offered with at least 3 years license period.

- Guarantee of a minimum 5 years before reaching the end of support date.

- Only DC power supply is to be provided.

- In case the provided Line cards can accommodate for more ports then what is requested, all remaining ports should be provided with their corresponding SFPs. All ports are requested to be populated with their SFP. If this case is applicable, the new additional ports should be equipped with either 10G (MM) and 1G (SM)

- Offered solution should be compatible with current MIC2 platforms (Cisco ASR9K, CRS, ASR900, NCS, Cisco switches and Huawei RTN900) in addition to Ogero’s fiber and IPMPLS network.

- Vendor should propose standard acceptance procedures for the offered solution, to be reviewed and validated by MIC2.

- Vendor shall ensure that the activities related to the scope of this RFP will have no or minimal impact on the current network traffic.

- The Vendor is fully responsible to replace any damaged network devices occurs during the dismantling/implementation process.

- Passive equipment along with their implementation (cabinets, patch panels, patch cords, ODF,…) should be included for all above requested routers and ports

- Implementation, integration and service reshuffle from current equipment to the new routers should be included.

- Custom clearance should be fully handled by the clearance (DDP).

- The Vendor shall be responsible for providing the warehouse for the equipment to be installed throughout the complete project duration

- The Vendor shall be responsible for delivering the equipment to site from their own warehouse

- The PIP time line is completed with the minimum amount of time taking into consideration the latest possible start and finish times for project activities, the uncertainties, the risks and assumptions  (Less than 4 months from PO issuance)

- The offer should include a responsibility matrix clearly splitting the role of the bidder and the operator

- The vendor’s Implementation proposal should cover but not limited to the following: installation, dismantling, swapping, traffic migration, configuration, integration, interoperability…

- Equipment delivery time shall be less than 60 days from  PO issuance

- Proposal includes satisfactory minimum number of resources with their qualifications & proposed organization structure during implementation and support, which is subject to MIC2 approval.

- Spare Parts Management:

  - In case the spares are managed by the vendor:

Vendor to insure hardware availability in Lebanon for all type of equipment and to handle the repair process

- In case the spares are managed by MIC2:

7% of the requested HW should be provided as spares and included in the BoQ. The spares BoQ should be separate from the main BoQ

- Trainings for the proposed solution should be included in the offer (each course should be delivered in 2 sessions)

- Knowledge transfer and provide the appropriate training program to have MIC2 engineers acquiring the needed knowledge and expertise and receiving the relevant hands-on knowledge transfer

- Additional information:

- SM SFP should be 1310nm LC

- MM SFP should be 850nm LC

- Management System

- The vendor needs to include all NMS requirements in his solution and offer covering FM, PM, CM, inventory, security, account management, system diagnostics, software management, and administration parts along with the necessary NBI licenses and features in addition to MML automated commands and inventory features

- Main and Standby NMS should be provided for redundancy.

- Virtual servers are highly recommended, vendor will supply all needed requirements (licenses, resources) to host the solution in Touch’s virtual environment.

- Integration with Touch’s Active Directory.

- Integration with INMS by providing available types of NBIs.

- Customized utilization reports

- Utilization storage granularity:

  - 30 seconds over period of 1 Day

  - 1 minute over period of 1 week

  - 15 minute over period of 1 month

  - 1 hour over period of 6 months

  - 1 day over period of 2 years

- Various KPI measurement such as delay, jitter, and packet loss rate

- Layer 3 end-to-end performance measurement (at IP flow level)

- Support, SLAs and KPIs:

- 3 years support for all above new HW. The warranty period starts after signing the PAC for all the sites.

- Support coverage is 24 hours a day, 7 Days a week including holidays

- Local support team available in Lebanon ready for immediate site intervention if needed.

- Health check Assessment: to audit all configuration changes that occurred on the network and validate it with Cisco Best Practices, Health check to be conducted twice per year.

- All software updates/releases should be included, delivered and installed once available during the support period.

- On-site visits (environmental check and preventive maintenance to be conducted twice a year)

  - System availability time [%] >99.999%

  - For P1 (Critical/Emergency) incidents, response time 1 hour, restoration time 3 hours, and resolution time 6 hours

  - For P2 (Major) incidents, response time 3 hour, restoration time 6 hours, and resolution time 24 hours

  - For P3 (Non Service Impacting) incidents, restoration time 24 hours, and resolution time 5 calendar days

- Documentation

The documentation shall include the following items:

- High level design solution description covering the implementation and the migration procedure

- Data sheets, technical specifications and manuals.

- Roadmap for both Hardware and Software

- List of all the features and licenses supported by the hardware and highlighting which are included in the offer

- References

- Bidder should mention references for the deployment of the proposed routers in a mobile operator environment. Minimum 3 references.

- Details of the reference need to be provided: Operator, Country, type of upgrades.