As part of LTE addition on existing IBS sites, Kindly find below the modifications to be done on the mentioned sites.

Please review from your side which activities can be executed by our implementation team, prior to assigning the remaining scope of work to subcontractors.

- Rotana_Gevinor_IBS (S1617): consists of 3 sectors with a total of 4 RRUs, where Sector B includes two cascaded RRUs.

  - 4 existing 3G 2100 MHz RRU’s to be replaced by 4 LTE 1800 MHz RRUs.

    - Sector 1 RRU location: Floor B4

    - Sector 2 RRUs locations: Floor C-Lev

    - Sector 3 RRU location: Floor F17

- Super_Value_IBS (S1306): consists of 1 indoor sector with 1 RRU (No U900).

  - 1 existing 3G 2100 MHz RRU to be replaced by 1 LTE 1800 MHz RRU.

    - RRUs located on the roof

    - 1 U900 RRU to be added on the indoor sector before 2100 MHz RRU dismantle

- Bank_Audi_Plaza_IBS (S1588): consists of 3 sectors with a total of 5 RRUs, where Sectors B and C includes each two cascaded RRUs (No U900).

  - 5 existing 3G 2100 MHz RRU to be replaced by 5 LTE 1800 MHz RRU.

    - Sector 1 RRU location: ABC-B1

    - Sector 2 RRUs locations: ABC-B1

    - Sector 3 RRUs locations: D-B1

    - 5 U900 RRUs to be added on the indoor sectors before 2100 MHz RRUs dismantle.

- Bank_Audi_Palladium_IBS (S1589): consists of 2 indoor sectors (No U900).

  - 2 existing 3G 2100 MHz RRUs to be replaced by 2 LTE 1800 MHz RRUs.

    - Sector 1 RRU location: B2

    - Sector 2 RRU location: Entrance

    - 2 U900 RRUs to be added on the indoor sectors before 2100 MHz RRUs dismantle.

- Aishti_IBS (S1072): consists of 1 indoor sector.

  - 1 existing 3G 2100 MHz RRU to be replaced by 1 LTE 1800 MHz RRU.

    - Sector 1 RRU location: L6

- Beirut_Tower_IBS (S1367): consists of 5 sectors with a total of 10 RRUs, 2 cascaded RRUs for each sector (No U900).

  - 10 existing 3G 2100 MHz RRU’s to be replaced by 10 LTE 1800 MHz RRUs.

    - Sector 1 RRU location: B1 and F8

    - Sector 2 RRUs locations: F4 and F6

    - Sector 3 RRU location: F9

    - Sector 4 RRUs location: F20

    - Sector 5 RRUs location: F28

    - 10 U900 RRUs to be added on the indoor sectors before 2100 MHz RRUs dismantle.

- Marina_Tower_IBS (S1367): consists of 4 sectors with a total of 8 RRUs, 2 cascaded RRUs for each sector (No U900).

  - 8 existing 3G 2100 MHz RRU’s to be replaced by 8 LTE 1800 MHz RRUs.

    - Sector 1 RRUs location: Ga_GF and Ba_B1

    - Sector 2 RRUs location: F7 and F13

    - Sector 3 RRUs location: F19 and F25

    - Sector 4 RRUs location: Co_B10 cascaded on the same floor.

    - 8 U900 RRUs to be added on the indoor sectors before 2100 MHz RRUs dismantle.

- Bel_Horizon_IBS (S1369): consists of 1 indoor sector.

  - 1 existing 3G 2100 MHz RRU to be replaced by 1 LTE 1800 MHz RRU.

    - Sector 1 RRU location: GF

- AUL_University_IBS (S969): consists of 3 sectors with a total of 3 RRUs (No U900).

  - 3 existing 3G 2100 MHz RRU’s to be replaced by 3 LTE 1800 MHz RRUs.

    - Sector 1 RRU location: Floor B1

    - Sector 2 RRUs locations: Floor B1

    - Sector 3 RRU location: Floor B1

    - 3 U900 RRUs to be added on the indoor sectors before 2100 MHz RRUs dismantle.

- ABC_Verdun_IBS (S1873): consists of 3 sectors with a total of 6 RRUs, 2 cascaded RRUs for each sector (No U900).

  - 6 existing 3G 2100 MHz RRU’s to be replaced by 6 LTE 1800 MHz RRUs.

    - Sector 1 RRU location: Floor B4

    - Sector 2 RRUs locations: Floor B1

    - Sector 3 RRU location: Floor B1

    - 6 U900 RRUs to be added on the indoor sectors before 2100 MHz RRUs dismantle.

- Airport_IBS1 (S1361): consists of 3 sectors with a total of 6 RRUs, 2 cascaded RRUs for each sector.

  - 6 LTE 800 MHz RRUs to be added.

    - Sector 1 RRUs location: F2 and Departure (RRU3 to RRU7 300 meters) (RRU3 to room 75 20 meters)

    - Sector 2 RRUs location: Arrival and Basement (RRU11 to RRU12 300 meters) (RRU12 to room 75 30 meters)

    - Sector 3 RRUs location: Arrival and Basement (RRU10 to RRU13 300 meters) (RRU13 to room 75 30 meters)

- Airport_IBS2 (S1366): consists of 4 sectors with a total of 7 RRUs, 2 cascaded RRUs for sectors A, C and D.

  - 7 LTE 800 MHz RRUs to be added.

    - Sector 1 RRUs location: F3 and F2 (RRU1 to RRU2 15 meters) (RRU2 to room 98 15 meters)

    - Sector 2 RRU location: Departure (RRU4 300 meters)

    - Sector 3 RRUs location: Arrival (RRU8 to RRU9 300 meters) (RRU9 to room 98 15 meters)

    - Sector 4 RRUs location: Departure (RRU5 to RRU6 200 meters)

- ATG_Holcom_IBS (S1613): consists of 2 indoor sectors (No U900).

  - 2 existing 3G 2100 MHz RRUs to be replaced by 2 LTE 1800 MHz RRUs.

    - Sector 1 RRU location: Rooftop

    - Sector 2 RRU location: Rooftop

    - 2 U900 RRUs to be added on the indoor sectors before 2100 MHz RRUs dismantle.