Optical Fiber Materials Specifications

Table of Contents

- SINGLE MODE FIBER OPTICAL CABLE (G.652D)

- FIBER CLOSURE

- ODFs pre-assembled

- SDFs pre-assembled

- SUB-DUCT (1 or 5 or 7-WAY – With Rope Inside)

- Optical Splitter (SLIM)

- FIBER DISTRIBUTION BOX (FDB)

- Specifications for the Outdoor FDT/FAT

- PRE-TERMINATED FIBER BREAKOUT CABLE

- PIGTAIL

- Patch Panel 48 SC/APC SINGLE MODE

TECHNICAL SPECIFICATIONS
1. SINGLE MODE FIBER OPTICAL CABLE (G.652D)

- GENERAL

  - SCOPE

This specification covers the minimum standards and requirements for the construction, properties, testing, and packing of outdoor single-mode optical fiber cables to be used in the telecommunications network of the Republic of Lebanon.

The cables shall be designed to protect the optical fibers against excessive elongation, water ingress, hydrogen and radiation effects, and chemical corrosion, etc., in the actual environmental conditions encountered along the cable route so that the overall segment performance requirements, as specified in this volume, can be met throughout the system design life.

  - INTENDED USE

Cable purchased in compliance with this specification is to be used in the telecommunications network of Lebanon on FTTX networks. The induct cable may be hauled into ducts or subducts of 28 mm outer diameter. The cable is a filled loose tube type. It shall incorporate a nonmetallic moisture barrier.

  - TYPE APPROVAL

Tenderers who have or have not previously supplied under this specification (or who have made changes to prior supplied products) shall submit a product sample for approval. An interim Type Approval may be granted on the basis of a compliance statement and other information from the manufacturer. Approval of a sample shall not be construed as waiving any requirements of this specification.

  - QUALITY ASSURANCE- QUALITY SYSTEM ACCREDITATION

Manufacturers of cables conforming to this specification must show evidence that the cable manufactured according to a Quality System conforming to ISO 9001 or equivalent, which is approved by MIC2.

At the time of the tender, Manufacturers may be required to supply a copy of the Quality Manual, which shall be utilized for the manufacture and delivery of cable, complying with this specification.

MIC2 may require the manufacturer to be accredited to the above standards by either MIC2 personnel or assessors acting on behalf of MIC2.

  - INSPECTION

MIC2 or its authorized representatives may inspect the Tenderer’s facilities at any time for the purpose of Quality Assurance surveillance.

If requested by MIC2, the tenderer shall supply evidence of the quality of raw materials and components used in the manufacturing process.

All cable manufactured to this specification may be inspected and tested by MIC2 to check compliance.

MIC2 has the right to request proof of compliance with this specification, either by witnessing actual performance of this specification's prescribed tests and/or the provisioning of documented test results at the discretion of the inspector.

In the case of a dispute, testing shall be performed by an independent authority at the expense of the tenderer.

  - RESERVED RIGHTS

MIC2 reserves the right to make changes to the specification.

  - PACKING AND MARKING

    - SHEATH MARKING

The method of marking shall not locally decrease the thickness of the sheath. The markings shall be durable and colored white or yellow.

    - LENGTH MARKING

Continuous sequentially numbered length markers shall be placed at regular longitudinal intervals of one meter on the outside sheath.

The number shall not be less than 3.0 mm in height and shall be spaced to produce good legibility.

The accuracy of the length marking shall be within 1%.

An occasional illegible marking is permissible if there is legible reading on either side of it.

The high-count length marker shall be on the outer end of the cable when drummed.

    - APPLICATION MARKINGS: OUTER SHEATH

Duct Cable

The word "DUCT" shall be marked at longitudinal intervals of not more than 1 m.

Cable Transmission Mode

The letters “SMF G.652D” (abbreviation for single mode) shall also be durably marked on the outer sheath.

  - CABLE DRUM LENGTHS

Cable shall be supplied either in nominal lengths of 4000 meters or in alternative lengths specified prior to delivery, but not shorter than the nominal length.

The delivered lengths may not be shorter than the nominal or specified length.

Each length of cable shall be wound on a separate drum.

  - CABLE DRUMS

The drums shall be substantial and constructed so as to prevent damage to the cables during shipment and handling.

The diameter of the drum barrel shall be large enough to prevent damage to the cables during reeling and unreeling. The diameter of the barrel shall not be less than 40 times the outside diameter of the cable.

Lags or other suitable means of protection shall be applied to the drums to prevent damage to the cables during shipment and storage.

Nails and staples used in the construction of the drums must not be placed in a position where they can damage the cables.

The spindle hole shall allow the use of a 75 mm diameter spindle without binding.

The drum size, including lags, shall not exceed 2.72 m in diameter and 1.5 m in width.

For testing purposes, the inner end of the cable shall be recessed into a slot in the drum flange and protected by a metal cover firmly secured to the flange. Alternatively, the inner end may protrude through the inside of the drum via a suitably constructed slot on the outside of the drum flange for a minimum length of 1 meter.

The cable ends shall be securely fastened so as not to protrude beyond any portion of the drum and to prevent the cable from becoming loose during transport.

  - DRUM MARKINGS

FLANGE MARKING

Details given below shall be distinctly marked in a weather-proof material on both outer sides of the drum flanges.

i. MOT/MIC2, Lebanon
ii. Arrow showing the direction the drum shall be rolled
iii. Country of origin
iv. The label, "CAUTION - OPTICAL FIBRE CABLE - NOT TO BE LAID FLAT".
v. Manufacturer's name or trade mark
vi. A mark indicating the location of the inner end of the cable if located internally

MARKING PLATES

Marking plates showing the following information in English shall be securely attached to the outer side of each drum flange.

The numerals in at least one of the languages shall be punched.

- Material specification number, plus DUCT and M-FREE
B. Number of fibers
C. Fiber Type i.e. mode field diameter, cladding diameter
D. Nominal Cable length in meters
E. Gross weight in kilograms
F. Maximum rated tension
G. Reel number
H. Manufacturer’s name
I. Year of manufacture
J. Drum number

  - UPGRADE SCENARIOS FOR CAPACITY EXPANSION

MIC2 intends that the System will start its commercial service with initially equipped capacity, and the capacity will be expanded in accordance with its traffic demand. The Optical Fiber cable shall be capable of being operated at its full design capacity for XGPON and DWDM networks, and its architecture shall allow flexible expansion of equipped capacity.

The Tenderer shall ensure that each Fiber Segment can be upgraded independently of others at any multiples of wavelengths.

  - TECHNICAL DESCRIPTION

The Tenderer shall provide all necessary information on all proposed cable types including cable cross-sections, chemical and physical characteristics of the raw materials. In addition to the physical, mechanical, electrical, and optical characteristics of cables.

    - CABLE TYPES

Cable types for use in various situations are required. The designs of the different types of cable shall ensure that they may be joined together without undue complexity.

    - ASSOCIATED SPECIFICATIONS

The performance shall conform, unless explicitly indicated, to the transmission standards recommended by the ITU-T where applicable as defined in this Specification.

Some Recommendations may presently be under consideration by ITU. The Contractor shall keep informed of the progress of such Recommendations, shall at any time evaluate whether MIC2 be made compliant to these Recommendations, shall evaluate the consequences in terms of cost and delay, and shall propose to the Purchasers System modifications so as to make the System compliant. Such modifications could result in Contract Variations. In case of differences between the present requirement and the ITU-T Recommendations, the former shall prevail.

    - DEFINITIONS AND STANDARD MIC2 CABLE CODES STANDARD MIC2 OPTICAL FIBER CABLE CODES

It shall be possible to identify the individual fibers at any point where the cable is cut and within joints.

Fiber marking shall be consistent throughout the system. Fiber cores and strands and directions shall be distinguished from one another. Where more than one fiber type used in line, they shall also be distinguished from one another. The Tenderer shall provide their fiber identification scheme.

FIRST LETTER

- Optical Fiber

SECOND LETTER = FIRST SHEATH MATERIAL TYPE.

- Polyethylene (polyolefin).

- Halogen-free flame retardant.

SUBSEQUENT LETTERS = OTHER KEY CHARACTERISTICS.

- Filled Cable.

- Moisture barrier/screen.

- Indoor (building) cable.

- Self-supporting (catenary) for aerial cable.

- Polyethylene outer protective sheath.

- Metallic or Non-metallic cable.

TRAILING NUMBER

Number of Fibers/Core Diameter/Cladding Diameter.

EXAMPLE:

FEFN/12/10/125 Polyethylene sheathed, non-metallic cable with 12 fibers with 10μm core diameter and 125μm cladding diameter.

  - DESIGN REQUIREMENTS

    - GENERAL

The cable product sets shall be designed to protect the optical fibers against excessive elongation, water ingress, hydrogen and radiation effects, and chemical corrosion, etc., in the actual environmental conditions encountered along the cable route.

The types of cable shall be suitable for:

- Pulling in sub-ducts or pipes already installed. Sub-ducts and pipes can be made of a variety of materials including high-density polyethylene, PVC, or polypropylene.

Fiber color coding for all cables shall be fully traceable over the entire cable length.

Only qualified Cable Product Sets that meet the requirements of this specification shall be accepted for use in the system.

- The fiber and cable products shall be designed such that the overall performance requirements shall be met throughout the Design Life of the System. The quality and materials and manufacturing process.

All system components shall conform to ITU-T recommendation G.652D.

    - GENERAL INFORMATION

General Information supplied on the Cable Product Set shall contain and not be limited to:

- Detailed dimensions.

- Mechanical characteristics.

- Electrical characteristics.

- Optical characteristics.

    - QUALIFICATION INFORMATION

The Qualification Information supplied on a Cable Product Set shall contain and not be limited to:

a. Where the product has not been qualified:

- Qualification Test Plans and Procedures.

- Qualification Test Specifications.

- The performance criteria and safety margins used to set the specifications.

b. Where previously qualified:

- Qualification Reports.

- Any relevant certification the Tenderer may have received from any qualified body.

- Previous supply record and service record of the product.

If the offered Cable Product Set is different to the samples used for qualifications in design, manufacture, and material, the Tenderer shall provide the full traceability of all design, manufacturing processes, and material changes along with additional test reports and analysis in order to provide justification for the change and the validity of the qualified status. Purchasers reserve the right to reject such a cable product set.

    - MANUFACTURING INFORMATION

Manufacturing Information supplied on the Cable Product Set shall contain and not be limited to:

- Manufacturing Process Flow Diagrams;

- Cable manufacturers and manufacturing locations;

- Production line qualifications and qualification procedures;

- Raw materials;

- Raw materials suppliers qualifications and qualification procedures;

- Operator qualification

This should include information on how and where fiber sets are compiled in order to ensure fiber dispersion limits are achieved.

Where the Cable Product Set is manufactured at multiple sites and / or on plant not used for the manufacture of the qualification samples, the Tenderer shall indicate all the differences between the products.

  - CABLE SPECIFICATIONS

The Tenderer shall provide with their response 2 meter samples of the offered cables.

COMPATIBILITY WITH MIC2 PRACTICES

All cables must be compatible with current MIC2 installation standards and operation and maintenance practices.

UNI-TUBE CABLE PROPERTIES for both non anti rodent (without protection yarns) and anti-rodent cable types (with protection yarns).

(Including sheath strength member)

Fiber Yarns

AERIAL CABLES “Figure 8 shape”

CABLE SIZES

| Cable Size | Number Of Tubes | Fibers Per Tube |
| --- | --- | --- |
| 4 Fibers | 1 | 4 |
| 12 Fibers | 1 | 12 |
| 24 Fibers | 2 | 12 |
| 48 Fibers | 4 | 12 |

CABLE PROPERTIES

| Type |  | Aerial Loose Tube, Self-Support |
| --- | --- | --- |
| Fiber Model |  | G.652D |
| Messenger Wire | Material | Galvanized Steel Wire |
|  | Type | Stranded Steel Messenger |
|  | Number of Wires | 7 |
|  | Diameter(mm)/Wire | 0.8-0.9 |
| Armoring | Type | Armored Loose Tube Self Support |
|  | Material | Steel Tape (if required in BoQ |
|  | Thickness(±0.03mm) | 0.25 |
| Size | Fiber Count | 4F, 12F, 24F, 48F |
|  | Fiber Strands/tube | 12 |

CSM Cable construction design is required for cables with 24-288 F

  - LONG TERM PERFORMANCE REQUIREMENTS

All cable supplied in compliance with this specification shall be capable of withstanding the typical service conditions of Lebanon for a period of Forty years without detriment to the transmission or operation and maintenance characteristics of the cable.

The environmental conditions of Lebanon may include ambient air temperature variations from -15°C to +55°C and the humidity range 0% to 100% while meeting the overall segment performance.

Cable shall be designed, manufactured, and packaged so that exposure to the environmental conditions of Lebanon during storage, transport, installation, and operation and the environmental conditions to be expected during the storage and transport of cable outside Lebanon shall not degrade the physical, transmission, or operation and maintenance characteristics of the cable.

The cables should be capable of being handled and deployed over the temperature range -10°C to +55°C without causing permanent impairment to their specified performance.

The cables should also be capable of being stored or transported at least over the temperature range -10°C to +55°C without causing permanent impairment to their specified performance.

The Tenderer shall state the recommended storage conditions for long-term storage of the cables, in terms of permissible ranges of temperature, humidity, and any other relevant environmental factors.

  - CABLE CONFIGURATION (for cables 24-288F)

    - The cable shall be a loose tube design, i.e. the fibers are contained in loose tubes. The cable design from center to the outer sheath shall conform to the following conﬁguration:

    - A central strength member (CSM) or an anti-buckling element.

    - A sheath strength member is required for unitube cable construction with <=12F.

    - A sheath over the CSM, if necessary, to increase the diameter.

    - Color coded loose tubes containing ﬁlling compound and optical fibers

    - The loose tubes are assembled around the CSM in a single layer, for 4F to 144F, and in two layers for 288F with reverse oscillating method SZ stranding. This layer may also incorporate ﬁller units, if required. The ﬁller units shall be colored differently to the loose tubes.

    - Cable shall be Jelly-ﬁlled to block water migration along the cable.

    - Wrapping yarns shall be used to hold loose tubes together

    - A wrapping tape around the core,

    - A longitudinal identiﬁcation tape

    - A non-metallic water blocking tape/moisture barrier to prevent water ingress (not required for uni-tube cable construction).

    - Uni-tube cable construction is required for cables with <=12F.  A sheath strength member is required for uni-tube cable construction.

    - Two rip cords are required for all cable types

    - Outer sheath shall be HDPE for induct cables and MDPE for aerial cables.

    - STRENGTH MEMBER

The primary function of the strength member is to provide the cable with stress and strain characteristics that will prevent the optical fibers from being stressed during installation, in service, and during operation and maintenance procedures. Two designs of strength member are allowed: central strength member or a sheath strength member. A combination of the two designs is also allowed.

Sheath-type strength members shall be manufactured from aramid cords or similar non-conductive, high tensile strength, high Young's modulus, and low elongation material.

Strength members shall be fiberglass or an alternative non-metallic material.

    - CENTRAL STRENGTH MEMBER SHEATH

If the central strength member is required to be a minimum diameter, a sheath may be extruded over the central strength member.

The central strength member sheath shall consist of polyethylene or other suitable material which shall neither be affected by nor affect the filling compound or any other components or materials of the cable.

    - LOOSE TUBES

The loose tubes protect the optical fibers from axial or radial stresses, allowing free movement of the fibers within the tube.

The loose tube secondary containment shall consist of polyamide or an alternative material which is equivalent or superior to polyamide.

Tenderers shall submit documentary evidence for alternative materials for approval by MIC2.

The loose tubes shall be easily removable for a length of 2.0 meters by the standard tools and techniques used by MIC2 without causing damage to the fibers or primary coating.

For cables containing more than one tube per cable, each tube shall be color-coded according to Table 1. The colors shall comply with IEC 304.

    - FILLING COMPOUND WITHIN THE LOOSE TUBES

The space around the primary coated fiber shall be completely filled with a filling compound which shall allow free movement of the fibers within the tubes.

The filling compound shall be compatible with the primary coating, the loose tubes, and any other components or materials of the cable which it may contact.

The compound shall allow free movement of the fiber in the tube.

The filling compound shall not be silicon-based.

    - OPTICAL FIBERS

The optical fiber shall conform to ITU-T Recommendations G.652D.

The Tenderer shall provide the main characteristics as listed in ITU-T for the proposed optical fiber.

The attenuation variations of the cabled optical fiber, if any, over the temperature range -15°C to +70°C will be indicated; it shall be fully reversible and taken into account in the optical power budget of the Sub-Segments.

All optical fibers shall maintain their geometrical properties for the required life, 40 years, of the cable.

    - OPTICAL FIBER PRIMARY COATING

The fiber coating may consist of one or more coatings.

The coating, or coatings, shall consist of a heat or ultraviolet-cured acrylic material or a suitable alternative which is compatible with all components and materials of the cable.

The diameter of the coating shall be 250±15 μm (QC).

The coating shall be easily removable without damage to the fiber. If the tenderer recommends a mechanical removal method, the tensile force exerted on the fiber during removal shall not exceed the tensile force exerted during proof testing.

If chemicals are required to soften the coating, they shall be non-toxic and dermatologically safe.

    - FIBER IDENTIFICATION

Loose tubes shall be color-coded so that tubes are colored according to table 2.

Tubes 13 to 24 will have black rings, black tubes will white rings.

Fibers inside the tubes shall be color-coded according to Table 1.

The colors shall comply with IEC 304. Single fibers per tube shall be natural color.

| Fiber Number | Color |
| --- | --- |
| 1 | Blue |
| 2 | Orange |
| 3 | Green |
| 4 | Brown |
| 5 | Grey |
| 6 | White |
| 7 | Red |
| 8 | Black |
| 9 | Yellow |
| 10 | Violet |
| 11 | Pink |
| 12 | Turquoise |

| Tube Number | Color | Tube Number | Color |
| --- | --- | --- | --- |
| 1 | Blue | 13 | Blue with black Rings |
| 2 | Orange | 14 | Orange with black Rings |
| 3 | Green | 15 | Green with black Rings |
| 4 | Brown | 16 | Brown with black Rings |
| 5 | Grey | 17 | Grey with black Rings |
| 6 | White | 18 | White with black Rings |
| 7 | Red | 19 | Red with black Rings |
| 8 | Black | 20 | Black with white Rings |
| 9 | Yellow | 21 | Yellow with black Rings |
| 10 | Violet | 22 | Violet with black Rings |
| 11 | Pink | 23 | Pink with black Rings |
| 12 | Turquoise | 24 | Turquoise with black Rings |

Table1:	Table2:

The fiber coloring shall not interfere with light injection used in fusion splicing machines.

    - CORE WRAPPING

The assembled filled core shall be completely covered with one or more continuous layers.

The wrapping shall be a fully dielectric material such as polyethylene or polypropylene.

The wrapping shall act as a heat barrier to prevent deformation, adhesion, or damage to the components and materials of the core.

The wrapping shall not adhere to the cable components.

    - INTERNAL IDENTIFICATION

Cable Identification

The outer sheath of the cable shall be marked and shall include the following details:

- Year of manufacture, and Manufacturer’s acronym,

- Number and type of fibers,

- Fiber cable type: “ANTI-RODENT” if used or “Duct Cable”

- Project name “MOT-MIC2”

- Length marking shall be permanently printed every meter.

    - MOISTURE BARRIER

Water-blocking Tape if needed to prevent water ingress.

    - OUTER SHEATH

The first sheath, Outer sheath, should be MDPE for aerial cables and HDPE for in duct cables.

The sheath shall be circular, free from pinholes, joints, repairs, and other defects.

The maximum diameter of duct cables shall be max 7.5mm for both 4F and 12F for unitube cable construction.

The nominal thickness of the first sheath shall be a minimum of 1.5 mm.

The average thickness at any cross-section shall not be less than 90% of the nominal thickness. The minimum spot thickness shall be not less than 90% of the nominal thickness.

    - POLYETHYLENE CLASSIFICATION

The first sheath shall consist of tough weather-resistant polyethylene conforming to the following classification from ASTM D 1248. The following classification tests shall be performed on raw material.

Type 1 or 2, corresponding to low or medium-density polyethylene with density ranging from 0.910 to 0.940 x 10³ kg/m³.

Type l or 2, corresponding to MDPE with density ranging from (925-940 kg/m3) or Type 3 HDPE with density ranging from 940 to 958 kg/m3 (ISO 1872-2/ISO 1183).

Class C, corresponding to black weather-resistant polyethylene containing 2.5-10.5% of well-dispersed carbon black and other additives as agreed upon.

Category 4 or 5, i.e., Polyethylene with a melt flow index less than or equal to 1.0.

Polyethylene, which has the physical properties of Grade J4, i.e., polyethylene that has the properties of grade J4 as detailed in Table 3 of ASTM D 1248.

The Polyethylene compound shall have:

a. Outstanding UV resistance

b. Excellent environmental stress crack resistance (ESCR) – IEC 60811-4-1/B

c. Excellent abrasion and scratch resistance

d. Low water permeability

    - ELONGATION OF SHEATH AFTER AGING)

Samples removed from completed cable shall comply with the requirements of IEC708-1.

    - MECHANICAL PROPERTIES OF SHEATH

Sheath removed from cable shall be tested according to the method of ASTM D2633 and shall satisfy the mechanical properties for the type of polyethylene used as shown in Table2

| MDPE | LDPE |  |
| --- | --- | --- |
| 16.5 | 12 | Tensile Strength (MPa) |
| 300 | 400 | Percent Elongation |

Table 2: Mechanical Properties of Sheath

  - MECHANICAL REQUIREMENTS GENERAL

    - DUCT CABLE

Fiber cable for duct installations shall be able to withstand a tensile force at least 2.0 KN, 1.5KN for unitube cable construction with <=12F.

Aramid yarns shall be used to meet the tensile requirements.

All cable intended for duct installation shall be capable of being field-fitted with a pulling eye. Tenderers shall submit full instructions for field fitting a pulling eye.

    - LONGITUDINAL WATER PENETRATION (QC)

The cable shall comply with the requirements of IEC 60794-1-2F5A/B.

    - TEMPERATURE CYCLING TEST (TA)

The temperature cycling test shall be performed according to IEC 60794-1-2F1. The sample length shall be at least 500 meters. The following test parameters shall be used:

| Preconditioning | 24 hours at 23°C ± 5° |
| --- | --- |
| Temperature TA | -10°C |
| Time t l | 24 hours |
| Temperature TB | 70°C |
| Number of cycle | 10 |

Attenuation shall be measured at 1310 nm and 1550 nm on six randomly selected fibers. Attenuation increase for any fiber at the temperature extremes or on return to the preconditioning temperature shall be less than 0.1 dB/km.

    - TENSILE TEST (TA)

A report shall be provided of the cable’s attenuation/load characteristics when tested according to IEC 60794-1-2E1.

The tensile force shall be applied at a jaw speed of 40 mm per minute, in increments of 100 N. The resultant plot shall include tensile force and cable elongation as test parameters.

The test shall be halted at 3 kN or when a cable element fails.

Tenderers shall also provide graphs of:

- Cable tension versus cable strain

- Cable strain versus fiber strain

    - CRUSH RESISTANCE TEST (TA)

Cable shall be tested according to IEC 60794-1-2E3, a load of 2500 N shall be gradually applied and maintained for a period of ten minutes for duct cable and aerial cable.

The increase in attenuation under load and on removal of the load shall be less than 0.1 dB at 1550 nm and 1300 nm.

There shall be no permanent damage to any of the cable components.

    - IMPACT TEST (TA)

Cable shall be tested according to IEC 60794-1-2E4. Impact force of 10 N.m, 3 Impacts spaced by R=30mm. The increase in attenuation shall be less than 0.1 dB at 1550 nm and 1300 nm. There shall be no damage to any of the cable components.

    - REPEATED BENDING TEST (TA)

Cable shall be tested according to IEC 60794-1-2E6. The bending radius shall be 15 times the outside diameter of the cable. The mass of the weight shall be 3 kg. The cable shall be tested for 50 cycles at a frequency of 12 cycles per minute.

There shall be no damage to any of the cable components. The increase in loss shall be less than 0.1 dB at 1550 nm and 1310 nm.

    - BENDING REQUIREMENTS (TA)

Cable shall be capable of meeting the following minimum bending radius without cable or fiber damage. Attenuation increase under full load and on removal of the load shall be less than 0.1 dB/km at 1550 nm and 1300 nm. Tenderers shall supply full details of the test used to confirm the bending properties of the cable.

Cable shall be able to be formed into a loop to pass through the standard MIC2 manhole entry of 680 mm diameter and for the hand hole, with no attenuation increase or damage to cable components.

For Cable, Acceptance tests shall be performed on, but not limited to, each cable length. These shall at least include but not be limited to:

- Two-way O.T.D.R at 1310 nm/1550 nm

- Insertion loss at 1310 nm/1550 nm,

- Chromatic dispersion,

- PMD

- Insulation resistance.

In addition to the paper format, disk copies of all O.T.D.R traces are to be supplied.

    - TORSION TEST (TA)

This test method is intended to establish the ability of a fiber optic cable to withstand mechanical twisting. The primary purpose of this procedure is to measure any variation in the optical power transmittance of a fiber when the cable is subjected to torsional forces external to the cable sheath. A secondary purpose is to evaluate the possibility of physical damage that may occur as a result of such stresses.

Test shall be performed according to IEC 60794-1-21-Method E7

10 Cycles

Sample Length under test: 1 or 2 m

Turns: +/- 180 deg

Load: 150 N

The additional attenuation after test shall be <=0.1dB

There shall be no damage to outer jacket and other cable components

  - FACTORY ACCEPTANCE TESTING

Factory Acceptance tests of cable shall be performed on, but not limited to, each cable length. These shall at least include but not be limited to:

o Two-way O.T.D.R at 1310 nm/1 550 nm

o Insertion loss at 1310 nm/ 1550 nm,

o Chromatic dispersion,

o PMD

o Insulation resistance.

In addition to the paper format, soft copies of all O.T.D.R traces are to be supplied.

  - OPTICAL TRANSMISSION REQUIREMENTS: (ITU-T G.652D)

| OPTICAL SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| Attenuation | Max Value (dB/km) |  |  |
| Attenuation at 1310 nm | < 0.34 |  |  |
| Attenuation at 1383 nm | < 0.31 |  |  |
| Attenuation at 1490 nm | < 0.24 |  |  |
| Attenuation at 1550 nm | < 0.21 |  |  |
| Attenuation at 1625 nm | < 0.24 |  |  |
| Attenuation vs. Wavelength |  |  |  |
| Wavelength range (nm) | Reference λ (nm) | (dB/km) |  |
| 1285-1330 | 1310 | ≤ 0.03 |  |
| 1360-1480 | 1385 | ≤ 0.04 |  |
| 1525-1575 | 1550 | ≤ 0.02 |  |
| 1460-1625 | 1500 | ≤ 0.04 |  |
| Point Discontinuities |  |  |  |
| No attenuation discontinuities greater than 0.05 dB at 1310 nm or 1550 nm |  |  |  |
| Attenuation with Bending |  |  |  |
| Number of Turns | Diameter(mm) | Wavelength (nm) | Attenuation (dB) |
| 1 | 32 | 1550 | ≤ 0.05 |
| 100 | 50 | 1310 | ≤ 0.05 |
| 100 | 50 | 1550 | ≤ 0.05 |
| 100 | 60 | 1625 | ≤ 0.05 |
| Cable cut-off wavelength(λcc) | ≤ 1260 nm |  |  |
| Mode Field Diameter |  |  |  |
| Wavelength (nm) | MFD (μm) |  |  |
| at 1310 | 9.2 ± 0.4 |  |  |
| at 1550 | 10.3 ± 0.5 |  |  |
| Chromatic Dispersion |  |  |  |
| Wavelength (nm) | Chromatic Dispersion (ps/nm × km) |  |  |
| 1285-1330 | ≤ 3 |  |  |
| 1550 | ≤ 18 |  |  |
| 1625 | ≤ 22 |  |  |
| Zero Dispersion Wavelength (λ0) | 1312±10 nm |  |  |
| Slope (S0) at λ0 | ≤ 0.090 ps/nm² × km |  |  |
| Polarization Mode Dispersion (PMD) |  |  |  |
| PMD Link Design Value | ≤ 0.06 ps/√km |  |  |
| Max.Individual Fiber | ≤ 0.1 ps/√km |  |  |

  - TYPE APPROVAL TESTS, SAMPLES AND REPORT REQUIREMENTS

Tenderers shall supply the following reports, results etc. for Type Approval

    - COMPLIANCE STATEMENT

Tenderers shall supply a clause by clause compliance statement, with the complete specification, in a side by side format

Three statements to describe Tenderer’s response with each clause are shown below.

- Complaint

The Tenderer agrees to the stated requirement without any reservation

- Notloose tube Compliant

The Tenderer does not meet the respective item or clause.  The reason for non-compliance shall be stated

- Not Compliant with an alternative proposal

The Tenderer does not meet the provisions of the clause but offers an equivalent or better alternative which shall be fully documented with supporting evidence.

    - EFFECT OF HYDROGEN

Evidence to verify that the properties of the fibers will not be degraded by the presence of hydrogen.

    - STRENGTH MEMBER

Full details of the strength member including materials used, ultimate breaking strength and Young's modulus.

    - CENTRAL STRENGTH MEMBER SHEATH

Type and thickness of central strength member sheath

    - CORE FILLING COMPOUND

Tenderers shall supply full details of the filling compound used including, dermatological hazards and evidence of compatibility.

    - LOOSE TUBES

Tenderers shall give full details of the loose tubes, including material, manufacturer and manufacturer's data sheet.

    - FILLING COMPOUND WITHIN LOOSE TUBES

Tenderers shall supply full details of the filling compound including manufacturer and evidence to verify the compatibility of the compound with other components of the cable which it contacts.

    - FIBER PROPERTIES

Fiber tensile test,

Macro bending loss

Long term minimum bending radius

Loss at small bending radius

The tenderer shall supply full details of the optical fiber including the manufacturer, numerical aperture, refractive index profile, effective group refractive index at 1300 nm and 1550 nm, a plot of the chromatic dispersion coefficient versus wavelength over the range 1285 nm to 1600 nm and a plot of attenuation versus wavelength from the cut off wavelength to 1600 nm.

    - PRIMARY COATING

The tenderer shall supply full details of the materials used for the fiber coating including recommended removal techniques. In addition, complete toxicological information for any chemicals required for removal shall be supplied with the tender bid. If a mechanical removal method is recommended the tenderer shall supply evidence that the force exerted on the fiber during removal does not exceed the force applied during proof testing.

    - CORE WRAPPING

Tenderer shall state the type of wrapping used, thickness and the number of layers.

    - MOISTURE BARRIER

Tenderers shall supply evidence to verify the effectiveness of non-metallic barriers.

Evidence to verify the compatibility of the non-metallic barrier with filling compound Moisture Barrier resistance

    - FIRST AND SECOND SHEATH (as applicable)

Manufacturer's data sheet of the raw material

    - PHYSICAL REQUIREMENTS OF ASSEMBLED CABLE

Temperature cycling

Tensile test

Crush resistance test

Impact test

Repeated Bending test

Bending Requirements

Torsion Requirements

    - SAMPLING PLAN FOR FIBER PROPERTIES

The sampling plan for the testing of fiber properties

shall be submitted for Type Approval.

    - TEST SHEETS FOR QUALITY CONTROL TESTS

TEST RESULTS TO BE PROVIDED ACCORDING TO THE MANUFACTURERS SAMPLING PLAN

- The fiber manufacturer shall be identified.

- Unless agreed otherwise the following fiber tests shall be performed on a sampling basis. The sampling plan shall be submitted to MIC2 as part of the Type Approval process.

- Mode field diameter

- Mode field concentricity error

- Cladding diameter.

- Cladding non circularity

- Coating diameter

- Zero dispersion slope and zero dispersion wavelength

- Chromatic dispersion

TEST RESULTS TO BE PROVIDED FOR EACH DELIVERY OR TEN (10) DRUMS

Core filling compound properties (not required for this tender)

Core filling compound tests shall only be provided per delivery.

      - Drop point

      - Flash point

      - Total acid value

      - Oil separation

Sheaths

        - First sheath classification

        - Second sheath classification if applicable

        - Elongation after aging

ASSEMBLED CORE

Longitudinal water penetration

TEST RESULTS TO BE PROVIDED FOR EACH DRUM

Sheath Properties

Sheath thicknesses at either end of the cable length.

Transmission Tests on Assembled cable

- Attenuation for all fibers in each drum

- Average attenuation for each drum length.

- Cut off wavelength for all fibers in each cable drum. For each fiber in each cable drum tenderers shall supply an Optical Time Domain Reflect meter (OTDR) trace at 1550 nm. If there are any irregularities or point losses, then a 1300 nm trace shall also be provided.

    - RECORDS

The manufacturer shall ensure the ready availability of suitable summary records for a period of not less than 5 years of all tests required by this specification in a format which identifies individual drums and is easily readable.

2. FIBER CLOSURE

2.1	Long Term Performance

a. The material supplied should be capable of withstanding the typical service conditions in Lebanon for a period of 25 years without detriment to the operation and maintenance characteristics.

b. The materials should be designed, manufactured, and packed so that exposure to the conditions of storage, transport inside and outside Lebanon, and installation in the environment will not affect the physical characteristics.

2.2 Type Approval

a. A sample should be provided for testing if the product has not been previously approved or used by MIC2.
b. All tests should be performed and submitted if requested.
c. All associated international specifications should be respected.
d. Products previously approved or used by MIC2 are accepted.

2.3 General Requirements

a. Delivered materials should be free from bursts, sharp edges, or projections that may be hazardous to personnel.
b. All products should have the following details:

- Manufacturer's name (built-in stamp).

2.4. QUALITY ASSURANCE

QUALITY SYSTEM ACCREDITATION

a. Manufacturers might be required to supply a copy of the Quality Manual, which shall be utilized in assessing the offered product's compliance with the provided specifications, at the time of tender.

b. MIC2 may require the manufacturer to be accredited to this specification either by MIC2 personnel or assessors acting on behalf of MIC2.

INSPECTION

- MIC2 or its authorized representatives may inspect the Tenderer’s facilities at any time for the purpose of Quality Assurance surveillance.

- If requested by MIC2, the tenderer shall supply evidence of the quality of raw materials and components used in the manufacturing process.

- All supplied items manufactured to this specification may be inspected and tested by MIC2 to check compliance.

- MIC2 has the right to request proof of compliance with this specification, either by witnessing actual performance of this specification's prescribed tests and/or the provisioning of documented test results at the discretion of the inspector.

- In the case of a dispute, testing shall be performed by an independent authority at the expense of the tenderer.

2.5. RESERVED RIGHTS

MIC2 reserves the right to make changes to the specification.

2.6 Components

The closure should be composed of:

- The joint enclosure shell

- The cable seals (for full capacity)

- The Splice Tray (for full capacity)

- Heat Shrink fiber splicing tubes (for full capacity)

|  | Compliance Chart |  |  |
| --- | --- | --- | --- |
|  | YES | NO | Alternative Solutions |
| Closures should be fully equipped with trays and accessories (Including Heat Shrink tubes/sleeves for fiber splice protection for the full capacity). |  |  |  |
| 2.7.The joint enclosure shell |  |  |  |
| Mechanical latch closing mechanism |  |  |  |
| The shell and its end plates shall be made of a polypropylene or polycarbonate. The material shall be fire resistance level HB according to UL 94. |  |  |  |
| Quick release latches material must be reinforced with a thermoplastic material fire resistance level HB according to UL 94. |  |  |  |
| The quick release latches can be linked to the base of the closure and can be replaced. |  |  |  |
| The manufacture logo shall be engraved on outer enclosure body. An indication shall be engraved on inner enclosure body for the month and year of manufacture. Delivered material shall be maximum of 18 months old at delivery date. |  |  |  |
| Environmental protection to provide stabilization and resistance from chemicals inside manholes. |  |  |  |
| The joint enclosure shall be available in different sizes. It should be compatible with unitube and multi-tube loose tube optical fiber cables from 12 up to 288 fibers, divided as follows: Number of Fibers; Cable Diameter (mm). 4-48 (small); 6-12. 4-96 (medium); 6-14. 4-288 (large); 6-18. |  |  |  |
| Closure should contain single Un-Cut main Cable entry. |  |  |  |
| Closure ports should be able to accept 24 drop cables by using single or multi-Cable seals or grommets. Seals shall be made of soft rubber (not dry) or gel. |  |  |  |
| Uncut tubes during mid-span splicing methods shall be able to securely loop in storage space without disturbing splice trays. |  |  |  |
| The humidity inside the joint enclosure shall not exceed 75% relative humidity. |  |  |  |
| The enclosure should have different mechanical sealing kits and accessories to enter the cable of different sizes to the closure. |  |  |  |
| Should provide a hermetically sealed interior environment. |  |  |  |
| Should provide secure storage for the fiber splices. |  |  |  |
| Should be re-enterable several times and replaceable without interrupting service and by the use of standard jointer tools, unless provided with each enclosure. |  |  |  |
| Should provide the highest standard of protection against water and dust rated IP68. |  |  |  |
| Drop Cables are terminated Individually |  |  |  |
| 2.8.The Cable Seals |  |  |  |
| Dedicated to cover all multi-drop ports application individually or through a multi cable seal, to allow increasing the number of drops within the Closure. |  |  |  |
| Single or Multi cable seal should match distribution and drop cables outer diameter of different sizes as the following Table: Sealing Grommets; Fiber Cable; Outer-cable diameter (mm). Un-Cut main cable entry; 48 Fibers; 10.4. 72 Fibers. 96 Fibers; 11.6. 144 Fibers; 14.2. 288 Fibers; 18.0. 4 with 4 outputs; 4 Fibers; 6. 12 Fibers. 2 with 2 outputs or 2 with 1 output; 24 Fibers; 10.4. 48 Fibers. 72 Fibers. 96 Fibers; 11.6. 144 Fibers; 14.2. 288 Fibers; 18.0. |  |  |  |
| Allow to open incrementally the holes in order to insert a new drop with the rest of the holes plugged. Plug shall be easy to remove. |  |  |  |
| Should be fully mechanical and it should not require any specific tool unless provided with each closure. Does not require chemicals, resins, torch or external power supply to be installed. |  |  |  |
| Should be used with clamps in order to secure the drops relief and insertion into the grommet. All cable assemblies for single and multi-cable entries shall be provided. |  |  |  |
| 2.9. Splice Tray |  |  |  |
| Each splicing tray shall have the capacity to secure 12 fiber splices. Fibers shall be completely retained within the splicing tray, with no possibility of trapping, pinching or other damage to the fibers during assemblies. Single stacking of protected sleeve is required. |  |  |  |
| Uncut fibers can be stored as single circuits in trays and/or as cable elements in the storage space between the profiles. |  |  |  |
| Different splice trays can be installed on the same organizer. |  |  |  |
| Closure Size; Num. of equipped Trays. 4-48 (small); 4-trays. 4-96 (medium); 8-trays. 12-288 (large); 24-trays. |  |  |  |
| Each splicing tray should be equipped with a clear plastic cover. |  |  |  |
| Splicing trays should be hinged to provide access to all splices without disturbing other splice trays. |  |  |  |
| Splicing trays should have a color code kit and a number identification kit. |  |  |  |
| Splicing trays shall include a mechanism to secure the loose tubes. The connection shall ensure that no movement or strain is included in the fibers. (1 loose tube in >=6 tubes out) |  |  |  |
| A retaining mechanism shall be provided to secure the splice protection without movement within the confines of the splicing tray. The retaining mechanism shall be resistant to vibration. |  |  |  |
| It shall be possible to remove splices from the splicing tray without affecting the remaining splices or fibers. |  |  |  |
| Each splicing tray should offer a splice protector holding part. |  |  |  |
| Additional splice trays and cable entry kits can be installed whenever new cables are added to the closure. |  |  |  |
| A mechanism shall be provided to hold trays in up position in order to access another splice tray. It is preferred for trays to have a semi-gloss finish using RAL color 7035 (light grey) |  |  |  |

Mechanical Test Requirements

|  | Small Size | Medium Size | Large Size |
| --- | --- | --- | --- |
| Sealing | Min 2.5 PSI, & IP68 | Min 7.0 PSI & IP68 | Min 7.0 PSI & IP68 |
| Impact | Min 14.5 ft lb | Min 70 PSI | Min 70 PSI |
| Pull Force - Drop | Min 22 lbs | Min 22 lbs | Min 22 lbs |
| Pull Force - Feeder | Min 22 lbs | Min 22 lbs | Min 22 lbs |

    - Wall/pole mounting metal bracket shall be provided.  All metal parts or screws shall be stainless steel or galvanized steel (non- corrosive).  Other proposals that are equivalent or superior can be considered.

    - Number of splice trays shall be compatible with wall/pole mount bracket

    - Grounding kit shall be provided for wall/pole fixation of closure

    - A bond clamp shall be provided for electrical continuity of metal shielded cables

    - Pressure testing valve shall be provided

Environmental Conditions:

    - The humidity inside the joint enclosure shall not exceed 75% relative humidity

    - The enclosure shall be able to withstand the following temperature ranges:

    - Storage: -20C to 70C

    - Operation: -20 to 55C

    - All tests shall be certified by a third party.

Different Cable entry configuration Grommets shall not affect the price.

MIC2 will decide the type of grommets in the contract.

3. ODF pre-assembled

(Optical Distribution Frame, Fiber Patch Panels, and Accessories)

3.1. Long Term Performance

a. The material supplied should be capable of withstanding the typical service conditions in Lebanon for a period of 20 years without detriment to the operation and maintenance characteristics.

b. The materials should be designed, manufactured, and packed so that exposure to the conditions of storage, transport inside and outside Lebanon, and installation in the environment will not affect the physical characteristics.

3.2. Type Approval

a. A sample should be provided for testing, if the product has not been used previously by MIC2.

b. All tests should be performed and submitted if requested.

c. All Associated International Specifications should be respected.

d. Products previously approved by MIC2 might be accepted.

3.3. QUALITY ASSURANCE

QUALITY SYSTEM ACCREDITATION

a. Manufacturers might be required to supply a copy of the Quality Manual, which shall be utilized in assessing the offered product’s compliance with the provided specifications at the time of tender.

b. MIC2 may require the manufacturer to be accredited to this specification by either MIC2 personnel or assessors acting on behalf of MIC2.

INSPECTION

a. MIC2 or its authorized representatives may inspect the Tenderer’s facilities at any time for the purpose of Quality Assurance surveillance.

b. If requested by MIC2, the tenderer shall supply evidence of the quality of raw materials and components used in the manufacturing process.

c. All supplied items manufactured to this specification may be inspected and tested by MIC2 to check compliance.

d. MIC2 has the right to request proof of compliance with this specification, either by witnessing the actual performance of this specification's prescribed tests and/or the provision of documented test results at the discretion of the inspector.

e. In the case of a dispute, testing shall be performed by an independent authority at the expense of the tenderer.

3.4. RESERVED RIGHTS

MIC2 reserves the right to make changes to the specification.

3.5. General Requirements

a. Delivered materials should be free from burrs, sharp edges, or projections that may be hazardous to personnel.

b. All products should have the following details:

- Manufacturer's name (built-in stamp or laser printed).

- Serial number/Part number.

3.6. Specifications

|  | COMPLIANCE CHART |  |  |
| --- | --- | --- | --- |
|  | YES | NO | Alternative Solutions |
| 1. Fiber Patch Panel: |  |  |  |
| 1.1. General |  |  |  |
| 1. Made of galvanized steel or plastic |  |  |  |
| 2. Can fit inside 19-inch rack cabinet |  |  |  |
| 3. 3U rack-mounted size unit to fit at least 144 connections |  |  |  |
| 4. Front cover fixed to a metal hinge |  |  |  |
| 6. Pre-installed connectors and Pig tails inside patch panel for full capacity. |  |  |  |
| 7. Clear Numbering Label for Ports (Figure 2) |  |  |  |
| 8. Cable Management: |  |  |  |
| - Patch cords management accessories: (Figure 2) Piano metal hinge for smooth patch cord accessibility |  |  |  |
| - FO cables management accessories (Figure 1) |  |  |  |
| - Full Front cover to prevent any removal of the patch cords |  |  |  |
| 1.2. Accessories: all accessories shall be included |  |  |  |
| 1. Mounting brackets, fiber tubes and fan-outs. |  |  |  |
| 2. Foam and tie-wraps to fix the cable elements onto the trays |  |  |  |
| 3. Splice protector holder |  |  |  |
| 4. One or two trumpets for guiding pigtails as they exit the shelf |  |  |  |
| 5. Mounting screws and nuts |  |  |  |
| 6. Fiber Heat Shrinks for fiber splices for full capacity |  |  |  |
| 2. ODF (Optical Distribution Frame): |  |  |  |
| 1. Frame material: Aluminum or Galvanized Steel |  |  |  |
| 2. Height >= 2.2m, 47U |  |  |  |
| 3. Accommodates up to 12 patch panels *144 connectors (3U) = 1728 connectors |  |  |  |
| 4. Plastic rings: polyamide or polycarbonate |  |  |  |
| 5. Pre-assembled and include installation and grounding accessories together with all bend-limiting fiber jump routing (within the same vertical or from vertical to vertical) |  |  |  |
| 6. Allows robust and rigid fixation and strain-relief for the OSP fiber cable end |  |  |  |
| 7. Supports the use of 2.0 mm patch cords with either ITU-T G.652D or bend-insensitive ITU-T G.657A1/A2 The pigtail color code shall match the IEC 304 fiber/tube color code |  |  |  |

  - SDFs pre-assembled

(Frame, Fiber Patch Panels, and Accessories)

4.1. Long Term Performance

a. The material supplied should be capable of withstanding the typical service conditions in Lebanon for a period of 20 years without detriment to the operation and maintenance characteristics.

b. The materials should be designed, manufactured, and packed so that exposure to the conditions of storage, transport inside and outside Lebanon, and installation in the environment will not affect the physical characteristics.

4.2. Type Approval

a. A sample should be provided for testing, if the product has not been used previously by MIC2.

b. All tests should be performed and submitted if requested.

c. All Associated International Specifications should be respected.

d. Products previously approved by MIC2 might be accepted.

4.3. QUALITY ASSURANCE

1. QUALITY SYSTEM ACCREDITATION

a. Manufacturers might be required to supply a copy of the Quality Manual, which shall be utilized in assessing the offered product’s compliance with the provided specifications at the time of tender.

b. MIC2 may require the manufacturer to be accredited to this specification by either MIC2 personnel or assessors acting on behalf of MIC2.

2. INSPECTION

a. MIC2 or its authorized representatives may inspect the Tenderer’s facilities at any time for the purpose of Quality Assurance surveillance.

b. If requested by MIC2, the tenderer shall supply evidence of the quality of raw materials and components used in the manufacturing process.

c. All supplied items manufactured to this specification may be inspected and tested by MIC2 to check compliance.

d. MIC2 has the right to request proof of compliance with this specification, either by witnessing the actual performance of this specification's prescribed tests and/or the provisioning of documented test results at the discretion of the inspector.

e. In the case of a dispute, testing shall be performed by an independent authority at the expense of the tenderer.

4.4. RESERVED RIGHTS

MIC2 reserves the right to make changes to the specification.

4.5. General Requirements

a. Delivered materials should be free from burrs, sharp edges, or projections that may be hazardous to personnel.

b. All products should have the following details:

- Manufacturer's name (built-in stamp or laser printed).

- Serial number/Part number.

4.6. Specifications

|  | COMPLIANCE CHART |  |  |
| --- | --- | --- | --- |
|  | YES | NO | Alternative Solutions |
| 1. Fiber Patch Panel: |  |  |  |
| 1.1. General |  |  |  |
| 1. Made of galvanized steel or plastic |  |  |  |
| 2. Can fit inside 19-inch rack cabinet |  |  |  |
| 3. 3U rack-mounted size unit to fit at least 144 connections |  |  |  |
| 4. Front cover fixed to a metal hinge |  |  |  |
| 5. Minimum of 240 connectors SC/APC to provide a minimum of 48 input and 192 output ports using 1:4 Splitters |  |  |  |
| 6. Pre-installed 1:4 PLC splitters (48 pcs): (Fig1) Built-in pre-terminated (SC/APC) splitters inside patch panel |  |  |  |
| 7. Clear Numbering Label for Input/output Ports (Figure 2) |  |  |  |
| 8. Cable Management: |  |  |  |
| - Patch cords management accessories: (Figure 2) Piano metal hinge for smooth patch cord accessibility |  |  |  |
| - Splitters' FO cables management accessories (Figure 1) |  |  |  |
| - Full Front cover to prevent any removal of the patch cords |  |  |  |
| 1.2. Accessories: all accessories shall be included |  |  |  |
| 1. Mounting brackets, fiber tubes and fan-outs. |  |  |  |
| 2. Foam and tie-wraps to fix the cable elements onto the trays |  |  |  |
| 3. Splice protector holder |  |  |  |
| 4. One or two trumpets for guiding pigtails as they exit the shelf |  |  |  |
| 5. Mounting screws and nuts |  |  |  |
| 2. SDF (Splitter Distribution Frame): |  |  |  |
| 1. Frame material: Aluminum |  |  |  |
| 2. Height >= 2.2m, 47U |  |  |  |
| 3. Accommodates up to 12 patch panels (3U) |  |  |  |
| 4. Plastic rings: polyamide or polycarbonate |  |  |  |
| 5. Pre-assembled and include installation and grounding accessories together with all bend-limiting fiber jump routing (within the same vertical or from vertical to vertical) |  |  |  |
| 6. Allows robust and rigid fixation and strain-relief for the OSP fiber cable end |  |  |  |
| 7. Splitters specs as per the specifications of Optical Splitter (SLIM) listed in section number 6 |  |  |  |
| - Supports the use of 2.0 mm patch cords with either ITU-T G.652D or bend-insensitive ITU-T G.657A2 Figure1: Built-in pre-terminated (SC/APC) splitters. Figure2: Piano metal hinge. |  |  |  |

5. SUB-DUCT (1 or 5 or 7-WAY – With Rope Inside)

5.1. Long Term Performance

a. The material supplied should be capable of withstanding the typical service conditions in Lebanon for a period of 20 years without detriment to the operation and maintenance characteristics.
b. The materials should be designed, manufactured, and packed so that the exposure to the conditions of storage, transport inside and outside Lebanon, and the installation in the environment will not affect the physical characteristics.

5.2. Type Approval

a. A sample should be provided for testing.
b. All tests should be performed and submitted if requested.
c. All associated international specifications should be respected.

5.3. QUALITY ASSURANCE

QUALITY SYSTEM ACCREDITATION

a. Manufacturers may be required to supply a copy of the Quality Manual, which shall be utilized for the manufacture and delivery of microduct complying with this specification, at the time of tender.

b. MIC2 may require the manufacturer to be accredited to this specification either by MIC2 personnel or assessors acting on behalf of MIC2.

INSPECTION

a. MIC2 or its authorized representatives may inspect the Tenderer's facilities at any time for the purpose of Quality Assurance surveillance.

b. If requested by MIC2, the tenderer shall supply evidence of the quality of raw materials and components used in the manufacturing process.

c. All microduct manufactured to this specification may be inspected and tested by MIC2 to check compliance.

d. MIC2 has the right to request proof of compliance with this specification, either by witnessing actual performance of this specification's prescribed tests and/or the provisioning of documented test results at the discretion of the inspector.

e. In the case of a dispute, testing shall be performed by an independent authority at the expense of the tenderer.

4.4 RESERVED RIGHTS

MIC2 reserves the right to make changes to the specification.

5.5. General Requirements

a. Delivered materials should be free from burrs, sharp edges, or projections that may be hazardous to personnel.
b. All products should have the following details:

- Manufacturer's name (built-in stamp).

5.6. Compliance Statement

The tenderer must provide the compliance statement with the specification requirements and indicate their compliance or non-compliance with all clauses of the specifications listed below. There are three statements to describe compliance:

- Complied

- Non-complied

- Non-complied - with alternative solution

5.7. Compliance List

| Configuration: | YES | NO | Alternative Solutions |
| --- | --- | --- | --- |
| Should be bundled with a polyethylene oversheath |  |  |  |
| Material: High-Density Polyethylene (HDPE) Silicore |  |  |  |
| Smooth inner & outer surface |  |  |  |
| Over Sheath Thickness: ~ 1.20 mm |  |  |  |
| Minimum Bending Diameter: ~1650 mm |  |  |  |
| Number of Micro-Ducts: 5 (With Rope Inside – Each Sub-duct) |  |  |  |
| Should be delivered on a standard reel |  |  |  |
| All tubes (single or bundled) should be colored (Blue-Green-White-Red-Yellow) |  |  |  |
| Installation application: inside duct 96 mm Internal Diameter |  |  |  |
| Footage markings: |  |  |  |
| The macro-duct shall be marked at 1 m intervals with: - Meters (length) - MIC2 Telecom |  |  |  |
| The marking shall be in an easily readable color with 3 mm high lettering. The marking shall not decrease the thickness of the materials by more than 0.3 mm. |  |  |  |
| Safe Working Pull Strength: 1288 kg |  |  |  |
| Internal Coefficient of Friction: ≤ 0.06 |  |  |  |
| Crush Performance: 1200N load applied for 60s; residual deformation shall not exceed 15% when checked after 1 hour of removing the load. |  |  |  |
| Micro-Duct Dimensions:. Outer Diameter; 28 mm. Inner Diameter; 24 mm. Test Methods. Base density; ISO 1183-1. Melt Flow Index; ASTM F 2160. ASTM D 1238. Reversion; ISO 2505. ESCR; ASTM D 1693. Crush Resistance; ASTM D 2412. Tensile Strength at yield Elongation at break; ASTM F 2160. Impact Strength; ASTM D 2444. |  |  |  |

6. Optical Splitter (SLIM)

6.1. Long Term Performance

a. The material supplied should be capable of withstanding the typical service conditions in Lebanon for a period of 20 years without detriment to the operation and maintenance characteristics.

b. The materials should be designed, manufactured, and packed so that exposure to the conditions of storage, transport inside and outside Lebanon, and installation in the environment will not affect the physical characteristics.

6.2. Type Approval

a. A sample should be provided for testing if requested.
b. All tests should be performed and submitted if requested.
c. All associated international specifications should be respected.
d. Products previously approved by MIC2/MoT are accepted.

6.3. QUALITY ASSURANCE

QUALITY SYSTEM ACCREDITATION

a. Manufacturers are required to supply a copy of the Quality Manual, which shall be utilized in assessing the offered product’s compliance provided with the specifications, at the time of tender.

b. MIC2 may require the manufacturer to be accredited to this specification either by MIC2 personnel or assessors acting on behalf of MIC2.

INSPECTION

a. MIC2 or its authorized representatives may inspect the Tenderer’s facilities at any time for the purpose of Quality Assurance surveillance.

b. If requested by MIC2, the tenderer shall supply evidence of the quality of raw materials and components used in the manufacturing process.

c. All supplied items manufactured to this specification may be inspected and tested by MIC2 to check compliance.

d. MIC2 has the right to request proof of compliance with this specification, either by witnessing actual performance of this specification's prescribed tests and/or the provision of documented test results at the discretion of the inspector.

e. In the case of a dispute, testing shall be performed by an independent authority at the expense of the tenderer.

f. Splitters (1:8, 1:16, 1:24, 1:32) shall have protected fiber strands.

g. Splitters (1:2, 1:4) might be with thin fiber strands.

h. Splitters should include 1m SC/APC pigtail with 1 SC/APC adapter

6.4. RESERVED RIGHTS

MIC2 reserves the right to make changes to the specification.

6.5. General Requirements

a. Delivered materials should be free from bursts, sharp edges, or projections that may be hazardous to personnel.
b. All products should have the following details:

- Manufacturer's name (built-in stamp) or logo

- Part number and/or serial number.

6.6. Specifications

|  | Compliance Chart |  |  |
| --- | --- | --- | --- |
|  | YES | NO | Alternative Solutions |
| The Optical splitter shall comply with all requirements in accordance with: - Telcordia GR-1209-CORE (Generic Requirements for Passive Optical Components) - GR-1221-CORE or IEC 61753-1 (Generic Reliability Assurance Requirements for Passive Optical Components) |  |  |  |
| Standard input: bend insensitive single mode fiber (ITU G.657A2) |  |  |  |
| Standard output: bend insensitive single mode ribbon or discrete fibers |  |  |  |
| Splitters outputs shall be numbered. |  |  |  |
| The PLC Optical splitter shall be a passive device to perform uniform optical signal splitting for GPON networks (FTTH application) |  |  |  |
| The Optical Splitter shall have one pre-connectorized input and 2, 4, 8, 16, 24, or 32 pre-connectorized outputs. |  |  |  |
| The splitting of signals shall be uniform across all the outputs. |  |  |  |
| Splitter should include 1m SC/APC pigtail with 1 SC/APC adapter |  |  |  |
| The optical splitter shall be applicable in an indoor or outdoor environment. |  |  |  |
| Location: 1. Inside the Fiber Distribution Box: 1.1. Should be pre-connectorized with SC/APC connector on both the input & output 1.2. Should fit within the tray of the FDB 1.3. Connector loss ≤ 0.3 dB (0.1 dB preferred). |  |  |  |
| The optical splitter output shall be color coded per TIA/EIA 598-B |  |  |  |
| Maximum length 6.5 cm (8cm for 1:32 splitter) |  |  |  |
| The optical splitter specifications should meet the following requirements: - 900 μm Input/outputs G.657A2 - Ideal for GPON, FTTH network - Operating wavelength 1260-1650 nm Standard; Max Ins. Loss (dB); Uniformity (dB); Polarization Depending Loss (dB) <=; Return Loss (dB); Directivity (dB). 1x2; 3.7; ≤0.6; 0.2; ≥55; ≥55. 1x4; 7.5; ≤0.6; 0.3; ≥55; ≥55. 1x8; 10.9; ≤0.8; 0.3; ≥55; ≥55. 1x16; 14; ≤1.0; 0.3; ≥55; ≥55. 1x24; 15.5; ≤1.1; 0.3; ≥55; ≥55. 1x32; 17.3; ≤1.3; 0.4; ≥55; ≥55. Minimum Temperature range of Installation; -20C to 55C. Minimum Temperature range of Storage; -20 to 70C. Maximum Input Power (mW); Tenderer to state the value. %Operating Humidity; <=90% (non-condensing). Standard compliance; GR-1221-CORE and GR-1209-CORE. 2011/65/EU RoHS And RoHS3; Tenderer to state compliance. Minimum IP protection Class; 20. |  |  |  |

Each delivered splitter shall be individually tested with test results provided for IL, RL, and PD, and uniformity unidirectional measured at 1310 and 1550nm.  The test report as shown below shall indicate that the result includes the connector/adapter loss

| Example: 1x2 Single Mode SFF Optical PLC Splitter Test Report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product Number |  |  |  |  |  |  |  |  |  |
| Serial Number |  |  |  |  |  | Test Date |  |  |  |
| Input | Output | Wavelength | Item | Channel |  | Max IL | Max PDL | Uniformity | Min RL |
| #1 | Fiber #1 | Channel | # | 1 | 2 | dB | dB | dB | dB |
|  |  | 1310nm | IL(dB) |  |  |  |  |  | >=55 |
|  |  |  | PDL (dB) |  |  |  |  |  |  |
|  |  | 1510 nm | IL(dB) |  |  |  |  |  |  |
|  |  |  | PDL (dB) |  |  |  |  |  |  |
| Fiber Type |  |  | ITU-T G.657A2 |  |  | 0.9mm optic cable |  |  |  |
| Fiber Length (m) |  |  | IN (m) OUT (m) | Connector |  | SC/APC |  |  |  |
|  |  |  |  | Connector Loss(dB) |  |  |  |  |  |
| Operating Temperature |  |  |  | Operating Wavelength |  | 1260-1650nm |  |  |  |
| Specs | Max IL(dB) with connector |  |  | Max PDL 9999(dB) |  |  |  | Pass or Fail |  |

Delivered splitters shall be securely packed

Optical splitter quality and performance is not only guaranteed by using high quality components and stringent manufacturing processes and equipment, but also by adherence to a successful Quality Assurance program. There are many factors which needs to be considered other than the insertion loss and return loss performance. The selection of the materials needs to be complimentary to each other to ensure proper cohesion when assembled and cured in the optimal condition. One of the most important factors is the epoxy which binds the fiber to the three main components of the splitter which ensure the adhesion of every component. The epoxy needs to be injected without introducing inconsistency or having trapped air bubbles and it needs to be cured at the right temperature at the right duration. The integrity, performance and long-term reliability of the optical splitter is paramount throughout the lifetime of the PON system. The adherence to the GR-1209 CORE and GR-1221 CORE test standards provides such an assurance.

The following reliability tests shall be carried out on the optical power splitter product (min of 1x2 splitter type) to be conducted under the environmental conditions defined in the Telcordia Standards GR-1221-CORE and GR-1209-CORE.

Tenderer to specify the sampling method used; randomly selected X samples from production lot XYZ following Lot Tolerance Percent Defective (LTPD) sampling plan.

A reliability test report shall be submitted following the tests showing the product name and model number.  Samples should be inspected visually before and after tests to examine their physical conditions.  IL and PDL data shall be recorded for each of the ten tests before and after the Telcordia reliability tests.  Real Time Delta IL and delta PDL shall also be recorded to determine if changes meet the Telcordia requirements for optical power splitter products.  Physical inspection such as detachment of silicone rubber boots, detachment of housing lid from the housing, fiber breakage, fiber pullout, loose tube crack, cable jacket damage or housing corrosion should be included in the test report.

- High Temperature Storage (Dry heat)

- Low Temperature Storage

- High Temperature Storage (Damp heat)

- Temperature Cycling

- Water immersion

- Thermal shock

- Vibration

- Mechanical shock

- Fiber Side Pull

- Fiber & Cable retention

| Environmental & Mechanical Tests | Test Conditions | Telcordia Standard |
| --- | --- | --- |
| High Temperature Storage (Dry heat) | 85C, Duration:2000 hrs | GR-1221-CORE |
| Low Temperature Storage | -40C, Duration: 2000 hrs | GR-1221-CORE |
| High Temperature Storage (Damp heat) | 85C/85%RH, Duration: 168 hrs and 2000 hrs respectively | GR-1209-CORE GR-1221-CORE |
| Temperature Cycling | -40C to 85C, Dwell Time >=15 min, Ramping Rate: ~1C/min; Cycle:10 and 500 respectively | GR-1209-CORE GR-1221-CORE |
| Water immersion | 43C, pH 5.5, Duration:168 hrs | GR-1209-CORE |
| Thermal shock | Delta T=100C (0C to 100C), liquid to liquid, Dwell Time: >=5min, Cycle:15 | GR-1221-CORE |
| Vibration | 20gr, 20-2000Hz, 4min/cycle, 4cycles/axis | GR-1221-CORE |
| Mechanical shock | Height of Drop: 1.8m, No of Drops:8, No of Direction: 6, Repeat: 5 times total | GR-1221-CORE |
| Fiber Side Pull | 0.23kg, 90degrees, 2 directions, Duration: 5 sec | GR-1209-CORE |
| Fiber & Cable retention | 0.45kg, 3 directions, Duration: 1min | GR-1209-CORE` |

Summary of Test Results

| Environmental & Mechanical Tests | GR-1209-CORE (Pass/Fail) | GR-1221-CORE (Pass/Fail) |
| --- | --- | --- |
| High Temperature Storage (Dry heat) |  |  |
| Low Temperature Storage |  |  |
| High Temperature Storage (Damp heat) |  |  |
| Temperature Cycling |  |  |
| Water immersion |  |  |
| Thermal shock |  |  |
| Vibration |  |  |
| Mechanical shock |  |  |
| Fiber Side Pull |  |  |
| Fiber & Cable retention |  |  |

Statistical Distribution of delta IL& delta PDL for 1x2 power splitter products before and after tests for all the tested samples

| Environmental & Mechanical Tests | Delta IL (dB) |  |  | Delta PDL (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mean | Std Dev | Abs Max | Mean | Std Dev | Abs Max |
| High Temperature Storage (Dry heat) |  |  |  |  |  |  |
| Low Temperature Storage |  |  |  |  |  |  |
| High Temperature Storage (Damp heat) |  |  |  |  |  |  |
| Temperature Cycling |  |  |  |  |  |  |
| Water immersion |  |  |  |  |  |  |
| Thermal shock |  |  |  |  |  |  |
| Vibration |  |  |  |  |  |  |
| Mechanical shock |  |  |  |  |  |  |
| Fiber Side Pull |  |  |  |  |  |  |
| Fiber & Cable retention |  |  |  |  |  |  |

7.FIBER DISTRIBUTION BOX (FDB)

7.1. Long Term Performance

a. The material supplied should be capable of withstanding the typical service conditions in Lebanon for a period of 25 years without detriment to the operation and maintenance characteristics.

b. The materials should be designed, manufactured, and packed so that exposure to the conditions of storage, transport inside and outside Lebanon, and installation in the environment will not affect the physical characteristics.

7.2. Type Approval

a. A sample should be provided for testing if the product has not been previously approved or used by MIC2/MoT.
b. All tests should be performed and submitted if requested.
c. All associated international specifications should be respected.
d. Products approved or used by MIC2/MoT are accepted.

7.3. General Requirements

a. Delivered materials should be free from burrs, sharp edges, or projections that may be hazardous to personnel.
b. All products should have the following details: Manufacturer's name (built-in stamp).

7.4. QUALITY ASSURANCE

QUALITY SYSTEM ACCREDITATION

a. Manufacturers may be required to supply a copy of the Quality Manual, which shall be utilized for the manufacture and delivery of the supplied material complying with this specification, at the time of tender.

b. MIC2 may require the manufacturer to be accredited to this specification either by MIC2 personnel or assessors acting on behalf of MIC2.

INSPECTION

- MIC2 or its authorized representatives may inspect the Tenderer’s facilities at any time for the purpose of Quality Assurance surveillance.

- If requested by MIC2, the tenderer shall supply evidence of the quality of raw materials and components used in the manufacturing process.

- All supplied items manufactured to this specification may be inspected and tested by MIC2 to check compliance.

- MIC2 has the right to request proof of compliance with this specification, either by witnessing actual performance of this specification's prescribed tests and/or the provisioning of documented test results at the discretion of the inspector.

- In the case of a dispute, testing shall be performed by an independent authority at the expense of the tenderer.

7.5. RESERVED RIGHTS

MIC2 reserves the right to make changes to the specification.

7.6. Specifications

|  | Compliance Chart |  |  |
| --- | --- | --- | --- |
|  | YES | NO | Alternative Solutions |
| The material used should be made of UV-resistant self-extinguishing thermoplastic material or galvanized sheet steel and powder coat painted for a long-life cycle, Agate Grey color. |  |  |  |
| Indoor Wall mountable. |  |  |  |
| Different sizes: Size 1: 16 ports Size 2: 24 ports Size 3: 32 ports Size 4: 48 ports |  |  |  |
| Cable Entry: Cable entry shall be simple, fast, and does not require special tools. Both incoming and outgoing cable entry grommets shall be suitable to accept drop and indoor cable. |  |  |  |
| Cable Entry Grommets: Cable entry grommets should be fixed properly and strongly to the opening of FDB. |  |  |  |
| Splice Cassette: The Fiber Box should include splice cassette on both sides to allow the splicing of a fiber optic cable to pigtails on both sides. |  |  |  |
| Splitter Modules: The FDB must be able to accommodate splitter modules properly on input side (left side). |  |  |  |
| Dual Separate Doors: Each door shall be equipped with separate lock for security. The FDB shall accommodate splicing splitter on left side and should accommodate internal cable entering the building on the right side, which should be connected with a pre-terminated SC/APC patch panel inside the Fiber Box. |  |  |  |
| Numbering Labels: The FDB shall have durable numbering labels providing clear fiber identification between the outgoing cable entry ports and the SC/APC coupling patch panel. The front doors shall be labeled according to MIC2 requirement with MIC2 and MoT Logos |  |  |  |
| SC/APC Couplers: The FDB shall be fully equipped with double-side SC/APC couplers. The couplers shall be secured with screws for both compartments. Adapter plate shall have legible and durable port number identification. |  |  |  |
| IP42 Compliance: The design of the FDB shall comply with IP42 in accordance with IEC 60529 Ed.2.1 standard (Degrees of Protection Provided by Enclosures). |  |  |  |
| Storage Area: The boxes should have a large storage area for micro modules and can accommodate a wide range of cable and drop cable diameters and the max number of customers drop cables. |  |  |  |
| Environmental Tolerance: The FDB shall be capable of tolerating a temperature range of -5°C to 50°C and relative humidity of 80% at 30°C. |  |  |  |

  - Specifications for the Outdoor FDT/FAT

Outdoor Distribution Box (ODB) Specifications

  - Long Term Performance

a. The material used should be capable of withstanding the typical service conditions for a period of 25 years without detriment to the operation and maintenance characteristics.

b. The materials should be designed, manufactured, and packed so that exposure to storage, transport inside and outside the installation area will not affect the physical properties.

  - Type Approval

a. A sample fully assembled with all necessary kits for wall and pole mounted should be provided for test.

b. All tests should be performed and submitted if requested.

c. All associated international specifications should be respected.

  - General Requirements

a. Delivered materials should be free from burrs, sharp edges, or projections that may be hazardous to personnel.

b. All products should have the following details: Manufacturer's name (built-in stamp).

  - Quality Assurance

    - QUALITY SYSTEM ACCREDITATION:

a. Manufacturers may be required to supply a copy of the Quality Manual, which shall be utilized for the manufacture and delivery of the supplied material complying with this specification at the time of tender.

b. Accreditation by MIC2 may be required by either MIC2 personnel or assessors acting on behalf of MIC2.

8.4.2. INSPECTION:

1. MIC2 or its authorized representatives may inspect the Tenderer's facilities at any time for Quality Assurance surveillance.

2. Evidence of raw materials and components used in manufacturing must be provided upon request.

3. All supplied items manufactured to this specification may be inspected by MIC2 for compliance.

4. MIC2 has the right to request proof of compliance with tests or documented test results.

Reserved Rights

MIC2 cannot guarantee that any of the requirements, standards, regulations, and conditions of this specification are not covered or protected by copyright or patents. MIC2 reserves the right to make changes without further notice.

  - Components

|  | COMPLIANCE CHART |  |  |
| --- | --- | --- | --- |
|  | YES | NO | Alternative Solutions |
| The material used should be made of UV resistant self-extinguishing thermoplastic |  |  |  |
| Outdoor Pole / Wall mountable fully equipped with all necessary accessories and components, (splice cassettes, splitter, couplers, etc.) |  |  |  |
| Cable entry shall be simple, fast and does not require special tool or special connectors. Both incoming and outgoing cable entry grommets shall be suitable to accept up to 16 drop or indoor cables. |  |  |  |
| Cable entry grommets shall be fixed properly to the opening of outdoor FDB. |  |  |  |
| Equipped with a splice cassette, the Fiber Box should allow the splicing of a fiber optic cable to pigtails or to a splitter & cable management system |  |  |  |
| The outdoor FDB must be able to accommodate one 1x16 pre-connectorised splitter. |  |  |  |
| One door/cover with locking options for flexibility and security. The cover shall be attached to the body and the outdoor FDB shall be able to accommodate pre-connectorised splitter and at least 17 splicing protections. The splitter should be connected with a pre-terminated SC/APC patch panel (up to 16 terminations) inside the Fiber Box. |  |  |  |
| The outdoor FDB shall have durable numbering labels providing clear fiber identification between the outgoing cable entry ports and the SC/AРС coupling patch panel. |  |  |  |
| The outdoor FDB shall be equipped with double side SC couplers. |  |  |  |
| Fiber cable entry at the bottom. |  |  |  |
| The design of the outdoor FDB shall comply with at least IP55 in accordance to IEC 60529 Ed.2.1 standard (Degrees of Protection Provided by Enclosures) and shall have an appropriate size and weight. |  |  |  |
| The box should have an enough storage area for mini-modules and can accommodate the required number of customer’s drops (up to 16). |  |  |  |
| The Outdoor FDB shall be capable to tolerate temperature range (-5°C to 55°C) and relative humidity of (80% at 30°С). |  |  |  |
| ODB Should serve as a closure and should contain trays for splitting the cable cores, and to place splitter and termination of 17 connectors |  |  |  |

  - PRE-TERMINATED FIBER BREAKOUT CABLE

  - Long Term Performance

  - The material supplied should be capable of withstanding the typical service conditions in Lebanon for a period of 20 years without determinant to the operation and maintenance characteristics.

  - The materials should be designed, manufactured and packed so that the exposure to the conditions of storage, transport inside and outside Lebanon and the installation in the environment will not affect the physical characteristics.

  - Type Approval

- A sample should be provided for test.

- All tests should be performed and submitted if requested.

- All Associated International Specifications should be respected.

- Products approved or used by MIC2/MoT are accepted.

  - General Requirements

  - Delivered materials should be free from burrs, sharp edges, or projections that may be hazardous to personnel.

  - All products should have the following details:

      - Manufacturer's name or Logo (built in stamp) on the cable.

  - QUALITY ASSURANCE

    - QUALITY SYSTEM ACCREDITATION

- Manufacturers are required to supply a copy of the Quality Manual which shall be utilized in assessing the offered product’s compliance with the provided speciﬁcations, at the time of tender.

- MIC2 may require the manufacturer to be accredited to this specification either by MIC2 personnel or assessors acting on behalf of MIC2.

    - INSPECTION

      - MIC2 or its authorized representatives may inspect the Tenderer‘s facilities at any time for the purpose of Quality Assurance surveillance.

      - If requested by MIC2 the tenderer shall supply evidence of the quality of raw materials and components used in the manufacturing process.

      - All supplied items manufactured to this speciﬁcation may be inspected and tested by MIC2 to check compliance.

      - MIC2 has the right to request proof of compliance with this speciﬁcation, either by   witnessing actual performance of this speciﬁcation's prescribed tests and/or the provisioning of documented test results at the discretion of the inspector.

      - In the case of a dispute, testing shall be performed by an independent authority at the expense of the tenderer.

  - RESERVED RIGHTS

MIC2 cannot guarantee that any of the requirements, standards, regulations and conditions of this speciﬁcation are not covered or protected by copyright or patent of a third party. MIC2 reserves the right to make changes to the specification without further notice.

  - INTENDED USE

- Breakout cable provides fast and precise connections from patch panels to high density ODFs.

- Reducing cable consumption

- Reducing cable management efforts.

Figure 1: Breakout Cable with Fan-out legs (both sides)

Figure 2: Overall Cable Assembly

Figure 1 and Figure 2 are considered a general description of the Breakout Cable Assembly and do not reflect exactly the requirements needed by MIC2 that are mentioned clearly in the following Specifications table.

SPECIFICATIONS

|  |  |  | COMPLIANCE CHART |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | YES | NO | Alternative Solutions |
| General |  |  |  |  |  |
| Protection Class IP | IP 20 |  |  |  |  |
| Cable Type | Breakout Cable |  |  |  |  |
| Cable Color | Yellow |  |  |  |  |
| Connector type at both sides | 1. Side1:SC/APC-Side2:SC/UPC 2. Side1:SC/APC-Side2:SC/APC |  |  |  |  |
| Jacket Material | LSZH |  |  |  |  |
| Cable Jacket Characteristics | Metal-Free |  |  |  |  |
| Cable Overall Diameter | 12.5 mm |  |  |  |  |
| Fiber Type | Single Mode |  |  |  |  |
| Fiber Class | G.657A2 |  |  |  |  |
| Number of Fiber connections | 16 |  |  |  |  |
| Each Fan-out leg should be numbered for the ease of identification at both sides ex. “Side1: 1A, Side2: 1B” (First and Second breakout length-Figure 2) |  |  |  |  |  |
| Jacket Marking - Manufacturer / Fiber Count / Fiber Type / Length Example: (xxx / 16F / G657A2 / 10M)) |  |  |  |  |  |
| Each fiber should be color coded as per IEC TIA/EIA-598 |  |  |  |  |  |
| Cable Length (Refer to Figures 1 & 2) |  |  |  |  |  |
| Breakout To Breakout length(as per Figure 2) |  | 1-3-5-7-10-15-20 m |  |  |  |
| Fan-out legs (First and Second end breakout length-Figure 2) |  | 1 meter |  |  |  |
| Mechanical |  |  |  |  |  |
| Pullout force | ≥ 100N |  |  |  |  |
| Insertion Loss | ≤ 0.3 dB |  |  |  |  |
| Return Loss | ≥ 55 dB |  |  |  |  |
| 2011/65/EU RoHS& 2015/863/EU RoHS | Fully compliant |  |  |  |  |

proof of compliance

Suppliers should clearly prove their compliance to every section (section 1 to section 7) in this Technical Specification

  - PIGTAIL

  - Long Term Performance

- The material supplied should be capable of withstanding the typical service conditions in Lebanon for a period of 20 years without determinant to the operation and maintenance characteristics.

- The materials should be designed, manufactured and packed so that the exposure to the conditions of storage, transport inside and outside Lebanon and the installation in the environment will not affect the physical characteristics.

  - Type Approval

- A sample should be provided for test.

- All tests should be performed and submitted if requested.

- All Associated International Specifications should be respected.

- Products approved or used by MIC2 are accepted.

Quality Assurance

  - QUALITY SYSTEM ACCREDITATION

- Manufacturers may be required to supply a copy of the Quality Manual which shall be utilized for the manufacture and delivery of the supplied material complying with this speciﬁcation, at the time of tender.

- MIC2 may require the manufacturer to be accredited to this specification either by MIC2 personnel or assessors acting on behalf of MIC2.

  - INSPECTION

- MIC2 or its authorized representatives may inspect the Tenderer ‘s facilities at any time for the purpose of Quality Assurance surveillance.

- If requested by MIC2 the tenderer shall supply evidence of the quality of raw materials and components used in the manufacturing process.

- All supplied items manufactured to this speciﬁcation may be inspected and tested by MIC2 to check compliance.

- MIC2 has the right to request proof of compliance with this speciﬁcation, either by   witnessing actual performance of this speciﬁcation's prescribed tests and/or the provisioning of documented test results at the discretion of the inspector.

- In the case of a dispute, testing shall be performed by an independent authority at the expense of the tenderer.

  - Reserved Rights

MIC2 cannot guarantee that any of the requirements, standards, regulations and conditions of this speciﬁcation are not covered or protected by copyright or patent of a third party. MIC2 reserves the right to make changes to the specification without further notice.

  - General Requirements

- Delivered materials should be free from burrs, sharp edges, or projections that may be hazardous to personnel.

- All products should have the following details:

- Manufacturer's name or Logo (built in stamp).

  - Compliance Statement

The tenderer must provide the compliance statement with the specifications requirements and indicate his compliance or non-compliance with all clauses of the specifications listed below. There are three statements to describe compliance:

- Complied

- Non complied

- Non complied - with alternative solution

  - Compliance List

| Specifications | Yes | No | Alternative Solutions |
| --- | --- | --- | --- |
| Design |  |  |  |
| Pigtail should provide a single connector terminated on 900µm buffered fiber. |  |  |  |
| Fiber cable: G.652D |  |  |  |
| Fiber type: Single mode. |  |  |  |
| Should be in 2 meters lengths. |  |  |  |
| Pigtail should come in SC/APC |  |  |  |
| Should be RoHS compliant. |  |  |  |
| Pigtail should be colored according to ANSI/TIA-598-C |  |  |  |
| Fibers are backward compatible with G.652D fibers for easy mechanical and fusion splice. |  |  |  |
| Optical Specifications |  |  |  |
| SC/APC; SC/UPC; LC/UPC; LC/APC. Attenuation (dB); 0.3; 0.3; 0.3; 0.3. Reflection (dB); ≥ 55 dB; ≥ 55 dB; ≥ 55 dB; ≥ 55 dB. Durability; 500 matings. Operating Temperature; -40°C to +75°C. |  |  |  |
| Standard Compliance |  |  |  |
| GR 1435 |  |  |  |
| ANSI/TIA-568-C |  |  |  |

  - Patch Panel 48 SC/APC SINGLE MODE

  - Long Term Performance

- The material supplied should be capable of withstanding the typical service conditions in Lebanon for a period of 20 years without determinant to the operation and maintenance characteristics.

- The materials should be designed, manufactured and packed so that the exposure to the conditions of storage, transport inside and outside Lebanon and the installation in the environment will not affect the physical characteristics.

  - Type Approval

- A sample should be provided for test.

- All tests should be performed and submitted if requested.

- All Associated International Specifications should be respected.

- Products approved or used by MIC2/MoT are accepted.

  - General Requirements

- Delivered materials should be free from burrs, sharp edges, or projections that may be hazardous to personnel.

- All products should have the following details:

- Manufacturer's name or Logo (built in stamp).

  - QUALITY ASSURANCE

    - QUALITY SYSTEM ACCREDITATION

- Manufacturers are required to supply a copy of the Quality Manual which shall be utilized in assessing the offered product’s compliance with the provided speciﬁcations, at the time of tender.

- MIC2 may require the manufacturer to be accredited to this specification either by personnel or assessors acting on behalf of MIC2.

    - INSPECTION

      - MIC2 or its authorized representatives may inspect the Tenderer ‘s facilities at any time for the purpose of Quality Assurance surveillance.

      - If requested by MIC2 the tenderer shall supply evidence of the quality of raw materials and components used in the manufacturing process.

      - All supplied items manufactured to this speciﬁcation may be inspected and tested by MIC2 to check compliance.

      - MIC2 has the right to request proof of compliance with this speciﬁcation, either by   witnessing actual performance of this speciﬁcation's prescribed tests and/or the provisioning of documented test results at the discretion of the inspector.

      - In the case of a dispute, testing shall be performed by an independent authority at the expense of the tenderer.

  - RESERVED RIGHTS

MIC2 cannot guarantee that any of the requirements, standards, regulations and conditions of this speciﬁcation are not covered or protected by copyright or patent of a third party. MIC2 reserves the right to make changes to the specification without further notice.

|  | COMPLIANCE CHART |  |  |
| --- | --- | --- | --- |
|  | YES | NO | Alternative Solutions |
| - GENERAL |  |  |  |
| - Should be made of galvanized steel with drawer |  |  |  |
| - single mode 48 SC/APC single mode connector adapters should be within Pre-connectorized plate |  |  |  |
| - Patching configuration should be - Between patch cords - Pre-connectorized break-out cable to patch cords - Pre-connectorized intrafacility cable to patch cords |  |  |  |
| - Allow splicing on the tray itself |  |  |  |
| - The patching areas of maximum 2 consecutive trays should not fall on each other but alter |  |  |  |
| - The drawer can be pulled out and held in position with a releasable spring |  |  |  |
| - A bracket is installed at the rear of the drawer on which the trays can be hinged |  |  |  |
| - Should be fully equipped |  |  |  |
| - Can be fit inside 19’’ rack |  |  |  |
| - 48 Pre-installed SC/APC pigtails and 48 Single Mode simplex adapters SC/APC |  |  |  |
| - Internal cable management protects splices and pigtails from lateral damage while maintaining the bend radius for optimal data packet throughput |  |  |  |
| - Accessories |  |  |  |
| - Metal chassis with drawer |  |  |  |
| - Mounting brackets |  |  |  |
| - Drawer includes: - Bracket onto which trays can be mounted - 2 tray support wedges - Hook and loop strap to secure trays - Bend control at the patching side for some configurations |  |  |  |
| - Patch versions include foam and tie-wraps to fix the cable elements onto the trays |  |  |  |
| - Trays include - Splice protector holder |  |  |  |
| - One or two trumpets for guiding pigtails as they exit the shelf |  |  |  |
| - Identification cards and card holders |  |  |  |
| - Mounting screws and nuts |  |  |  |
| - Installation instructions |  |  |  |