# REQUEST FOR PROPOSAL (RFP)

## 1. Introduction

MIC2 invites qualified vendors to submit proposals for the supply of 32 brand-new vehicles to support operational activities across Lebanon.

These vehicles will be used for telecom field operations, logistics support, and administrative functions across urban, suburban, and rough terrain environments.

This RFP aims to procure vehicles across four categories:

- Category A – Sedan (Electric): 4 Units (Administrative use)

- Category B – Mid-size SUV (4WD/AWD, Gasoline/hybrid equivalent): 20 Units (Field operations)

- Category C – Mid-size SUV (4WD/AWD, Electric): 7 Units (Field operations)

- Category D – Mid-size Van (2WD, Gasoline): 1 Unit (Logistics)

All vehicles must:

- Be brand new (0 km)

- Be latest production year (2026)

- Be suitable for road and climate conditions in Lebanon

## 2. Scope of Work

The selected bidder(s) shall:

- Supply vehicles across any category as per specifications

- Ensure delivery within the specified timeline

- Provide warranty and after-sales support

- Ensure availability of spare parts and service in Lebanon

## 3. Technical Specifications

### 3.1 General Requirements (All Categories)

- Model year: 2026

- Non-electric and hybrid vehicles must be compatible with unleaded gasoline (RON 95/98)

- For categories B, and C, the supplier shall demonstrate vehicle durability and suitability for field operations through technical specifications, design features (e.g., suspension type, underbody protection), and/or reference deployments in similar environments

- The offered price must include vehicle registration, number plates, and all administrative fees required for registration in Lebanon.

- Condition: Brand new (0 km)

- Specifications suitable for Lebanon (GCC or equivalent)

- The supplier shall ensure that branding is applied prior to vehicle delivery and conforms to the approved design standards

- All newly procured vehicles must be branded in accordance with the specifications and design provided in point #4  Vehicle Branding and Color Specifications

- Any deviation from the specified color or branding design will not be accepted

- Maintenance service interval: ≥ 10,000 km (longer maintenance interval, better technical grade)

- Automatic transmission

- Official import through authorized dealer in Lebanon

- Delivered fully registered and ready for operation

- Includes spare tire, tools, and standard accessories

### 3.2 Category A – Sedan (Administrative Use)

### Powertrain & Performance

- Vehicle type: Fully Electric Vehicle (BEV)

- Drive configuration: FWD or RWD

- Maximum power output: ≥ 120 kW

- Maximum torque: ≥ 250 Nm

- Energy consumption: ≤20 Kwh/100 Km

### Battery & Range

- Battery type: Lithium-ion (LFP or NMC)

- Battery capacity: ≥ 50 kWh

- Driving range: ≥ 400 km (WLTP standard mandatory)

### Charging Requirements

- AC Charging: ≥ 7 kW onboard charger

- DC Fast Charging: ≥ 50 kW

- Charging standard: CCS2 (mandatory)

- Charging port: Compatible with public charging infrastructure

### Dimensions & Capacity

- Only 4-door sedan body configuration shall be accepted

- Seating: Minimum 5 passengers

- Ground clearance: ≥ 130 mm (higher ground clearance better grade)

### Safety

- Minimum 4 airbags

- ABS, ESC

- Battery protection system (thermal management mandatory)

- High-voltage system safety compliance

### Features

- Air conditioning suitable for operation at ≥ 45°C

- Power windows, power steering, central locking

- Infotainment system with Bluetooth

- Rear parking sensors and/or camera

- Cruise control

- Low cabin noise

### Electrical & Auxiliary

- Minimum 2 × USB ports

- Minimum 1 × 12V outlet

- Capability to support auxiliary telecom devices (GPS, tracking)

### Climate & Operating Conditions

- Vehicle must operate efficiently under Lebanon climate conditions

- Battery cooling system (liquid cooling preferred)

### Additional Requirements

- Regenerative braking system

- Energy consumption indicator and driver information display

- Supplier to confirm availability of diagnostic tools and trained EV technicians in Lebanon

### 3.3 Category B – Mid-size SUV (Gasoline/Hybrid - Field Operations)

### Engine & Performance

- Power: ≥ 160 HP

- Torque: ≥ 220 Nm

- Transmission: Automatic

- Drivetrain: AWD or 4WD

### Fuel & Efficiency

- Vehicle type: Gasoline, Hybrid Electric Vehicle (HEV), or equivalent hybrid technology

- Fuel tank: ≥ 50 liters

- Fuel consumption: ≤ 10 L/100 km (WLTP or equivalent standard

- Hybrid systems offering improved fuel efficiency shall be positively evaluated

### Dimensions & Capacity

- Seating: Minimum 5 passengers

- Ground clearance: ≥ 185 mm (higher ground clearance, better grade)

- Trunk capacity: 500–750 liters (with rear seats in place)

### Safety

- Minimum 4-star safety rating (Euro NCAP or equivalent)

- Minimum 4 airbags

- ABS, ESC, Traction Control

- Hill Start Assist

- Hill Descent Control (preferred)

### Terrain Capability

- Vehicle shall be suitable for operation on unpaved roads, rough terrain, and moderate off-road conditions typical of telecom field operations

- The supplier shall demonstrate capability through technical specifications and/or reference deployments

- Vehicles equipped with terrain management modes (e.g., sand, mud, snow) are preferred (better grade)

- Electronic traction control system mandatory

### Features

- High-performance air conditioning suitable for ≥ 45°C

- Durable interior materials (suitable for field use)

- Infotainment system with Bluetooth

- Foldable rear seats

- Rear parking sensors and/or camera

- Cruise control

- Power windows, power steering, central locking

- Low cabin noise

### Electrical & Auxiliary

- Minimum 2 × USB ports

- Minimum 1 × 12V outlet

- Electrical system capable of supporting auxiliary telecom equipment (GPS, radios, tools)

### Climate & Operating Conditions

- Vehicle must operate efficiently under Lebanon climate conditions

- Cooling systems must support prolonged operation in high ambient temperatures

### Tires & Equipment

- All-terrain or all-season tires suitable for mixed road conditions

- Spare tire: Full-size (mandatory)

- Tire brand must be internationally recognized

### 3.4 Category C – Electric SUV (Operational Use)

### Powertrain & Performance

- Vehicle type: Fully Electric Vehicle (BEV)

- Drive configuration: AWD / 4WD

- Maximum power output: ≥ 150 kW

- Maximum torque: ≥ 300 Nm

### Battery & Range

- Battery type: Lithium-ion (LFP or NMC)

- Battery capacity: ≥ 70 kWh

- Driving range: ≥ 420 km (WLTP standard mandatory)

### Charging Requirements

- AC Charging: ≥ 7 kW onboard charger

- DC Fast Charging: ≥ 80 kW

- Charging standard: CCS2 (mandatory)

- Charging port: Compatible with public charging infrastructure

### Energy Efficiency

- Energy consumption: ≤ 21 kWh / 100 km (WLTP) (lower consumption, better grade)

### Dimensions & Capacity

- Seating: Minimum 5 passengers

- Ground clearance: ≥ 180 mm

- Trunk capacity: ≥ 400 liters

### Safety

- Minimum 4-star safety rating (Euro NCAP or equivalent)

- Minimum 4 airbags

- ABS, ESC, Traction Control

- Battery protection system (thermal management mandatory)

- High-voltage system safety compliance

- Hill Start Assist

- Hill Descent Control (preferred)

### Terrain Capability

- Vehicle shall be suitable for operation on unpaved roads, rough terrain, and moderate field conditions

- The supplier shall demonstrate capability through technical specifications and/or reference deployments

- Vehicles equipped with terrain management modes are preferred

- Electronic traction control system mandatory

### Features

- High-performance air conditioning suitable for ≥ 45°C

- Durable interior materials (field use)

- Infotainment system with Bluetooth

- Rear parking sensors and/or camera

- Foldable rear seats

- Cruise control

- Power windows, power steering, central locking

- Low cabin noise

### Electrical & Auxiliary

- Minimum 2 × USB ports

- Minimum 1 × 12V outlet

- Electrical system must support auxiliary telecom equipment (GPS, radios, tools)

### Climate & Operating Conditions

- Vehicle must operate efficiently under Lebanon climate conditions

- Battery cooling system (liquid cooling preferred)

- Thermal management system mandatory

### Tires & Equipment

- All-season tires suitable for mixed road conditions

- Spare tire or manufacturer-approved alternative (mandatory)

### Additional Requirements

- Regenerative braking system

- Energy consumption indicator and driver information display

- Supplier to confirm availability of diagnostic tools and trained EV technicians in Lebanon

### 3.5 Category D – Mid-size Van (Logistics Use)

### Engine & Performance

- Engine: ≥ 2.0L petrol, 4-cylinder

- Transmission: Manual or Automatic

- Drivetrain: 2WD

- Power Output: Minimum 140 HP

- Torque: Minimum 200 Nm

### Capacity and dimensions

- Seating: 3 passengers + rear cargo configuration

- Adequate cargo space for telecom equipment

- Payload Capacity: Minimum 800 kg

- Wheels: Minimum 15-inch steel or alloy

### Features

- Sliding side door and double rear doors (mandatory)

- Air conditioning

- power steering

- power windows

- central locking

- Durable interior

- Flat cargo floor (mandatory)

- Minimum 4 tie-down points

- Rear door opening ≥ 180°

- Internal cargo lighting

### Safety

- Minimum 2 airbags

- ABS, ESC

### 3.6 Warranty & Support

- Vehicle warranty: Minimum 5 years / 200,000 km

- Battery warranty: Minimum 8 years / 160,000 km

- Battery capacity retention: ≥ 70% at end of warranty period

- Higher warranty coverage will be positively evaluated

- Warranty obligations shall remain enforceable directly against the OEM or its authorized representative in Lebanon, regardless of any change in dealership or distributorship structure.

- Any warranty obligations shall remain valid regardless of changes in distributor or dealership structure during the contract period.

- Vehicles immobilized for more than seven (7) working days under warranty shall be subject to replacement mobility support.

### 3.7 Additional Technical Requirements

Electrical

- Minimum 2× 12V outlets + 2 USB ports

- Alternator must support continuous auxiliary load (telecom tools, GPS, radios)  (non electric SUV)

- LED or Halogen headlights

- Daytime Running Lights (preferred)

### Telecom Readiness

- Provision for installation of GPS tracker (all categories)

- Provision for installation of radio equipment (categories B, and C)

- Accessible fuse box for auxiliary connections

Climate

- A/C performance at ≥ 45°C

- Rear AC (SUV )

Tires

- SUV (Category B,C): All-terrain tires mandatory

- Spare tire: Full-size (mandatory)

- Tire brand must be internationally recognized

Fuel

- Compatible with RON 95

### 4. Platform Maturity and Field Provenness Requirements

#### 4.1 Platform Maturity Requirement

The proposed vehicle model(s) shall demonstrate established market maturity and production stability.

At minimum:

- The model shall have been in continuous commercial production and publicly available for at least twelve (12) months at the time of bid submission.

- The model shall not be a first-generation platform, prototype, pilot vehicle, pre-production model, or limited launch edition.

- Facelifted, refreshed, or updated versions are acceptable only where the core platform architecture (chassis and drivetrain concept) remains unchanged and in continuous production for the required period.

#### 4.2 Fleet Deployment Requirement

The proposed vehicle model shall have documented large-scale operational deployment in real fleet environments.

At minimum:

- The model shall have been supplied to at least three (3) independent fleet operators in telecom, utilities, oil & gas, government, or large commercial fleets. (more references, better grade)

- Each referenced fleet deployment shall include a minimum of twenty (20) units of the same model under operational use.

- Deployments must be active or historically verified operational deployments, excluding pilot programs, short-term trials, or evaluation fleets.

Acceptable supporting evidence:

- Signed reference letters from fleet operators

- Valid purchase orders or supply contracts (with sensitive financials redacted if required)

- OEM or distributor-certified case studies with identifiable client references

- Government or corporate procurement records

Both requirements under Sections 4.1 and 4.2 are mandatory qualification criteria.
Failure to comply with either requirement shall result in automatic technical disqualification.

The bidder shall ensure that the offered vehicle model has a minimum remaining production lifecycle of at least 3 years from the date of bid submission, ensuring availability of parts, service continuity, and model stability during the contract period.

### 5. Authorized Dealer Requirement

The Bidder must be an official authorized dealer, distributor, or representative of the vehicle manufacturer for the offered models in Lebanon.

## 5.1 Minimum Requirements

The Bidder shall demonstrate that it is fully authorized by the Original Equipment Manufacturer (OEM) to:

- Import and sell the proposed vehicle models in Lebanon

- Provide full warranty coverage on behalf of the manufacturer

- Perform or coordinate after-sales service and repairs for the offered models

- Supply genuine spare parts through approved channels

The authorization must be valid for the entire duration of the contract or be supported by a written commitment from the OEM to maintain authorization throughout the contract period.

## 5.2 Service Infrastructure Requirements

The Bidder shall have or guarantee access to a certified service infrastructure in Lebanon, including:

- At least one authorized service center capable of handling the proposed vehicle models

- Adequate workshop facilities for mechanical, electrical, and diagnostic repairs

- Availability of trained and certified technicians for both ICE and EV systems (as applicable)

- Access to OEM-approved diagnostic tools and software

## 5.3 Submission Requirements

The Bidder shall submit the following documents:

- Official authorization letter from the OEM or regional distributor

- Proof of dealership status (exclusive or non-exclusive, as applicable)

- List and location of authorized service centers in Lebanon

- Confirmation of technician training and certification levels

- Confirmation of access to OEM spare parts supply chain

### 6. Spare Parts Commitment

The Bidder shall ensure long-term availability and continuity of genuine spare parts for all offered vehicle models.

At minimum, the Bidder shall:

- Guarantee availability of genuine OEM spare parts in Lebanon or through an authorized regional supply chain.

- Ensure availability of fast-moving spare parts within a maximum lead time of ≤ seven (7) working days.

- Maintain adequate stock levels to support continuous fleet operation and minimize vehicle downtime.

- Ensure supply continuity for the full duration of the warranty period and beyond, as applicable.

The Bidder shall submit:

- Official spare parts price list covering at least the top twenty (20) fast-moving items per model

- Declaration of supply chain structure (local stock, regional warehouse, or OEM direct supply)

- Written confirmation of spare parts lead times

Non-compliance with spare parts availability requirements shall be considered as technical disqualification

### 7. Right to Inspection

MIC2 reserves the unconditional right to verify compliance with all technical and operational requirements before and during contract execution.

At minimum, MIC2 may:

- Request one or more demo vehicles of the offered model(s) for physical inspection

- Conduct technical inspections at dealership, warehouse, or service center facilities

- Verify compliance with specifications, including safety, performance, and build quality

- Request validation of fleet references and deployment claims

- Reject any vehicle or model that does not fully comply with the RFP specifications

## 8. Vehicle Branding and Color Specifications

Vehicle Color:

- Recommended base colors: white or silver.

- These colors provide a clean, professional look and ensure touch logo stands out clearly.

- They are also easy to maintain and consistent with modern corporate fleet standards.

Branding Elements:

- Company logo: prominently displayed on both side doors and the rear of each vehicle.

- Website: www.touch.com.lb

- Complaints Number

- Cars should be numbered

The branding should be vinyl printed (wrapped) rather than painted for the below reasons:

- Professional finish: high-resolution graphics with consistent colors

- Durability: weather-resistant and UV-protected materials last several years

- Flexibility: easy to remove or update if branding changes

- All associated costs shall be fully reflected in the Bill of Quantities (BoQ) Appendix 5

- Below is a simulation of the branding. The logo size on both front doors is A4, while the logo on the front hood of the cars is A3

- This is only a simulation; final branding should be validated with MIC2 before proceeding based on the exact car make and specifications

## The bidder shall use high-quality, durable vinyl wraps for all logos, text, and other required branding elements.

## 9. Vehicle Licensing and Registration

## The Bidder shall handle all vehicle registration formalities prior to delivery, including but not limited to:

- Vehicle inspection

- Customs clearance (if applicable)

- Vehicle licensing and registration with the relevant authorities

- Insurance coverage (if applicable)

- The Bidder shall provide all relevant documents to MIC2, including registration papers, vehicle license, insurance certificates, and guarantee booklet.

- All associated costs for registration; licensing, inspection, and insurance shall be fully included in the total bid price and reflected in the Bill of Quantities (BoQ) Appendix 5

- MIC2 reserves the right to verify that all registration and licensing formalities have been properly completed before accepting delivery of the vehicles.

## 10. Vehicle Delivery Requirements

- The Bidder shall deliver all vehicles to Touch HQ or any other location specified by MIC2

- All vehicles must be delivered in fully operational condition, complete with all specified accessories, documents, and branding requirements

- The Bidder shall ensure that each vehicle successfully completes all pre-delivery inspections and testing prior to delivery

- Delivery shall include the provision of all relevant documentation, including registration papers, insurance (if applicable) and any other legal documents related to the vehicle

- All vehicles must be delivered within maximum 6 months from the date of down payment

- MIC2 reserves the right to inspect and reject vehicles that do not meet the specifications or delivery conditions

## 11. Compliance Matrix

Mandatory submission required.

## 12. Evaluation

#### A. Technical specifications (40 Points)

- Refer to appendix_10

#### B. Commercial (60 Points)

- Price: 60

Formula: Score = (Lowest Price / Bid Price) × 60

## 13. Terms & Conditions

- MIC 2 reserves the right to accept or reject any proposal

- Incomplete submissions may be rejected

- Lowest price does not guarantee award

- Only compliant bidders will be evaluated commercially

- MIC2 reserves the right to award one or more categories to the same bidder or to multiple bidders, based on the evaluation results.

- Bidders may submit proposals for one or more categories.

- Each category will be evaluated independently, and award decisions will be made based on the highest overall score per category.

- MIC2 is not obligated to award all categories to a single supplier.

## 14. Contact Information

For clarifications: Rabih Imad 03964924

End of RFP Document