Dear Vendor,

Kindly send your best priced quotation for the supply of waveguides from different frequencies, IF cables, and their related accessories to support the maintenance and implementation of microwave transmission links.

These materials will be used for corrective and preventive maintenance activities on live microwave sites to ensure continued link availability and performance.

| Item description | Quantity |
| --- | --- |
| Microwave Accessory,Flexible Waveguide,18/23/26G,0.9m,PBR220,With English doc | 24 |
| Microwave Accessory,Flexible Waveguide,28/32/38G,0.9m,PBR320,With English doc | 24 |
| Microwave Accessory,Flexible Waveguide,15G,0.9m,PBR140,With English doc | 24 |
| Microwave Accessory,Flexible Waveguide,10/11G,1.8m,PBR100,With English doc | 8 |
| Microwave Accessory,Flexible Waveguide,10/11G,0.9m,PBR100,With English doc | 8 |
| Twistflex Hanger | 200 |
| Coaxial Cable, Copper-clad Aluminium Wire,50ohm,7.6mm,4.8mm,1.8mm, Black, 5D | 20,000 meter |
| Patch Cord Accessories of Double IF (5D) | 200 |

Below are the technical specs for the waveguides required:

| Parameter | Specification |
| --- | --- |
| Type | Flexible rectangular microwave waveguide |
| Application | Microwave transmission maintenance |
| Length | 0.9 meters |
| Frequency Range | As specified for each requested frequency band |
| Waveguide Size | Standard WR series corresponding to the operating frequency |
| Material | High-conductivity brass or copper with corrosion-resistant finish |
| Flanges | Standard IEC/CPR/UBR or equivalent, factory assembled |
| VSWR | ≤ 1.15:1 |
| Insertion Loss | Low-loss design as per manufacturer's specifications |
| Operating Temperature | -40°C to +70°C |
| Environmental Protection | Suitable for indoor and outdoor installations |

All waveguides should be compatible with Andrew and HATA antennas.

Suppliers should provide a sample for technical acceptance.

The sealed envelope should be submitted by hand no later than Monday 3 August 2026 before 12:00 PM and marked visibly with the reference TechProcurement /WAVEGUIDESIF /24/7/2026 and submitted

to the following address:

Procurement Department 8th floor

Beirut Central Building (Touch Building)

Plot # 1526 / Bashoura - Block B

Fouad Chehab Avenue

Beirut

PO. BOX 175051

For contact:  Mohamed  70 703 181

Best Regards,