LEBANESE REPUBLIC

MINISTRY OF PUBLIC WORKS AND TRANSPORT

PORT OF TRIPOLI AUTHORITY

GRAIN SILO AT THE PORT OF TRIPOLI

DBOT (Design, Build, Operate, Transfer)

Tender Document – Conditions of Tendering

ST-2030-TD-V001-O

March 2025

ST-2030-TD-V001-O

March 2025

GRAIN SILO AT THE PORT OF TRIPOLI

Tender Document – Conditions of Tendering

Prepared for

LEBANON – Tripoli, El Mina Ph : 00961 6 60 04 13 Fax : 00961 6 22 01 80 E-mail : info@oept.gov.lb

Prepared by

Lebanon, Byblos, Down Town, 31 Ernest Renan St, Beayni Center, 1st Floor

Ph	: 00961 9 55 00 22

Fax	: 00961 9 55 00 44

Mob	: 00961 3 29 10 30

Web	: www.seatec-sarl.com

E-mail	: seatec@seatec-sarl.com

March 2025

@SEATEC sarl 2025

This Document had been prepared by SEATEC sarl for the sole use of the Client and in accordance with engineering standards and generally accepted consultancy principles. Any information provided by third parties and referred to herein has not been checked or verified by SEATEC sarl, unless otherwise expressly stated in this document. No third party may rely upon this Document without the prior and express written agreement of SEATEC sarl.

All rights reserved. No section or element of this document may be removed from this document, reproduced, electronically, stored or transmitted in any form without the written permission of SEATEC sarl.

Grain Silo at the Port of Tripoli

Tender Document

Quality Information

| Client Port of Tripoli Authority (OEPT) | Contract Nbr |
| --- | --- |
| Project Grain Silo at the Port of Tripoli | Project Nbr ST-2030 |
| Document reference ST-2030-TD-001-O | Controlled Nbr |
| Document Title Tender Document |  |
| Cover letter / Transmittal ref. Nbr | Date 25-03-2025 |

Revision, review and approval records

| O | Tender Document | GS 20-03-2025 | MG 24-03-2025 | PS 25-03-2025 |
| --- | --- | --- | --- | --- |
| Revision | Description | Prepared by / date | Reviewed by / date | Approved by / date |

Document Revision Register

| Issue Nbr | Date of issue | Section | Revision Details | Revision by Name & Position |
| --- | --- | --- | --- | --- |
| 001-O | 25-03-2025 |  |  |  |

SEATEC sarl 									Tender Document

Table of Contents

1.	INSTRUCTIONS TO TENDERERS	8

1.1.	OBJECT OF THE TENDER	10

1.2.	DEFINITIONS, LAW AND LANGUAGE	13

1.2.1.	DEFINITIONS	13

1.2.2.	LANGUAGE	17

1.2.3.	LAW	17

1.3.	ELIGIBITY	18

1.3.1.	ELIGIBLE TENDERERS	18

1.3.2.	ELIGIBLE MATERIALS, EQUIPMENT AND PLANT	18

1.4.	QUALIFICATION OF THE TENDERER	19

1.4.1.	APPROVED TENDERERS	19

1.4.2.	EVIDENCE OF QUALIFICATION	23

1.4.3.	JOINT VENTURES	23

1.4.4.	NUMBER OF TENDERS	24

1.5.	TENDER DOCUMENTS	24

1.5.1.	CONTENT	24

1.5.2.	CONFIDENTIALLY AND EXAMINATION OF TENDER DOCUMENTS	25

1.5.3.	DISCREPANCIES AND CLARIFICATIONS	25

1.5.4.	AMENDMENT	26

1.5.5.	COMMUNICATIONS	26

1.5.6.	LANGUAGE OF TENDER	27

1.5.7.	INFORMATION	27

1.6.	SITE VISIT	28

1.7.	APPOINTMENT	28

1.8.	COST OF TENDERING	29

1.9.	COMPLETION OF TENDER DOCUMENTS	29

1.10.	SIGNING OF TENDER	29

1.11.	ALTERATIONS AND CORRECTION OF ERRORS	30

1.12.	CURRENCY	30

1.13.	DUTIES, TAXES, ETC.	30

1.14.	TENDER VALIDITY	31

1.15.	TENDER SECURITY	31

1.16.	TENDER SUBMISSION	32

1.16.1.	SEALING AND MARKING OF TENDERS	32

1.16.2.	VARIANT SOLUTIONS	35

1.16.3.	TIME LIMIT FOR SUBMISSION FOR TENDERS	36

1.16.4.	MODIFICATION AND WITHDRAWAL OF TENDERS	36

1.17.	TENDER OPENING AND EVALUATION	37

1.17.1.	UNSUCCESSFUL TENDERERS	37

1.17.2.	TENDER OPENING	37

1.17.3.	TENDERS EVALUATION	37

1.17.4.	CORRECTION OF ERRORS	40

1.18.	AWARD AND SIGNING OF CONTRACT	41

1.18.1.	AWARD	41

The Contracting Authority will award the contract to the bidder who submitted the administrative and technical proposal that substantially meets the requirements in the specifications document and offered the highest price.	41

1.18.2.	NOTIFICATION OF AWARD	42

1.18.3.	SIGNING THE CONTRACT AGREEMENT	42

1.18.4.	PERFORMANCE AND DBOT AGREEMENT SECURITY	43

2.	forms for Tender	44

3.	LETTER OF ACCEPTANCE	72

4.	Contract Agreement	76

5.	DBOT AGREEMENT	80

-	PART ONE	84

-	PART TWO	91

-	PART THREE	98

-	PART FOUR	101

-	PART FIVE	105

-	PART SIX	107

-	PART SEVEN	111

6.	PROJECT REPORT	115

6.1.	GENERAL	116

6.2.	PROJECT ASSUMPTION: MINIMUM SILO SIZE	116

6.3.	PORT ENGINEERING	116

7.	CONTRACTING AUTHORITY’S REQUIREMENTS	118

7.1.	NATURAL CONDITIONS	119

7.2.	THE CONTRACTOR OBLIGATIONS	119

7.3.	PROJECT OBLIGATIONS AND CONSTRAINTS	119

7.4.	SILO CHARACTERISTICS	119

7.5.	PRINCIPLE OF OPERATION AND UTILITIES	120

7.6.	EQUIPMENT	120

7.7.	ELECTRICAL POWER SUPPLIES	120

7.8.	UTILITIES	120

7.9.	OPERATION AND MAINTENANCE	121

7.9.1.	PERSONNEL	121

7.9.2.	MAINTENANCE AND REPAIR	121

7.10.	REGULATIONS	122

8.	TECHNICAL SPECIFICATIONS	123

8.1.	GENERAL	124

8.2.	STANDARDS, CODES OF PRACTICE AND REFERENCES	124

8.3.	RESPONSIBILITIES OF THE DBOT AGREEMENTNAIRE FOR THE WORKS EXECUTION	126

8.4.	PROVISIONS	128

8.5.	INSPECTION	128

8.6.	SCHEDULE FOR THE WORKS	128

8.7.	SUBMITTALS	128

8.8.	SITE SPECIFICATION	129

8.8.1.	GENERALITY	129

8.8.2.	CLIMATIC DATA	130

8.8.3.	SOME GRAINS DATA FOR BULK STORAGE	133

8.8.4.	CONFORMITY TO ATEX REGULATIONS	133

8.8.5.	HANDLING EQUIPMENT AND WEIGHT BRIDGES	134

8.8.6.	STEEL COMPONENTS	134

8.9.	MATERIALS	135

8.9.1.	CEMENT	135

8.9.2.	AGGREGATE	137

8.9.3.	REINFORCEMENT	140

8.9.4.	WATER	141

8.9.5.	ADMIXTURES	141

8.10.	DELIVERY AND STORAGE	142

8.10.1.	CONCRETE GRADE	143

8.10.2.	TRIAL MIXES	146

8.10.3.	QUALITY CONTROL	147

8.10.4.	BATCHING	149

8.10.5.	CONTROL OF CHLORIDES AND SULPHATES	150

8.10.6.	PLACING	151

8.10.7.	FORMWORK	155

8.10.8.	REINFORCEMENT	156

8.10.9.	JOINTS	158

8.10.10.	CURING AND PROTECTION	159

8.10.11.	FINISHES	160

8.10.12.	SPECIAL CONCRETE	161

8.10.13.	PROTECTIVE COATINGS	163

8.10.14.	TOLERANCES	165

8.10.15.	CONCRETE SURFACE TREATMENT	165

8.10.16.	LIST OF TEST REQUIREMENT	166

APPENDICES	172

# INSTRUCTIONS TO TENDERERS

تعليمات للعارضين - ١

### OBJECT OF THE TENDER

Port of Tripoli

Office d'Exploitation du Port de Tripoli

Tel: 961 6 220180

Tripoli, El Mina

LEBANON

Hereinafter called the “Contracting Authority” wishes to supplement the Port of Tripoli's facilities by implementing and operating silo for grain storage within the Port Terminal, in the location specified in the attached drawings.

According to Article 13 of the Public Procurement Law, the Contracting Authority must update its estimates for the value of the project before announcing it, except in exceptional cases where this is not possible, in which case the reasons must be clearly and duly explained.

The Contracting Authority prepares the estimated value of the project based on realistic market prices and by gathering prices from other departments. This is done after considering any increase that may arise from applying the optional items noted in the specifications document (for example, increases in quantities, incentives, etc.).

Regarding framework agreements, the maximum estimated value of all expected contracts over the duration of the agreement is taken into account.

When the project is carried out in groups, the estimated value of all the groups that make up the commitment is calculated.

The estimated value of the project must be determined by the Contracting Authority at the time of its announcement or when starting the tendering procedures.

It is prohibited for the Contracting Authority to calculate the estimated value of the procurement project to be less than or greater than its actual value in order to avoid the application of the provisions of the law.

When calculating the estimated value of the project, all necessary elements for its execution must be considered.

The estimated value of the project must remain confidential before and after the contract award, unless the Contracting Authority deems otherwise for purposes related to competition and ensuring the success of the contract, or in transactions where prices are clearly known. In such cases, the estimated value is disclosed according to the proper procedures. In any case, confidentiality cannot be claimed before the Court of Audit and controlling authorities.

The Ministry of Economy and Trade shall have the full right to control and supervise all grain silo operations related to food security as per the Lebanese regulations.

The required installations are:

- Grain silo with the following minimum characteristics, dimensions and capacities:

● 120,000T total capacity.

● 5,000T capacity per steel bin with a flat bottom.

● Possibility for storage of wheat, corn, barley, sunflower and other.

The grain silo must be completely operational and ready for use, according to international rules and standards and fully classified by recognized classification societies.

- Two ship unloaders (Minimum unloading capacity 200T/hr each).

- One ship loader (Minimum loading capacity 400T/hr).

- 9 flat bottom bins (1,500T each).

- Several loading truck stations.

- One unloading truck station.

- Necessary mooring and quay accessories for ship berthing in front of the mentioned silo.

- Auxiliary civil and building works that must be executed on the same land area of the silo, which will be occupied by the signatory of the DBOT Agreement and supplied by the Port of Tripoli Authority.

These new installations will be accomplished under a DBOT contract.

The signatory of the DBOT Agreement, who will be both investor and developer, will develop the site by fitting out and installing the silo, by executing all necessary auxiliary civil works and will then operate it for a predetermined time, on expiry of which all equipment, facilities, structures and constructions used and occupied by the Contractor will be transferred to the Port of Tripoli Authority knowing that all operations related to food security shall remain under the control and supervision of the Ministry of Economy and Trade as per the Lebanese regulations.

All design, construction, operation and maintenance works related to this Tender shall be executed according to international standards and regulations and following the Port requirements and guidelines.

١-١ موضوع الإلتزام

مرفأ طرابلس
مصلحة إستثمار المرفأ
00961 6 220180 هاتف: 
طرابلس، الميناء
لبنان

إن مصلحة إستثمار مرفأ طرابلس والتي يُشار إليها فيما بعد بـ "سلطة التعاقد" ترغب في تعزيز منشآت مرفأ طرابلس من خلال تنفيذ وتشغيل إهراءات لتخزين الحبوب ضمن مساحات المرفأ، في الموقع المحدد في الخرائط المرفقة.

وفقاً للمادة 13 من قانون الشراء العام، يجب على سلطة التعاقد تحديث تقديراتها لقيمة المشروع قبل الإعلان عنه إلا في الحالات الإستثنائية التي يتعذَّر فيها ذلك، على أن تُبيِّن الأسباب بشكل معلَّل وواضح.

تُعِدّ سلطة التعاقد القيمة التقديرية للـمشروع استناداً إلى أسعار السوق الواقعية وتَقصّي الأسعار لدى الإدارات الأخرى، وبعد الأخذ بالاعتبار كل زيادة يمكن أن تَنتُج عن تطبيق البنود الاختيارية الـملحوظة في دفتر الشروط (على سبيل الـمثال زيادة في الكميات، الحوافز، الخ...).

فيما يتعلّق باتفاقات الإطار، تؤخذ بالاعتبار القيمة التقديرية القصوى لـمجمل العقود الـمتوقّعة طوال فترة الاتفاق.

عند إجراء المشروع على أساس مجموعات، يتمّ احتساب القيمة التقديرية لكافة الـمجموعات التي يتألف منها الالتزام.

يجب أن تكون قيمة المشروع التقديرية محدَّدة لدى سلطة التعاقد عند الإعلان عنها أو عند البدء بإجراءات التلزيم.

يُحظَّر على سلطة التعاقد احتساب قيمة مشروع الشراء التقديرية بأقل أو أكثر من قيمته الفعلية بهدف تفادي تطبيق أحكام القانون.

عند احتساب القيمة التقديرية للـمشروع يجب الأخذ بالاعتبار كافة العناصر اللازمة لتنفيذها.
يجب أن تبقى القيمة التقديرية للـمشروع سرّية قبل التلزيم وبعده، إلا إذا رأت سلطة التعاقد عكس ذلك لغايات تتعلَّق بالـمنافسة والحرص على نجاح التلزيم، أو في الصفقات الـمعلومة أسعارها بشكل جليّ وواضح، وعندها يُعلَن عن القيمة التقديرية وفق الأصول. في جميع الأحوال لا يُحتَجّ بالسرية أمام ديوان الـمحاسبة والهيئات الرقابية.

سوف يكون لوزارة الاقتصاد والتجارة الحقّ الكامل بمراقبة والإشراف على جميع عمليات إهراءات الحبوب والمتعلقة بالأمن الغذائي وفقًا للقوانين اللبنانية.

التجهيزات المطلوبة هي:

- صوامع حبوب بالخصائص، الأبعاد والسعات الدنيا التالية: 
●  سعة إجمالية ١٢٠,٠٠٠ طن.
●  سعة ٥,٠٠٠ طن لكل صومعة فولاذية ذات قاع مسطح.
●  إمكانية تخزين القمح، الذرة، الشعير، دوار الشمس وغير ذلك.

يجب أن تكون إهراءات الحبوب جاهزة تمامًا للعمل وفقًا للقواعد والمعايير الدولية ومصنفة بالكامل من قبل هيئات التصنيف المعترف بها.

- جهازان لتفريغ السفن (قدرة تفريغ لا تقلّ عن ٢٠٠ طن/ساعة لكل منهما).

- جهاز واحد لتحميل السفن (قدرة تحميل لا تقلّ عن ٤٠٠ طن/ساعة).

- حاويات ذات قاع مسطح عدد ٩ (سعة ١,٥٠٠ طن لكل منها).

- عدة محطات لتحميل الشاحنات.

- محطة واحدة لتفريغ الشاحنات.

- الملحقات اللازمة لرسو البواخر على الرصيف أمام الإهراءات المذكورة.

- الإنشاءات والمباني الملحقة التي يجب تنفيذها على نفس المساحة الأرضية المخصصة للإهراءات، والتي سيشغلها موقّع إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية والموضوعة بتصرفه من قبل مرفأ طرابلس.

سيتم تنفيذ هذه المنشآت الجديدة بموجب إتفاقية (DBOT) تصميم وإنشاء وتشغيل ونقل ملكية.

يقوم موقّع إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية، الذي سوف يكون مستثمراً ومطوراً، بتطوير الموقع من خلال تجهيز وتركيب الإهراءات، وتنفيذ جميع الأعمال الإنشائية الملحقة اللازمة، ثم تشغيلها لمدة محددة، حيث وبعد إنقضاء هذه الفترة، يتم نقل جميع المعدات، المنشآت، الإنشاءات والأبنية التي استخدمها وشغلها المتعاقد إلى إدارة مرفأ طرابلس مع العلم أن جميع العمليات المتعلقة بالأمن الغذائي ستظل تحت مراقبة وإشراف من وزارة الإقتصاد والتجارة وفقًا للقوانين اللبنانية.
يجب تنفيذ جميع أعمال التصميم، البناء، التشغيل والصيانة المتعلقة بهذا الإلتزام وفقًا للمعايير والقوانين الدولية مع إتباع متطلبات وإرشادات المرفأ.

### DEFINITIONS, LAW AND LANGUAGE

### DEFINITIONS

The following words and expressions shall have the meanings stated in the DBOT Agreement and as stated hereafter, except where in the context another meaning is obviously required:

“Approved” and “Approval” means approved in writing, including subsequent written confirmation of previous verbal approval.

“Certificate of completion” means with respect to the Permanent Works, the certificate issued when the entire Permanent Works are complete, and the issuance of such certificate shall constitute evidence of the Contracting Authority’s acceptance or “reception” of the Permanent Works.

“Completion” or “Complete(d)” means the full execution and performance of the design, supply, execution, construction and testing of the entire Permanent Works, in strict conformity in all respects with the Contract.

“Contracting Authority” or “Employer” means the “Office d’Exploitation du Port de Tripoli” named as Contracting Authority in the DBOT Agreement.

“Contractor” is the signatory of the DBOT Agreement which means the company named as the beneficiary of the DBOT Agreement.

“Consultant Engineer“ or “Engineer“ means the entity notified or which will be notified to the Contractor by the Contracting Authority, appointed by the Contracting Authority or not according to the article 31 of the Public Procurement Law to act as the Consultant Engineer in connection with the performance and completion of the Works and whose principal duties will be to act as a technical advisor to the Contracting Authority, to review contract submittals to the extent deemed necessary by the Contracting Authority and to verify that the signatory of the DBOT Agreement’s design, construction methods, construction, operation and maintenance of the Works are in accordance with the requirements and with all applicable norms, standards and specifications.

“Contract” means the DBOT Agreement, the Contracting Authority's Requirements, the Tender, and the further Documents (if any) which are listed in the Contract Agreement.

“Contractor staff” means those directors, officers, employees, agents and servants employed by the Contractor for time to time in the performance of its duties and obligations in relation to the Works, including, without limitation, the Engineer, and Project Superintendents.

“Construction Materials” means the equipment, fixtures, goods, building systems, building materials and other thing to form part of or to be incorporated into any part of the Permanent Works.

“Contract Submittal” means any submittal if any kind which the Contractor is required under the Contract to submit to the Contracting Authority’s and/or Construction Manager’s review and Approval, including, without limitation, material samples, drawings and specifications during the development of the design, working drawings and final specifications, shop drawings, coordination drawings, other drawings, catalog cuts design calculations. Test results, as-built Documentation, schedules, method statements, Detailed Program and the like as required under any of the Contract Documents.

“Drawings” means the drawings forming part of the Tender and any additions or modifications to, or clarifications of, such drawings made by the Contracting Authority and / or the Contractor and Approved by the Contracting Authority or on its behalf by the Consultant Engineer.

“Letter of Guarantee” means any letter of guarantee provided or to be provided by the Contractor under and/or in connection with the Contract in favor of the Contracting Authority, including, without limitation, the Advance Payment Letter of Guarantee (if existing). Any of the Performance Letters of Guarantee, any of the Conditional Letters of Guarantee, the Interim Letter Guarantee and any of the Retention Letters of Guarantee, may be modified from time to time in accordance with Contract especially regarding the modification of works and requirements.

“Notice to Proceed” means the notice to be issued by the Contracting Authority to the signatory of the DBOT Agreement, instructing the Contractor to commence performance of all, or, such part only, of the Works, as shall be specified in such notice.

“OEPT” means Office d’Exploitation du Port de Tripoli.

“Permanent Works” means everything of a permanent nature, including, without limitation, the Construction Materials, to be designed, supplied, executed, constructed, completed, tested and guaranteed under the Contract.

“Public Authority” means any governmental, regional, or municipal entity or agency or any individual representing any such entity or agency.

“Requirements” means any and all design and/or technical criteria and/or descriptions and/or requirements set forth in the Tender Documents or elsewhere in the Contract, including, without limitation, those relating to the Works, Construction Materials, equipment, design, design systems and/or methods, construction, construction systems, construction methods and all other information describing and/or required for the Completion and guarantee of the Works and forming part of the Contract and any modification thereof or addition thereto as may from time to time be furnished by the Contracting Authority and/or the Contractor and Approved by the Contracting Authority, or, which may be agreed by the parties.

“Scope of Works” means the scope of Works and the Drawings set forth in the Tender Documents.

“Site” means the land and/or sea-bed and/or reclaimed land in, on or through which the Permanent Works are to be constructed, the boundaries of which are identified on the attached Drawings, and/or any other land and places which may be from time to time provided in writing or Approved at its entire discretion by the Contracting Authority to the Contractor for working space.

“Subcontractor” means any person, firm or legal entity (other than a Supplier or an individual who is a member of the signatory of the DBOT Agreement’s staff or any Supplier’s staff), at any time, retained or employed by the Contractor to perform any part of the Works.

“Supplier” means any person, firm or legal entity which sells or otherwise provides Construction Materials to the Contractor or any Subcontractor or otherwise undertakes to assist the Contractor or any Subcontractor in the performance of any part of the Works, but which, in each such case, does not perform other than incidental or immaterial services on the Site with respect thereto.

“Temporary Works” means all Temporary Works of every kind required in or about the performance of the Works.

“Tender” or “Bid” means the Tenderer's signed offer for the Contracting Authority's Requirements and all other Documents which the Contracting Authority submitted there with as included in the Contract.

“Works” means the Permanent Works and everything whether of a temporary nature (including the Temporary Works and Construction Plant) or of a permanent nature required in or for the Completion and guarantee of the Permanent Works, so far as the necessity thereof is specified in or may reasonably be inferred from the Contract or is otherwise required for the Completion and guarantee of any part of the Permanent Works.

١-٢ التعريفات، القانون واللغة
١-٢-١ التعريفات

تكون الكلمات والعبارات التالية ذات المعاني المحددة في إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية كما هو موضح أدناه، إلا إذا كان المعنى الآخر مطلوبًا بوضوح في السياق:

"موافق عليه" و"الموافقة" تعني الموافقة كتابيًا، بما في ذلك التأكيد الكتابي اللاحق للموافقة الشفهية السابقة.

"شهادة الإنجاز" تعني بالنسبة للأعمال الدائمة، الشهادة التي يتم إصدارها عندما تكتمل جميع الأعمال الدائمة، وإصدار هذه الشهادة يمثل دليلًا على قبول سلطة التعاقد أو "إستلام" الأعمال الدائمة.

"الإنجاز" أو "مكتمل" يعني التنفيذ الكامل والأداء للتصميم، التوريد، التنفيذ، البناء والاختبار لجميع الأعمال الدائمة، بما يتوافق بدقة من جميع النواحي مع العقد.

"سلطة التعاقد" أو "صاحب العمل" تعني "مصلحة استثمار مرفأ طرابلس" الذي تمّ تسميته كسلطة مانحة أو سلطة التعاقد في إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية.

"المتعاقد" هو موقّع إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية أي الشركة التي تم تسميتها كملتزمة في إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية.

"المهندس الإستشاري" أو "المهندس" تعني الكيان الذي يتم إعلام أو سيتم إعلام المستثمر بإسمه من قبل سلطة التعاقد، والذي تم تعيينه من قبل سلطة التعاقد أو من خارجها وفقاً لأحكام المادة 31 من قانون الشراء العام للعمل كمهندس إستشاري فيما يتعلق بأداء وإتمام الأعمال، وتتمثل واجباته الرئيسية في العمل كمستشار تقني لسلطة التعاقد، مراجعة مستندات العقد إلى المدى الذي يراه سلطة التعاقد ضروريًا، التحقق من أن تصميم المستثمر، طرق البناء، البناء، التشغيل وصيانة الأعمال تتوافق مع المتطلبات ومع جميع المعايير والمواصفات المعمول بها.

"العقد" يعني إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية، متطلبات سلطة التعاقد، الإلتزام والمستندات الأخرى (إن وجدت) المدرجة في اتفاقية العقد.

"موظفو المتعاقد" يعني المدراء، الموظفين، العاملين، الوكلاء والأجراء الذين يعملون لدى المتعاقد من حين لآخر في أداء واجباته والتزاماته المتعلقة بالأعمال، بما في ذلك، على سبيل المثال لا الحصر، المهندس، ومدراء المشروع.

"مواد البناء" تعني المعدات، الأدوات، السلع، الأنظمة الإنشائية، مواد البناء وغيرها من الأشياء التي تشكل جزءًا من الأعمال الدائمة أو التي سيتم دمجها في أي جزء من الأعمال الدائمة.

"المستندات للتقديم بموجب العقد" يعني أيّ مستندات للتقديم من أي نوع يطلب من المتعاقد تقديمها بموجب العقد لسلطة التعاقد و/أو مدراء المشروع للمراجعة والموافقة عليها، بما في ذلك، على سبيل المثال لا الحصر، عينات المواد، الخرائط، المواصفات خلال تطوير التصميم، الخرائط أثناء العمل والمواصفات النهائية، خرائط التنفيذ، خرائط التنسيق، والخرائط الأخرى، الكتالوجات وحسابات التصميم، نتائج الاختبارات، وثائق الأعمال المنجزة، الجداول الزمنية، طرق التنفيذ، البرنامج التفصيلي وما شابه ذلك كما هو مطلوب في أي من مستندات العقد.

"الرسومات" تعني الرسومات التي تشكل جزءًا من الإلتزام وأي إضافات أو تعديلات على هذه الرسومات أو توضيحات لها من قبل سلطة التعاقد و/أو المتعاقد والموافق عليها من قبل سلطة التعاقد أو من قبله بواسطة المهندس الاستشاري.

"كتاب الضمان" يعني أي كتاب ضمان يُقدمه أو سيتم تقديمه من قبل المتعاقد بموجب العقد لصالح سلطة التعاقد، بما في ذلك، على سبيل المثال لا الحصر، ضمان الدفعة على الحساب (إن وجدت). يمكن تعديل أي من كتب ضمان حسن التنفيذ، أو كتب الضمان المشروطة، أو كتاب الضمان المؤقت وأي من كتب الضمان للتوقيفات وفقًا للعقد وبالأخص لناحية تعديل الأعمال والشروط المطلوبة.

"إشعار البدء" يعني الإشعار الذي يجب أن يصدره سلطة التعاقد لموقّع إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية، للبدء بتنفيذ جميع الأعمال أو أي جزء فقط من الأعمال كما هو مذكور في هذا الإشعار.

"OEPT"  تعني مصلحة إستثمار مرفأ طرابلس.

"الأعمال الدائمة" تعني كل شيء ذو طابع دائم، بما في ذلك، على سبيل المثال لا الحصر، مواد البناء، التي سيتم تصميمها، توريدها، تنفيذها، بناؤها، واختبارها وضمانها بموجب العقد.

"السلطة العامة" تعني أي كيان حكومي أو إقليمي أو بلدي أو أي وكالة أو أي شخص يمثل أيًا من هذه الكيانات أو الوكالات.

"المتطلبات" تعني أي وكل المعايير أو المواصفات الفنية و/أو المتطلبات الواردة في مستندات الإلتزام أو في أي مكان آخر في العقد، بما في ذلك، على سبيل المثال لا الحصر، تلك المتعلقة بالأعمال، مواد البناء، المعدات، التصميم، انظمة التصميم و/أو الطرق، البناء، طرق البناء وكل المعلومات الأخرى التي تصف و/أو تتطلب لإنجاز وضمان الأعمال وتشكل جزءًا من العقد وأي تعديل لها أو إضافة إليها قد يتم توفيرها من قبل سلطة التعاقد و/أو المتعاقد والموافقة عليها من قبل سلطة التعاقد، أو التي قد يتم الاتفاق عليها من قبل الأطراف.

"نطاق الأعمال" يعني نطاق الأعمال والرسومات الواردة في مستندات الإلتزام.

"الموقع" يعني الأرض و/أو قاع البحر و/أو الأرض المستصلحة التي يتم فيها أو من خلالها تنفيذ الأعمال الدائمة، والحدود المحددة في الرسومات المرفقة، و/أو أي أرض وأماكن قد يتم توفيرها من وقت لآخر كتابيًا أو معتمدة من قبل سلطة التعاقد بالكامل لإنجاز المتعاقد العمل.

"المقاول الثانوي" يعني أي شخص أو شركة أو كيان قانوني (بخلاف المورد أو الفرد الذي هو من موظفي المتعاقد أو أي موظف للمورد)، يتم توظيفه أو الإحتفاظ به من قبل المتعاقد لأداء أي جزء من الأعمال.

"المورد" يعني أي شخص أو شركة أو كيان قانوني يبيع أو يقدم مواد البناء للمتعاقد أو أي مقاول ثانوي أو يتعهد بمساعدة المتعاقد أو أي مقاول ثانوي في أداء أي جزء من الأعمال، ولكن في كل حالة، لا يؤدي سوى خدمات عرضية أو غير مهمة في الموقع بخصوص ذلك.

"الأعمال المؤقتة" تعني جميع الأعمال المؤقتة من جميع الأنواع المطلوبة في أو حول أداء الأعمال.

"العرض" يعني العرض الموقع من قبل المتقدم على الإلتزام وفقا لمتطلبات سلطة التعاقد ولجميع المستندات التي قدمها سلطة التعاقد مع هذا العرض والتي تعتبر جزءًا من العقد.

"الأعمال" تعني الأعمال الدائمة وكل شيء سواء من الطابع المؤقت (بما في ذلك الأعمال المؤقتة ومعدات البناء) أو من الطابع الدائم المطلوب في أو من أجل الإنجاز وضمان الأعمال الدائمة، طالما أن ضرورة ذلك محددة في العقد أو يمكن استنتاجها بشكل معقول من العقد أو تتطلب من أجل الإنجاز وضمان أي جزء من الأعمال الدائمة.

### LANGUAGE

Arabic and English languages shall be the languages used for all correspondence and Tender Documents. Language shall be as stated in the DBOT Agreement.

In all cases and at any time, the Contractor must supply to the Contracting Authority, in reasonable time, at the Contracting Authority verbal or writing request, an Arabic translation, of any document previously prepared and submitted by the Contractor, whatever nature or destination is. In the event of any dispute as to the meaning between the Arabic and English texts, the Arabic version shall prevail.

١-٢-٢ اللغة

تكون اللغتين العربية والإنجليزية هما اللغتين المستخدمتين لجميع المراسلات ومستندات الإلتزام. ستكون اللغة كما هو مذكور في إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية.

في جميع الحالات وفي أي وقت، يجب على المتعاقد أن يزود سلطة التعاقد، في وقت معقول، بناءً على طلب سلطة التعاقد شفهيًا أو كتابيًا، بترجمة إلى اللغة العربية لأي مستند تم إعداده مسبقًا وتقديمه من قبل المتعاقد، بغض النظر عن طبيعته أو وجهته. في حال حدوث أي نزاع حول المعنى بين النصوص العربية والإنجليزية، تسود النسخة العربية.

### LAW

The Contract shall be governed by and construed in all respects in accordance with the laws of the Republic of Lebanon.

This Document, entitled “Instructions to Tenderers” shall be considered as a part of the DBOT Agreement.

١-٢-٣ القانون

يخضع العقد ويفسر من جميع النواحي وفقًا لقوانين الجمهورية اللبنانية ومن ضمنها قانون الشراء العام.

يعتبر هذا المستند، بعنوان "تعليمات للعارضين"، جزءًا من إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية.

### ELIGIBITY

### ELIGIBLE TENDERERS

This invitation to Tender is open to all eligible firms of the type of joint-stock companies or their equivalents under foreign laws.

Tenderers shall provide evidence of their eligibility to the satisfaction of the Contracting Authority as requested according to article 7 of the Public Procurement Law and to the conditions of participation stipulated in articles 7 and 8 of the said law.

The Tenderers, including all members of a Joint Venture and all subcontractors shall not be affiliated with a firm or entity:

- That has provided consulting services related to the Works to the Contracting Authority during the preparatory stages of the Works,

- That has been hired (or is proposed to be hired) by the Contracting Authority as Engineer or Consultant for the Contract.

١-٣ الأهلية

١-٣-١ العارضون المؤهلون

هذه الدعوة لتقديم العروض مفتوحة لجميع الشركات المؤهلة من نوع الشركات المساهمة أو ما يماثلها في القوانين الأجنبية.

يجب على العارضين تقديم دليل على أهليتهم بحسب متطلبات سلطة التعاقد وفقاً لأحكام المادة 7 من قانون الشراء العام وبحسب شروط المشاركة المنصوص عنها في المادتين 7 و8 من قانون الشراء العام.

يجب ألا يكون العارضون، بما في ذلك جميع العارضين الشركاء وجميع المقاولين الثانويين، مرتبطين بشركة أو كيان:

١ - قدمت خدمات إستشارية تتعلق بالأعمال لصالح سلطة التعاقد خلال مراحل التحضير للأعمال.

٢ - تم توظيفها (أو يتم اقتراح توظيفها) من قبل سلطة التعاقد كمهندس أو مستشار للعقد.

### ELIGIBLE MATERIALS, EQUIPMENT AND PLANT

All materials, Plant and Contractor’s Equipment, other supplies, and services to be supplied under the Contract, shall have their origin in determined eligible source countries as defined in the Lebanese regulations, and all expenditures made under the Contract will be limited to such materials, Plant or Contractor’s Equipment, other supplies, and services.

١-٣-٢ المواد، المعدات والمراكز المقبولة

يجب أن تكون جميع المواد، مراكز، المعدات الخاصة بالمتعاقد، الإمدادات الأخرى، الخدمات التي سيتم توفيرها بموجب العقد، من منشأ دول مقبولة ومحددة بحسب القوانين اللبنانية، وعلى جميع النفقات التي تم دفعها بموجب العقد أن تكون مقتصرة على هذه المواد، المراكز أو المعدات الخاصة بالمتعاقد، الإمدادات الأخرى والخدمات.

### QUALIFICATION OF THE TENDERER

### APPROVED TENDERERS

To be qualified for award of the Contract, Tenderers shall provide the following information and Documents within their Tender.

Foreign companies shall not submit information and/or Documents mentioned in item (c) of this Clause.

All certificates should be originals or legally certified copies of the originals and valid on the date of the bid opening session, otherwise the Tenderers’ Tender will be rejected.

- Legally certified copies of original Documents defining the constitution or legal status, place of registration and principal place of business.

- Legally certified copy of the firm registration at the registry of Commerce.

- Legally certified copy of the firm registration at the VAT; A wining foreign company of this bid shall register a company in Lebanon, according to Lebanese regulations and accomplish all its work regarding this Tender through this company. Furthermore, all related administrative papers, including VAT registration, shall be submitted to the Client.

- The commercial broadcast belonging to the establishing company, if the offer is made by a company or institution, or the Power of Attorney duly notarized by the notary.

- A certificate issued by the municipality within whose jurisdiction the bidder’s head office is located, according to the certificate of registration in the commercial register, confirming that all due municipal fees have been fully paid. (For the Lebanese companies)

- A certificate issued by the court of competent jurisdiction (Commercial Register) proving that the bidder is not in a state of bankruptcy.

- A certificate issued by the court of competent jurisdiction (Commercial Register) proving that the bidder is not in a state of judicial liquidation.

- A copy of the identification cards (IDs/Passports) of the beneficial owner(s).

- A copy of the identification cards (IDs/Passports) of each person representing the bidder (those acting on behalf of the bidder in their relationship with the contracting authority: legal agent, representative of the legal entity, or who has Power of Attorney, etc.).

- A commitment to waive banking secrecy pursuant to Decision No. 17 dated 12/05/2020 issued by the Council of Ministers.

- A copy of the company’s Articles of Incorporation, if applicable.

- The legal and administrative tender document, initialed and signed on all its pages by the bidder, with signature and stamp.

- The technical specifications document, initialed and signed on all its pages by the bidder, with signature and stamp.

- A written power of attorney authorizing the signatory of the Tender to commit the Tender

- Receipt for purchase of the Tender Documents.

- Certificate of quittance issued by the National Social security Fund. Its validity extending beyond the Tender opening date; Similar for foreign companies in their countries of origin.

- List of silos projects executed and operated by the Tenderer, in Lebanon and/or abroad, with duly certified copies of support Documents (Works starting and handing over certificates, operation time…), to be provided by the Clients, listing in minimum the above-mentioned works nature and amount, in addition to silos operation.

These latest works will be taken in account by the evaluation committee.

- In any case, the Tenderer must show his capacity to undertake the execution and operation of all civil works and silos, either through his own team or through a nominated subcontractor (knowing that subcontracted works amount must not exceed 50% of the total works amount), and which experience is to be shown by certificates mentioning at least the following, within the past fifteen (15) years (year 2024 included):

- Construction and implementation of at least one steel grain silo plant with a minimum capacity of 150,000T with all auxiliary civil and MEP works.

- Operating and managing one or several grain silos with a minimum total capacity of 1,000,000T over these fifteen years.

- Statement of non-exclusion to participate in biddings from Port of Tripoli Authority (OEPT) according to article 7 of the Public Procurement Law.

- Major items of Construction equipment proposed for carrying out this Contract.

- The names and CVs of key personnel proposed for the administration and execution of this Contract, both on and off site. If the Contractor is awarded the Contract, he shall obtain the Contracting Authority approval on the CVs and key personnel list and all remaining employees prior to begin working on site.

Every Lebanese engineer, whether he is a Contractor or an Engineer employed by a company, or establishment classified on the basis of his employment thereby, shall enclose in his Tender a certificate confirming his membership to any one of the two Orders of Engineers in Lebanon for the year in which the Tender is submitted.

- Proposals for subcontracting any elements of the Works (knowing that subcontracted works amount must not exceed 50% of the total works amount). Tenderers should refer to related Sub-clauses of the Conditions of Contract.

- Manufacturers list.

- Bank certificate for access to liquid assets and/or evidence of access to or availability of credit facilities for a minimum amount of 50% of the Tenderer proposal.

- Information regarding any current litigation in which the Tenderer is involved.

- Program of Work and Method Statement in sufficient detail to demonstrate the adequacy of the Tenderer’s proposals to meet the Technical Specifications and the completion time. No alterations or changes can be made to the Specifications.

- Contractual commitment completed, stamped and signed according to the forms of Tender.

- Certificate indicating the permanent address of the Tenderer.

- Any Additional Documents for qualification of the Tenderer that may be required.

- A legal record of the authorized signatory or his legal representative, dated no more than three months from the date of the awarding session, free of any disgraceful judgment.

- The Joint Venture contract certified by the public notary, if required.

- Tenderer registration certificate at the Ministry of Finance - Directorate of incomes.

- A comprehensive statement issued by the Commercial Registry showing: Founders - Members - Shareholders - Authorized Signatories - Manager - Capital – Tenderer Activity - Commercial Incidents.

- A statement issued by the competent court (commercial registry) proving that the Tenderer is not in a state of bankruptcy or liquidation.

- Integrity declaration document duly signed by the bidder.

- The obligation to declare the economic right of the Tenderer.

For foreign companies, all the above required Documents must be duly certified at their Countries Embassies in Lebanon, in addition to the Ministry of Foreign Affairs in Lebanon.

The right for objection and/or request for any claim whatever nature is, shall be made according to the Public Procurement Law in Lebanon of N244 dated July 29, 2021, Section 7 (Right for objection).

Furthermore, all Tenderers shall submit a certificate from the Ministry of Economy and Commerce in Lebanon proving the applicability of the provisions of the Israeli enemy law to the Tenderer. The date of its ratification does not go back more than three months from the date of the DBOT Agreement session.

In the event that an offer is submitted by a foreign company, this company must take into account one of the following conditions:

- To be part of a consortium that includes at least one Lebanese company that fulfills the conditions required by this Tender Document.

- The personal presence of the legal representative of the company in the bidding procedures.

- To have an agent or representative in Lebanon assigned to sign the contract on its behalf.

١-٤ تأهل العارض

١-٤-١ العارضون المقبولون

لكي يكون العارض مقبولا للحصول على العقد، يجب عليه أن يقدم المعلومات والمستندات التالية ضمن عرضه.
لن تقدم الشركات الأجنبية المعلومات و/أو المستندات المذكورة في البند (ج) من هذه المادة.
يجب أن تكون جميع الشهادات أصلية أو نسخًا طبق الأصل مصادق عليها قانونيًا وصالحة بتاريخ جلسة فض العروض، وإلا سيتم رفض عرض المقدم من العارض.

أ) نسخ من المستندات الأصلية مصادق عليها قانونيًا والتي تحدد الموضوع أو الوضع القانوني، مكان التسجيل، والمكان الرئيسي للشركة.

ب) نسخة مصادق عليها قانونيًا من تسجيل الشركة في السجل التجاري.

ج) نسخة مصادق عليها قانونيًا من تسجيل الشركة في ضريبة القيمة المضافة؛ يجب على الشركة الأجنبية الفائزة بهذا العطاء أن تقوم بتسجيل شركة في لبنان وفقًا للقوانين اللبنانية، وتنفيذ جميع أعمالها المتعلقة بهذا الإلتزام من خلال هذه الشركة.

د) الإذاعة التجارية العائدة للشركة/المؤسسة إذا كان العرض بإسم شركة أو مؤسّسة أو التفويض بالتوقيع مصدّقاً حسب الأصول لدى كاتب العدل.

ه) إفادة صادرة عن البلدية التي يقع المركز الرئيسي للعارض ضمن نطاقها بحسب شهادة التسجيل في السجل التجاري، تفيد بأنه سدد كامل الرسوم البلدية المتوجبة عليه. (للشركات اللبنانية)

و) إفادة صادرة عن المحكمة المختصة (السجل التجاري) تُثبت أن العارض ليس في حالة إفلاس.

ز) إفادة صادرة عن المحكمة المختصة (السجل التجاري) تُثبت أن العارض ليس في حالة تصفية قضائية.

ح) نسخة عن بطاقات التعريف (هوية / جواز سفر) لصاحب (أصحاب) الحق الاقتصادي.

ط) نسخة عن بطاقات التعريف (هوية / جواز سفر) لكل شخص يمثل العارض (من ينوب عن العارض في علاقته مع سلطة التعاقد: وكيل قانوني، ممثل الشخص المعنوي أو المفوّض بالتوقيع عنه...).

ي) تعهــــــّـــــد برفع السرية المصرفية سنداً للقرار رقم 17 تاريخ 12/5/2020 الصادر عن مجلس الوزراء.

ك) نسخة عن عقد تأسيس الشركة في حال وجودها.

ل) دفتر الشروط القانوني والإداري مؤشّرٌ وموقــــّعٌ على جميع صفحاته بإمضاء وختم العارض.

م) دفتر المواصفات الفنية مؤشّرٌ وموقــــّعٌ على جميع صفحاته بإمضاء وختم العارض.

بالإضافة إلى ذلك، يجب تقديم جميع الأوراق الإدارية ذات الصلة، بما في ذلك التسجيل في ضريبة القيمة المضافة، إلى سلطة التعاقد.

ن) توكيل مكتوب يُخوّل الموقع على الإلتزام ربح الإلتزام.

س) إيصال شراء مستندات الإلتزام. (في حال وجوده)

ع) شهادة براءة ذمة صادرة عن الصندوق الوطني للضمان الاجتماعي. يجب أن تكون صالحة الى ما بعد تاريخ فتح العروض؛ ينطبق نفس الموضوع على الشركات الأجنبية في دولها الأصلية.

ف) قائمة بمشاريع الإهراءات التي نفذها وأدارها المتعاقد، في لبنان و/أو في الخارج، مع نسخ من المستندات الداعمة مصادق عليها قانونيا (شهادات بدء الأعمال وتسليمها، وقت التشغيل...)، على أن يتم توفيرها من قبل أصحاب العمل، توضح على الأقل نوع الأعمال المنوه عنها أعلاه وقيمتها، بالإضافة إلى تشغيل الإهراءات.

سيتم أخذ هذه الأعمال الأخيرة في الإعتبار من قبل لجنة التقييم.

ص) في جميع الحالات، يجب على العارض أن يظهر قدرته على تنفيذ وتشغيل جميع الأعمال الإنشائية والإهراءات، إما من خلال فريقه الخاص أو من خلال مقاول ثانوي معين (مع العلم أن قيمة الأعمال الملزمة لمقاولين ثانويين يجب ألا تتجاوز 50% من إجمالي قيمة الأعمال)، على أن تُبين خبرته بواسطة شهادات تذكر على الأقل ما يلي، وذلك خلال الخمس عشرة (15) سنة الماضية (بما في ذلك عام 2024):

- بناء وتنفيذ محطة إهراءات حبوب معدنية واحدة على الأقل بسعة لا تقل عن 150,000 طن مع جميع الأعمال الإنشائية والإلكتروميكانيكية الملحقة.

- تشغيل وإدارة إهراء أو عدة إهراءات حبوب بسعة إجمالية لا تقل عن 1,000,000 طن خلال هذه السنوات الخمس عشرة.

ق) إفادة بعدم الاقصاء من المشاركة في الإلتزام صادرة من قبل مصلحة إستثمار مرفأ طرابلس (OEPT) تطبيقاً للمادة 7 من قانون الشراء العام.

ر) لائحة بالمعدات الرئيسية للبناء المقترحة لتنفيذ هذا العقد.

ش) الأسماء والسير الذاتية للموظفين الرئيسيين المقترحين لإدارة وتنفيذ هذا العقد، سواء في الموقع وخارجه. إذا تم منح المتعاقد العقد، يجب عليه الحصول على موافقة سلطة التعاقد على السير الذاتية وقائمة الموظفين الرئيسيين وجميع الموظفين المتبقين قبل البدء بالعمل في الموقع. يجب على كل مهندس لبناني، سواء كان مقاولًا أو مهندسًا يعمل لدى شركة أو مؤسسة مصنفة بناءً على عمله، أن يرفق في عرضه شهادة تؤكد عضويته في أي من نقابتي المهندسين في لبنان للسنة التي تم تقديم العرض فيها.

ت) إقتراحات المقاولة من الباطن لأي جزء من الأعمال (مع العلم أن قيمة الأعمال الملزمة لمقاولين ثانويين يجب ألا تتجاوز 50% من إجمالي قيمة الأعمال). يجب على العارضين الرجوع إلى المواد ذات الصلة في شروط العقد.

ث) قائمة بالمصنعين.

خ) شهادة بنكية للوصول إلى الأصول السائلة و/أو دليل على الوصول إلى أو توفر تسهيلات إئتمانية بمبلغ لا يقل عن 50% من عرض مقدم العروض.

ذ) معلومات بشأن أي قضايا قانونية حالية يكون العارض طرفًا فيها.

ض) برنامج العمل وطريقة العمل بتفاصيل كافية لإظهار صوابية مقترحات العارض لتلبية المواصفات الفنية ووقت التنفيذ. لا يمكن إجراء أي تعديلات أو تغييرات على المواصفات.

ظ) تعهدّ مكتمل، مختوم وموقع وفقًا لنماذج الإلتزام.

غ شهادة توضح العنوان الدائم للعارض.

أأ) أي مستندات إضافية لتأهيل العارض قد تكون مطلوبة.

أب) سجل قانوني للمخول بالتوقيع أو لممثله القانوني، بتاريخ لا يتجاوز ثلاثة أشهر من تاريخ جلسة منح العقد، خال من أي حكم.

أج) عقد الشراكة مصادق عليه من كاتب العدل، إذا لزم الأمر.

أد) شهادة تسجيل العارض في وزارة المالية - مديرية الإيرادات.

أه) بيان شامل صادر عن السجل التجاري يظهر: المؤسسين - الأعضاء - المساهمين - الموقعين المفوضين - المدير - رأس المال - نشاط العارض- الحوادث التجارية.

أح) بيان صادر عن المحكمة المختصة (السجل التجاري) يثبت أن العارض ليس في حالة إفلاس أو تصفية.

أط) نموذج تصريح النزاهة الموقعة من العارض.

أي) إلتزام بإعلان صاحب الحق الاقتصادي للعارض.

بالنسبة للشركات الأجنبية، يجب أن تكون جميع المستندات المطلوبة مصادق عليها قانونيًا في سفارات دولها في لبنان، بالإضافة إلى وزارة الخارجية اللبنانية.

يجب تقديم الإعتراض و/أو أي مطالبة مهما كانت طبيعتها وفقًا لقانون الشراء العام في لبنان رقم 244 بتاريخ 29 يوليو 2021، الفصل 7 (حق الاعتراض).

بالإضافة إلى ذلك، يجب على جميع العارضين تقديم شهادة من وزارة الإقتصاد والتجارة في لبنان تثبت تطبيق أحكام قانون العدو الإسرائيلي على العارض. يجب ألا يعود تاريخ تصديقها لأكثر من ثلاثة أشهر من تاريخ جلسة العقد.

في حال تم تقديم عرض من قبل شركة أجنبية، يجب أن تأخذ هذه الشركة بعين الاعتبار أحد الشروط التالية:

- أن تكون جزءًا من مجموعة يشمل شركة لبنانية واحدة على الأقل تفي بالشروط المطلوبة في هذا الإلتزام.

- الحضور الشخصي للممثل القانوني للشركة في إجراءات التلزيم.

- أن يكون لديها وكيل أو ممثل في لبنان مفوض للتوقيع على العقد نيابة عنها.

### EVIDENCE OF QUALIFICATION

All information and Documents required under the Sub-section 1.4.1 shall serve as evidence of qualification and of the Tender’s capability to carry out the Contract.

Unless specifically listed in the Letter of Acceptance or in the Contract Agreement, these information and Documents shall not from a Contract Document, but the Tenderer will remain responsible for the validity, authenticity, and correctness of the information provided.

Any Incorrect or fraud Document will lead to the direct rejection of the Tender.

١-٤-٢ دليل التأهل

يجب أن تعتبر جميع المعلومات والوثائق المطلوبة بموجب المادة ١-٤-١ دليلًا على التأهل وقدرة العارض على تنفيذ العقد.

ما لم يتم ذكره بشكل محدد في كتاب القبول أو في إتفاقية العقد، لن تشكل هذه المعلومات والوثائق مستندًا من مستندات العقد، ولكن يبقى العارض مسؤولاً عن صحة وموثوقية ودقة المعلومات المقدمة.

أي مستند غير صحيح أو مزور سيؤدي إلى رفض العرض مباشرة.

.

### JOINT VENTURES

Tenders submitted by a Joint Venture of two or more firms as partners shall comply with the following requirements:

- Only eligible firms are allowed to form a Joint Venture.

- The Joint Venture Agreement shall include all the information listed in Sub-Clause 1.4.1 above.

- The Joint Venture Agreement shall be certified at a by a public notary, signed by the Joint Venture partners and submitted with the Tender.

- One of the partners shall be nominated as being in charge and this authorization shall be evidence by submitting a power of attorney signed by legally authorized signatories of all the partners.

- The partner in charge shall be authorized to incur liabilities and receive instructions for and on behalf of any and all partners of the Joint Venture and the entire execution of the Contract including payment shall be done exclusively with the partner in charge.

- In case of a successful Tender, the Tenderers shall sign the Form of Agreement as to be legally binding on all partners. All partners of the Joint Venture shall be liable jointly and severally from the execution of the Contract in accordance with the Contract items.

- The Joint Venture Agreement cannot be formed after the submission of tenders.

The Joint Venture between companies may not change its composition or legal status after submitting the proposal or establishing the foundations. Moreover, no company in the Joint Venture has the right to change its capital except for an increase, and it is not permitted to transfer its shares to any person except the state during the contract period. It is also mandatory to maintain the entire workforce and its capabilities and qualifications to execute the contract.

١-٤-٣ الشراكات

يجب أن تلتزم العروض المقدمة من قبل شراكة بين شركتين أو أكثر بالمتطلبات التالية:

أ) يجب أن تكون الشركات المؤهلة فقط هي التي يحق لها تشكيل شراكة.

ب) يجب أن تتضمن إتفاقية الشراكة جميع المعلومات المدرجة في المادة ١-٤-١ أعلاه.

ج) يجب أن يتم المصادقة على إتفاقية الشراكة من قبل كاتب عدل، ويوقعها الشركاء ويتم تقديمها مع العرض

د) يجب تفويض أحد الشركاء كمسؤول ويتم إثبات هذا التفويض من خلال تقديم توكيل موقع من الممثلين القانونيين لجميع الشركاء.

هـ) يجب أن يكون الشريك المسؤول مفوضًا بتحمل المسؤوليات وإستلام التعليمات نيابة عن جميع الشركاء، ويجب أن يتم تنفيذ العقد بما في ذلك الدفع حصريًا مع الشريك المسؤول.

و) في حالة نجاح العرض، يجب على المتعهدين التوقيع على نموذج الإتفاقية بحيث يصبح العقد ملزمًا لجميع الشركاء. جميع الشركاء يكونون مسؤولين بشكل مشترك ومنفرد عن تنفيذ العقد وفقًا لبنود العقد.

ز) لا يمكن تشكيل التحالف بعد تقديم العروض.

ح) لا يجوز لتحالف الشركات تغيير تكوينه ووضعه القانوني بعد تقديم العرض أو تحديد الأسس. كما أنّه لا يحق لكلّ شركة من التحالف تغيير رأسمالها سوى زيادةً كما أنّه لا يحق التفرّغ عن أسهمها لأي شخص كان بإستثناء الدولة طوال فترة العقد، مع إلزامية المحافظة على كامل فريق العمل وقدراتها ومؤهلاتها لتنفيذ العقد.

### NUMBER OF TENDERS

A firm shall submit one Tender in the same Tendering process, either individually as a Tenderer or as a partner in a Joint Venture.

No firm can be a Subcontractor while submitting a Tender individually or as a partner of a Joint Venture in the same Tendering process.

A firm, if acting in the capacity of Subcontractor in the Tender, may not participate in more than one Tender. A Tenderer or a subcontractor who submits or participates in more than one Tender cause all the proposals in which the Tenderer has participated to be disqualified.

١-٤-٤ عدد العروض

يجب على الشركة تقديم عرض واحد فقط في نفس عملية الإلتزام، إما بشكل فردي كعارض أو كشريك ضمن مجموعة شركاء.

لا يجوز لأي شركة أن تكون متعهدًا ثانوياً أثناء تقديم العرض بشكل فردي أو كشريك ضمن مجموعة شركاء في نفس عملية الإلتزام.

إذا كانت تعمل الشركة بصفة متعهد ثانوي ضمن العرض، لا يجوز لها المشاركة في أكثر من عرض واحد. إذا قدم أو شارك عارض أو متعهد ثانوي بأكثر من عرض، سيتم إستبعاد جميع العروض التي شارك فيها العارض.

### TENDER DOCUMENTS

### CONTENT

The Tender Documents are those stated below, and should be read in conjunction with any Addendum issued in accordance with related Clauses.

- INSTRCUTIONS TO TENDEREERS.

- FORMS OF TENDERS.

- DBOT AGREEMENT.

- DRAWINGS.

- TECHNICAL SPECIFICATIONS.

١-٥ مستندات الإلتزام

١-٥-١ المحتوى

مستندات الإلتزام هي المستندات المذكورة أدناه ويجب قراءتها بالإشتراك مع أي ملحق يصدر وفقًا للمواد ذات الصلة.

١ -تعليمات للمتعهدين: نوع التلزيم، نطاق المشروع، مدته، طريقة إرساء التلزيم (الصفقة وأساسها)، سعر الافتتاح، نظام الغلاف.

٢ -نماذج العروض.

٣ -إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية.

٤ -الرسومات.

٥ -المواصفات الفنية.

### CONFIDENTIALLY AND EXAMINATION OF TENDER DOCUMENTS

Confidentiality and examination of Tender documents shall respect Article 6 of the Public Procurement Law N244 dated July 29, 2021. The Tender Documents shall not be used for any purpose other than that for which they are intended. The Tenderer shall examine carefully all Documents for completeness and clarity. If any part is missing or indistinct, he shall inform Contracting Authority. Failure to comply with the requirements of the Tender submission will be at the Tenderer/s own risk.

Pursuant to Clause 1.4.1, Tenders which are not substantially responsive to the requirements of the Tender Documents will be rejected.

١-٥-٢ السرية وفحص مستندات العروض

يجب أن يتم الحفاظ على سرية وفحص مستندات العروض وفقًا للمادة 6 من قانون الشراء العام رقم 244 بتاريخ 29 يوليو 2021. يجب عدم إستخدام مستندات الإلتزام لأي غرض آخر غير ما تم تحديده لها. يجب على العارض فحص جميع المستندات بعناية للتأكد من إكتمالها ووضوحها. إذا كانت هناك أجزاء مفقودة أو غير واضحة، يجب عليه إبلاغ سلطة التعاقد. سيكون عدم الإمتثال لمتطلبات تقديم العروض على مسؤولية العارض.

وفقًا للفقرة ١-٤-١، سيتم رفض العطاءات التي لا تفي بشكل جوهري بمتطلبات مستندات العطاء.

### DISCREPANCIES AND CLARIFICATIONS

If the Tenderer finds any ambiguities in the Documents or discrepancies between Documents or any other matters that are unclear, or if he is requiring any clarification of the Tender Documents, he shall inform in writing and by hand the Contracting Authority, in a minimum of ten (10) days prior to the date for submission of Tenders, who will respond with an addendum six (6) days prior to the date for submission of Tenders. Copies of the Contracting Authority's addendum will be forwarded to all Tenderers, including a description of the inquiry but without identifying its source. Except as provided for in this Clause no representation, explanation or statement made to a Tenderer prior to the Contracting Authority’s written acceptance of his Tender whether by the Engineer or any of his assistants or by any officer or member of the Contracting Authority’s staff or by anyone as to the meaning of any of the Documents forming the Contract or as to anything to be done or not to be done by the Tenderer shall bind the Contracting Authority or bind the judgment or powers of the Engineer in the exercise by him of his duties under the Contract.

١-٥-٣ التناقضات والتوضيحات

إذا وجد العارض أي غموض في المستندات أو تناقضات بين المستندات أو أي مسائل غير واضحة، أو إذا كان بحاجة إلى أي توضيح لمستندات الإلتزام، يجب عليه إعلام سلطة التعاقد كتابةً قبل عشرة (10) أيام على الأقل من الموعد المحدد لتقديم العروض، وسوف ترد سلطة التعاقد بإصدار ملحق قبل ستة (6) أيام من الموعد المحدد لتقديم العروض. سيتم إرسال نسخ من الملحق إلى جميع العارضين مع وصف الإستفسار دون الكشف عن مصدره. بإستثناء ما هو منصوص عليه في هذه المادة، فإن أي توضيح أو بيان أو تفسير يتم تقديمه لعارض قبل قبول العرض من سلطة التعاقد سواء من قبل المهندس أو مساعديه أو أي مسؤول أو عضو من موظفي سلطة التعاقد أو أي شخص آخر لن يلزم سلطة التعاقد أو يحكم على حكم المهندس في أداء واجباته بموجب العقد.

### AMENDMENT

At any time prior to the original or extended date for submission of Tenders, the Contracting Authority may, for any reason, whether at his own initiative or in response to a clarification requested by a Tenderer(s), modify the Tender Documents by the issue of an addendum.

Any addendum thus issued shall be part of the Tender Documents, and shall be communicated in writing by Email or by facsimile to all tenderers of the Tender Documents. Prospective Tenderers shall acknowledge receipt of each addendum in writing by hand or facsimile to the Contracting Authority. To give prospective Tenderers reasonable time in which to take an addendum into account in preparing their Tenders, the Contracting Authority can, if deemed necessary, extend the deadline for submission of Tenders.

Any addendum thus issued shall be part of the Tender Documents, and shall be communicated in writing by Email or by facsimile to all tenderers and shall be published on the central website of the public procurement committee.

١-٥-٤ التعديل

في أي وقت قبل التاريخ الأصلي أو التاريخ الممد لتقديم العروض، يمكن لسلطة التعاقد، ولأي سبب كان، سواء بمبادرته الخاصة أو إستجابةً لطلب توضيح من أحد العارضين، تعديل مستندات الإلتزام من خلال إصدار ملحق.
يعتبر أي ملحق يتم إصداره جزءًا من مستندات الإلتزام، ويتم إبلاغ الجميع لهذه المستندات كتابةً عبر البريد الإلكتروني أو الفاكس. يجب على العارضين تأكيد إستلام كل ملحق كتابيًا عن طريق اليد أو الفاكس لسلطة التعاقد. لإتاحة الوقت الكافي للعارضين لإدراج الملحق في تحضير عروضهم، يمكن لسلطة التعاقد، إذا لزم الأمر، تمديد الموعد النهائي لتقديم العروض.

يجب أن يُعتبر أي ملحق يتم إصداره جزءًا من مستندات الإلتزام، ويجب إبلاغه كتابةً عبر البريد الإلكتروني أو الفاكس لجميع العارضين لمستندات الإلتزام وينشر على الموقع الإلكتروني المركزي لهيئة الشراء العام.

### COMMUNICATIONS

All notes, requests for information and other communications from the Tenderer shall be sent by letter to the Contracting Authority, duly registered at the Contracting Authority headquarter, at the following address:

Port of Tripoli

Office d'Exploitation du Port de Tripoli

Tel: 00961 6 220180

Tripoli, El Mina

LEBANON

١-٥-٥ الاتصالات

يجب إرسال جميع الملاحظات، طلبات المعلومات والاتصالات الأخرى من العارضين عن طريق البريد إلى سلطة التعاقد، وتسجيلها بشكل رسمي في مقر سلطة التعاقد، على العنوان التالي:

مرفأ طرابلس
مصلحة إستثمار مرفا طرابلس
هاتف: 00961 6 220 180
طرابلس، الميناء
لبنان

### LANGUAGE OF TENDER

With reference to sub-Clause 1.2.2, stating that all Tender correspondences, supporting Documents and printed literature supplied by the Tenderer must be in Arabic and English, and for special cases where these mentioned Documents and literature are to be supplied in a language other than English, an accurate Arabic and English translation of the relevant passages must be attached to the initial supplied Documents, in which case, for purposes of interpretation of the Tender, the Arabic translation shall prevail.

١-٥-٦ لغة العرض

بالإشارة إلى المادة ٢-٢-١، يجب أن تكون جميع المراسلات المتعلقة بالعرض، الوثائق الداعمة، والنصوص المطبوعة المقدمة من العارض باللغتين العربية والإنجليزية وفقاً لأحكام المادة 4 من قانون الشراء العام. في الحالات الخاصة التي يجب فيها تقديم هذه الوثائق والنصوص بلغة أخرى غير الإنجليزية، يجب إرفاق ترجمة دقيقة للجزء المعني إلى اللغتين العربية والإنجليزية مع الوثائق الأساسية المقدمة، وفي هذه الحالة، سيكون التفسير بالترجمة إلى العربية هو المعتمد.

### INFORMATION

All information provided in the Tender Documents concerning the site, levels related to topographic and bathymetric surveys, environmental conditions (waves, wind and tidal data…), means of access, availability, of services, history, is given in good faith, without any responsibility on the part of the Contracting Authority.  The Tenderer shall verify and interpret all information given, at his own responsibility and expense, to enable himself to submit a complete Tender, and shall make therefore necessary and adequate investigation and research in order to obtain accurate information which will be him to prepare his offer.

١-٥-٧ المعلومات

أعطيت جميع المعلومات الواردة في مستندات الإلتزام حول الموقع، المستويات المتعلقة بالكيول الطوبوغرافية والبحرية، الظروف البيئية (الأمواج، الرياح، وبيانات المد والجزر...)، وسائل الوصول، توفر الخدمات، التاريخ، بحسن نية دون أي مسؤولية على عاتق سلطة التعاقد. يجب على العارض التحقق من وتفسير جميع المعلومات المقدمة على مسؤوليته الخاصة وعلى نفقته، ليتمكن من تقديم عرض كامل، وعليه إجراء التحقيقات والبحوث اللازمة للحصول على المعلومات الدقيقة التي ستمكنه من إعداد عرضه.

### SITE VISIT

The Tenderer shall visit and examine the Port, the Site and the surroundings and obtain for himself all details required for preparing and submitting his Tender.

The Contracting Authority shall not be responsible for any loss, damage or injury, whatsoever, caused to the Tenderer during the site visit.

The costs of visiting the Site shall be at the Tenderer’s own expense and his sole responsibility.

١-٦ زيارة الموقع

يجب على العارض زيارة وفحص المرفأ، الموقع، المحيط، والحصول على كافة التفاصيل المطلوبة لإعداد وتقديم عرضه.

لن يكون سلطة التعاقد مسؤولاً عن أي خسارة أو ضرر أو إصابة قد تحدث للعارض خلال زيارة الموقع.
إن تكاليف زيارة الموقع سوف تكون على نفقة العارض ومسؤوليته الخاصة.

### APPOINTMENT

Site visits and request for information must be made by prior appointment with the Contracting Authority.

The Tenderer or his official representative may be invited to attend a pre-tender meeting which will take place on the date stated in the letter of invitation and when needed.

The purpose of the pre-tender meeting will be to clarify issues and to answer questions on any matters that may be raised at that stage.

The Tenderer is requested to submit any questions in writing or by fax, to the Contracting Authority not later than one week before the pre-tender meeting.

Minutes of meeting, including the text of the questions raised and the responses given, will be transmitted without delay to all purchasers of the Tender Documents.

Any modification of the Tender Documents that may become necessary as a result of the pre-tender meeting shall be made by the Contracting Authority exclusively through the issue of an Addendum pursuant to Clause 1.5.4 and not through the minutes of the pre-tender meeting.

Non-attendance at the pre-tender meeting will not be a cause for disqualification of a Tenderer.

١-٧ التعيين

يجب أن تتم زيارات الموقع وطلبات المعلومات عن طريق تحديد موعد مسبق مع سلطة التعاقد.
قد يُدعى العارض أو ممثله الرسمي لحضور إجتماع سابق لتقديم العروض، والذي سيعقد في التاريخ المحدد في كتاب الدعوة وعند الحاجة.

الغرض من إجتماع سابق لتقديم العروض هو توضيح القضايا والإجابة على الأسئلة المتعلقة بأي مسألة قد تُطرح في تلك المرحلة.
يُطلب من العارض تقديم أي أسئلة كتابيًا أو عبر الفاكس إلى سلطة التعاقد في موعد لا يتجاوز أسبوعًا قبل الإجتماع السابق لتقديم العروض.

سيتم إرسال محضر الإجتماع، بما في ذلك نص الأسئلة التي تم طرحها والإجابات التي تم تقديمها، دون تأخير إلى جميع المشتركين في الإلتزام.

سيقوم رب العمل بأي تعديل على مستندات الإلتزام قد يصبح ضروريًا نتيجة للإجتماع السابق لتقديم العروض، حصريًا من خلال إصدار ملحق وفقًا للبند ٤-٥-١ وليس من خلال محضر الإجتماع السابق لتقديم العروض.

عدم حضور الإجتماع السابق لتقديم العروض لن يؤدي إلى إستبعاد العارض.

### COST OF TENDERING

The Tenderer shall bear all costs associated with the preparation and submission of his Tender and the Contracting Authority will in no way be responsible for those costs, or the conduct or outcome of the Tendering process, nor will then Contracting Authority bear any liability in respect thereof.

١-٨ تكاليف العرض

يتحمل العارض جميع التكاليف المرتبطة بإعداد وتقديم عرضه، ولن يكون سلطة التعاقد مسؤولًا عن هذه التكاليف أو عن سير عملية تقديم العروض أو نتيجتها، ولن يتحمل سلطة التعاقد أي مسؤولية في هذا الصدد.

### COMPLETION OF TENDER DOCUMENTS

The Tender must be for the whole of the Works, fully in accordance with the Tender Documents.

A partial Tender will not be considered.

The originals of all Tender Documents and all copies required to be submitted, shall be completed in indelible ink or type writing. All forms of Tender any other Document requiring completion shall be filled in.

١-٩ إكمال مستندات العرض

يجب أن يكون العرض شاملًا لجميع الأعمال، وفقًا الكامل لمستندات الإلتزام.

لن يتم النظر في العروض الجزئية. يجب إتمام النسخ الأصلية لجميع مستندات العرض وجميع النسخ المطلوبة بشكل واضح بإستخدام الحبر الدائم أو الكتابة الآلية.

يجب تعبئة جميع نماذج العرض وأي مستند آخر يتطلب إتمامه.

### SIGNING OF TENDER

The Tenderer shall prepare one original and one copy of the Documents comprising the Tender.

The Documents shall be clearly marked “ORIGINAL” and “COPY” as appropriate.

In the event of discrepancy between them, the original shall prevail.

The original and the copy of the Tender shall be returned duly completed by writing in indelible ink in the space provided for unit rates and totals.

In the case of copies, photocopies are acceptable. All originals and copies shall be signed by a person or persons duly authorized to sign on behalf of the Tenderer, as the case may be.

All pages of the Tender, where entries or amendments have been made, shall be signed by the person or persons signing the Tender and stamped.

The originals of all other Documents shall be stamped and initiated by the person or persons signing the Tender. In all cases, the form of Tender shall be signed by a person duly authorized to bind the Tenderer to the Contract. Proof of authorization shall be provided in the form of a power of attorney, which shall accompany the Tender.

١-١٠ توقيع العرض

يجب على العارض إعداد نسخة أصلية واحدة ونسخة طبق الأصل عنها من المستندات المكونة للعرض.

يجب أن تكون المستندات مميزة بوضوح بـ "أصلي" و"نسخة" بحسب الحاجة.

في حال وجود تعارض بينهما، يجب أن يسود الأصل.

يجب تقديم الأصل والنسخة من العرض مكتملين وموقعين بإستخدام الحبر الدائم في المساحة المخصصة للأسعار والوحدات الإجمالية.

في حالة النسخ، يُسمح بإستخدام النسخ المصورة. يجب أن تكون جميع المستندات الأصلية والنسخ موقعة من قبل شخص أو أشخاص مخولين بالتوقيع نيابة عن العارض، بحسب الحالة.

يجب توقيع جميع صفحات العرض، حيثما تم إدخال تعديلات أو تغييرات، من قبل الشخص أو الأشخاص الذين وقعوا على العطاء وختمها. يجب أن تكون النسخ الأصلية لجميع الوثائق الأخرى مختومة وموقعة من قبل الشخص أو الأشخاص الذين يوقعون العرض. في جميع الحالات، يجب أن يتم توقيع العرض من قبل شخص مفوض بشكل قانوني لربط العارض بالعقد. يجب تقديم إثبات التفويض في شكل توكيل قانوني، والذي يجب أن يرفق مع العرض.

### ALTERATIONS AND CORRECTION OF ERRORS

The completed Tender Documents shall contain no alterations, omissions or additions, except those to comply with instructions issued by the Contracting Authority, or as necessary to correct errors made by the Tenderer, in which case such corrections shall be initiated by the person or persons signing the Tender. Errors shall be corrected by striking out the original text and inserting the correction by hand using indelible ink. The correction of errors by overtyping with or without the use of correction fluid shall not be permitted and any Tender so corrected shall be rejected by the Contracting Authority.

١-١١ التعديلات وتصحيح الأخطاء

يجب ألا تحتوي وثائق العرض المكتملة على أي تعديلات أو حذف أو إضافات، بإستثناء تلك التي تمت وفقًا للتعليمات التي يصدرها سلطة التعاقد، أو كما هو ضروري لتصحيح الأخطاء التي إرتكبها العارض، وفي هذه الحالة يجب أن يتم تصحيح الأخطاء من قبل الشخص أو الأشخاص الذين يوقعون العرض. يجب تصحيح الأخطاء عن طريق شطب النص الأصلي وإدراج التصحيح يدويًا بإستخدام حبر غير قابل للمسح. لا يُسمح بتصحيح الأخطاء عن طريق الكتابة فوق النص سواء بإستخدام سائل تصحيح أو بدونه، وأي عرض تم تصحيحه بهذه الطريقة سيتم رفضه من قبل سلطة التعاقد.

### CURRENCY

All financial information and all Tender prices and rates shall be stated in United States Dollars (USD), which will be the currency for all transactions under the Contract. Payments for the Contractor are to be made in fresh US Dollars.

١-١٢ العملة

يجب أن تكون جميع المعلومات المالية وجميع أسعار العروض والأسعار مذكورة بالدولار الأمريكي (USD)، الذي سيكون العملة المعتمدة لجميع المعاملات بموجب العقد. سيتم دفع المدفوعات للمستفيدين بالدولار الأمريكي الفريش.

### DUTIES, TAXES, ETC.

The capital cost for Equipment and costs for operations and management shall include levied customs duties and all other duty payable in Lebanon and elsewhere. The Contracting Authority will not be held responsible for paying any dues, taxes, or charges of any kind.

١-١٣ الرسوم، الضرائب، إلخ.

يجب أن تشمل التكلفة للمعدات وتكاليف العمليات والإدارة، الرسوم الجمركية وجميع الرسوم الأخرى المستحقة في لبنان وأماكن أخرى.

لن يكون سلطة التعاقد مسؤولًا عن دفع أي مستحقات أو ضرائب أو رسوم من أي نوع.

### TENDER VALIDITY

The Tender shall remain valid and open for acceptance for a period of one hundred and eighty (180) days from the latest date fixed for receiving Tenders.

In exceptional circumstances, prior to expiry of the original Tender validity period, the Contracting Authority may request that the Tenderers extend the period of validity for a specified additional period.

The request and the responses thereto shall be made in writing or facsimile.

A Tenderer may refuse the request without forfeiting his Tender security.

A Tenderer agreeing to the request will be required to extend the validity of his Tender security for the period of the extension, and in compliance with the Tender Clauses in all respects.

Furthermore, the Tenderer can amend his offer and withdraw it before the deadline for submitting offers without confiscating the guarantee of his offer. The amendment or request to withdraw the offer is valid when the Authority receives it before the deadline for submitting offers.

١-١٤ صلاحية العرض

يجب أن يظل العرض صالحًا ومفتوحًا للقبول لمدة مئة وثمانين (180) يومًا من آخر تاريخ محدد لتقديم العروض.
في حالات إستثنائية، وقبل انتهاء فترة صلاحية العرض الأصلية، قد تطلب سلطة التعاقد من مقدمي العروض تمديد فترة الصلاحية لمدة إضافية محددة. يجب أن يتم تقديم الطلب والرد عليه كتابة أو عبر الفاكس. يمكن للعارض رفض الطلب دون مصادرة ضمان العرض. سيُطلب من العارضين الذي يوافقون على الطلب، تمديد صلاحية ضمان العرض لفترة التمديد، ووفقًا لبنود العرض من جميع النواحي. علاوة على ذلك، يمكن للعارض تعديل عرضه أو سحبه قبل الموعد النهائي لتقديم العروض دون مصادرة ضمان عرضه. يكون التعديل أو طلب سحب العرض ساريًا عندما تتسلم سلطة التعاقد الطلب قبل الموعد النهائي لتقديم العروض.

### TENDER SECURITY

The Tenderer shall furnish, as part of his Tender, a Tender security in the amount of eight hundred thousand (800,000) US Dollars.

In case of a Joint Venture, the Tender Security shall be in the name of the Joint Venture.

The Tender Security shall be in the form of a guarantee from a first-class Lebanese Bank, or a foreign Bank with a local branch / affiliate in Lebanon, or through a cash deposit of the whole amount (fresh) at Office d’exploitation du Port de Tripoli. The format of the Bank guarantee shall be in accordance with the sample form of Tender Security included in the Tender Documents.

The Tender Security shall be valid for the period stated in the bidding. Any Tender not accompanied by an acceptable Tender Security shall be rejected by the Contracting Authority and shall be considered as non-responsive. The Tender Securities of unsuccessful Tenderers will be returned Within a maximum period of the contract's entry into force.

The Tender Security of the successful Tenderer will be returned when the Tenderer has signed the Agreement and furnished the required Performance Security.

The Tender Security may be forfeited if:

- The Tenderer withdraws his Tender after the deadline for submitting offers, or

- The Tenderer, having been notified of the acceptance of his Tender by the Contracting Authority during the period of Tender validity:

- Fails or refuses to execute the Form of Agreement; or

- Fails or refuses to furnish or increase the Performance Security; or

- Refuses to accept the correction of errors in his Tender.

١-١٥ ضمان العرض

يجب على العارض تقديم ضمان عرض ضمن عرضه بمبلغ ثمانمائة ألف (٨٠٠,٠٠٠) دولار أمريكي.
في حالة وجود شراكة، يجب أن يكون ضمان العرض بإسم الشراكة. يجب أن يكون ضمان العرض في شكل ضمان صادر عن بنك لبناني من الدرجة الأولى، أو عن بنك أجنبي له فرع / تابع محلي في لبنان، أو من خلال إيداع نقدي للمبلغ الكامل (فريش) لدى مصلحة إستثمار مرفأ طرابلس. يجب أن يكون نموذج الضمان البنكي وفقًا لنموذج ضمان العرض المدرج في وثائق الإلتزام. يجب أن يكون ضمان العرض ساريًا خلال تاريخ جلسة فض العروض. سيتم رفض أي عرض غير مصحوب بضمان عرض مقبول من قبل سلطة التعاقد وسيتم إعتباره غير مستجيب. سيتم إعادة ضمانات العروض للعارضين غير الناجحين في مهلة أقصاها بدء نفاذ العقد.

سيتم إعادة ضمان العرض لمقدّم للعارض الناجح عندما يوقع العارض العقد ويقدم الضمان المطلوب لحسن التنفيذ.

قد يتم مصادرة ضمان العرض في حال:

١) سحب العارض لعرضه بعد الموعد النهائي لتقديم العروض، أو

٢) إذا العارض، بعد أن تم إخباره بقبول عرضه من قبل سلطة التعاقد خلال فترة صلاحية العرض:

أ- فشل أو رفض تنفيذ نموذج الاتفاقية؛ أو

ب - فشل أو رفض تقديم أو زيادة ضمان حسن التنفيذ؛ أو

ج - فض قبول تصحيح الأخطاء في العرض.

### TENDER SUBMISSION

### SEALING AND MARKING OF TENDERS

All Documents shall be signed, dated, completed and returned as described in Tender Conditions and Procedures.

The Tenderer shall seal the Tender package and address it to:

Port of Tripoli

Office d'Exploitation du Port de Tripoli

Tel: 961 6 220180

Tripoli, El Mina

LEBANON

And shall mark the Tender package with ONLY the following identification:

“DBOT TENDER FOR INSTALLING GRAIN SILO AT THE PORT OF TRIPOLI: DO NOT OPEN WITHOUT AUTHORITY"

The package shall contain two separate and sealed inner envelopes marked as envelope N1 and envelope N2.

Each envelope shall contain one set of original Documents and one set of copies clearly marked “ORIGINAL” and “COPY”.

The inner Envelope N1 shall bear the following clear identification:

“DBOT TENDER FOR INSTALLING GRAIN SILOS AT THE PORT OF TRIPOLI”

Envelope N1 “Administrative and Technical Offer”

Name of Tenderer / Address of the Tenderer

This envelope should contain one original and one copy of each of the following:

- Statement of Tender in accordance with the form included in the Tender Documents, sealed and signed by the Tenderer.

- Tender Security.

- Certified operational and financial Documentation concerning Company structure and profile of the Tenderer.

- Details of the joint-venture Agreement or other commercial arrangements (if any), sealed and signed by the Tenderer.

- Power of attorney.

- Draft DBOT Agreement stamped with the official seal of the Tenderer and initialed.

- Contracting Authority's Requirements signed, stamped with the official seal of the Tenderer and initial

- All certificates listed in Section 1.4: Qualification of the Tenderer.

- All Tender documents (including addendums, drawings…) signed and stamped by the Tenderer.

- A market study: will be used to identify potential clients, determine their demand, determine corresponding supply and tariffs in Lebanon area, adapt the supply of these new installations and determine the share of the market expected to be captured in function of services and tariffs proposed by these new installations.

The final objective of this market study will be to determine grain import/export quantity and tariff forecast which will be then used in the business plan in order to determine the opportunity of these installations.

Thus, for these new installations, a market study shall be submitted including:

- demand: potential imported quantity, storage duration and type of services.

- supply: potential exported grain quantity, storage duration and type of services.

- present similar installations in Lebanon area and tariffs.

- Development program: This shall set out the installations that the Contractor undertakes to provide to the DBOT Agreement in respect with the Contracting Authority’s Requirements. Tenderers should keep in mind that the Evaluation Committee will pay particular attention to this part of Tender and any precisions, information about installations to be built, Equipment to be supplied, water and electrical power supplies, bins already in procession will have a major positive impact to the Tender value.

- Time schedule for the initial development work. This should set out the phasing of the principal activities that will take place between the Commencement Date and the Commencement of Operations, showing especially the phasing for the execution of the civil and MEP infrastructure, till the installation of the silo and launching the operation phase.

- Any addendum (if any) issued by the Contracting Authority, signed and stamped by the Tenderer.

The inner Envelope N2 shall bear the following clear identification:

“TENDERING DOCUMENTS FOR INSTALLING GRAIN SILO AT THE PORT OF TRIPOLI”

Envelope N2 “Financial Offer” according to the template provided by the Contracting Authority.

Name of Tenderer / Address of the Tenderer.

This envelope should contain one original and one copy of each of the following:

- Forms of Tender, completed, signed, stamped with the official seal of the Tenderer and initialed.

This shall indicate the Annual Rental Fee.

- Business plan

A business plan for the operation of the DBOT Agreement shall be submitted including:

- an import/export forecast.

- investment costs.

- Operating costs (maintenance, staff, electricity etc. ...).

- provisional revenues.

- capital investment pan (equity and debt).

- cash flow.

- financial indicators (Project IRR, ROE, Payback).

- Example of performance security, DBOT Agreement security, operating regulations and list of tariffs initialed by the Tenderer.

١-١٦ تقديم العروض

١-١٦-١ ختم ووسم العروض

يجب أن تكون جميع الوثائق موقعة، مؤرخة، مكتملة ومعادة كما هو موضح في شروط وإجراءات الإلتزام.

يجب على العارض ختم ملف العرض وتوجيهه إلى:

مرفأ طرابلس

مصلحة إستثمار مرفأ طرابلس

هاتف: 00961 6 220 180

طرابلس، الميناء

لبنان

ويجب أن يتم عنونة ملف العرض وفق التالي فقط:

“DBOT TENDER FOR INSTALLING GRAIN SILO AT THE PORT OF TRIPOLI: DO NOT OPEN WITHOUT AUTHORITY”

يجب أن يحتوي الملف على ظرفين داخليين منفصلين ومختومين معنونين بالظرف N1 والظرف N2.
يجب أن يحتوي كل ظرف على مجموعة أصلية واحدة من الوثائق ومجموعة من النسخ معنونة بوضوح بـ "أصلي" و"نسخة".

يجب أن يحتوي الظرف الداخلي N1 على التعريف الواضح التالي:

“DBOT TENDER FOR INSTALLING GRAIN SILOS AT THE PORT OF TRIPOLI”

ظرف N1 " العرض الإداري والتقني"

اسم العارض / عنوان العارض

يجب أن يحتوي هذا الظرف على نسخة أصلية واحدة ونسخة مصورة واحدة من كل مما يلي:

أ - تعهدّ العارض وفقًا للنموذج المرفق في وثائق الإلتزام، مختوم وموقع من قبل العارض.

ب - ضمان العرض.

ج - وثائق تشغيلية ومالية مصدقة تتعلق بهيكلية الشركة والسيرة الذاتية للعارض.

د - تفاصيل إتفاقية الشراكة أو أي ترتيبات تجارية أخرى (إن وجدت)، مختومة وموقعة من قبل العارض.

ه - توكيل قانوني.

و - مسودة إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية مختومة بختم العارض الرسمي وموقعة.

ز - متطلبات سلطة التعاقد موقعة، مختومة بختم العارض الرسمي وموقعة.

ح - جميع الشهادات المدرجة في المادة ١-٤: تأهل العارض

ط - جميع وثائق الإلتزام (بما في ذلك الإضافات، الرسومات...) موقعة ومختومة من قبل العارض.

ي - دراسة السوق: ستستخدم لتحديد العملاء المحتملين، تحديد الطلب، تحديد العرض والتعريفات في لبنان، تكييف العرض لهذه المنشآت الجديدة وتحديد حصة السوق المتوقع بناءً على الخدمات والتعريفات التي تقترحها هذه المنشآت الجديدة.

الهدف النهائي من دراسة السوق هذه هو تحديد كمية الواردات / الصادرات من الحبوب وتوقعات التعرفة التي ستستخدم بعد ذلك في خطة العمل لتحديد جدوى هذه المنشآت.

لذا، يجب تقديم دراسة السوق لهذه المنشآت الجديدة والتي ستشمل:

- الطلب: الكمية المحتملة المستوردة، مدة التخزين ونوع الخدمات.

- العرض: الكمية المحتملة المصدرة من الحبوب، مدة التخزين ونوع الخدمات.

- المنشآت المماثلة الحالية في لبنان والتعريفات.

ك - برنامج التطوير: يجب أن يتضمن المنشآت التي يتعهد المتعاقد بتوفيرها في إطار متطلبات سلطة التعاقد. يجب أن يضع العارضون في إعتبارهم أن لجنة التقييم ستولي إهتمامًا خاصًا لهذا الجزء من العرض وأية تفاصيل أو معلومات حول المنشآت التي سيتم بناؤها، المعدات التي سيتم توريدها، إمدادات المياه والطاقة الكهربائية، الصوامع التي تم الحصول عليها سيكون لها تأثير إيجابي كبير على قيمة العرض.

ل - الجدول الزمني للأعمال التطويرية الأولية. يجب أن يحددّ هذا الجدول مراحل الأنشطة الرئيسية التي ستحدث بين تاريخ المباشرة وبداية العمليات، موضحًا خاصة مراحل تنفيذ البنية التحتية المدنية والكهربائية والميكانيكية، وصولاً إلى تركيب الإهراءات وبدء مرحلة التشغيل.

م - أي إضافة (إن وجدت) صادرة عن سلطة التعاقد، موقعة ومختومة من قبل العارض.

يجب أن يحتوي الظرف الداخلي N2 على التعريف الواضح التالي:

“TENDERING DOCUMENTS FOR INSTALLING GRAIN SILO AT THE PORT OF TRIPOLI”

وفقاً للنموذج الموضوع من قبل سلطة التعاقد."العرض المالي" N2ظرف

اسم العارض/ عنوان العارض

يجب أن يحتوي هذا الظرف على نسخة أصلية واحدة ونسخة مصورة واحدة من كل مما يلي:

أ - نماذج العرض، مكتملة، موقعة، مختومة بختم العارض الرسمي وموقعة.

يجب أن تشير هذه النماذج إلى رسم الإيجار السنوي.

ب - خطة العمل

يجب تقديم خطة عمل لتشغيل العقد والتي ستشمل:

- توقعات الاستيراد / التصدير.

- تكاليف الاستثمار.

- تكاليف التشغيل (الصيانة، الموظفين، الكهرباء، إلخ...).

- الإيرادات التقديرية.

- خطة الاستثمار (الأسهم والديون).

- التدفق النقدي.

- المؤشرات المالية (العائد الداخلي على المشروع، العائد على حقوق الملكية، فترة الاسترداد).

ج - نموذج ضمان حسن التنفيذ، ضمان العقد، القوانين التشغيلية وقائمة التعريفات موقعة ومختومة من قبل العارض.

### VARIANT SOLUTIONS

Alternatives being not required for the present Tender; the Contracting Authority is not obliged to evaluate any alternative.

١-١٦-٢ الحلول البديلة

نظرًا لأن البدائل غير مطلوبة في العرض الحالي، فإن سلطة التعاقد ليست ملزمًة بتقييم أيّ بديل.

### TIME LIMIT FOR SUBMISSION FOR TENDERS

The Tender must be received by the Contracting Authority at the address stated above not later than 12:00 hours on the date stated in the form of invitation to Tender. The announcement deadline should allow sufficient time for tenderers to prepare their tenders, given the size, complexity, and requirements of the project.

Any Tender received after the specified time limit will be returned unopened to the Tenderers.

The Contracting Authority may, at his discretion, extend the time limit for the submission of Tender by issuing an amendment, in accordance with the Tender Clauses, in which case all rights and obligations of the Contracting Authority and the Tenderer previously subject to the original time limit deadline shall thereafter be subject to the new time limit as extended.

Any Tender received by the Contracting Authority after the time limit for submission of Tenderers will be returned unopened to the Tenderer.

١-١٦-٣ الحد الزمني لتقديم العروض

يجب أن يتم استلام العرض من قبل سلطة التعاقد في العنوان المذكور أعلاه في موعد أقصاه الساعة ١٢:٠٠ من اليوم المحدد في نموذج دعوة تقديم العروض. يجب أن تتيح مهلة الإعلان الوقت الكافي للعارضين لتحضير عروضهم بالنظر الى حجم المشروع وتعقيداته ومتطلباته.

أي عرض يُستلم بعد الموعد المحدد سيتم إعادته دون فتحه إلى العارض.

يجوز لسلطة التعاقد، حسب تقديرها، تمديد الحد الزمني لتقديم العروض عن طريق إصدار تعديل، وفقًا لمواد الإلتزام، وفي هذه الحالة، فإن جميع الحقوق والالتزامات التي كانت تخضع للموعد النهائي الأصلي لسلطة التعاقد والعارض، ستكون بعد ذلك خاضعة للحد الزمني الجديد كما تم تمديده.

أي عرض يُستلم من قبل سلطة التعاقد بعد الموعد المحدد لتقديم العروض سيتم إعادته دون فتحه إلى العارض.

### MODIFICATION AND WITHDRAWAL OF TENDERS

The Tenders’ modification or notice of withdrawal shall be written, sealed, marked and delivered prior to the prescribed time limit for submission of Tenders, and in accordance with provisions for the submission of Tenders, with the inner envelope marked "modification" or "withdrawal" as appropriate.

Withdrawal of a Tender during the interval between the time limit for submission of Tenders and the expiration of the period of validity may result in the forfeiture of the Tender security, pursuant to Tender Clauses.

No Tender may be modified by the Tenderer after the time limit for submission.

١-١٦-٤ تعديل وسحب العروض

يجب أن يكون تعديل العرض أو إشعار السحب مكتوبًا، مختومًا، موسومًا ومُسلّمًا قبل الموعد المحدد لتقديم العروض، ووفقًا لأحكام تقديم العروض، مع وضع الظرف الداخلي الموسوم بـ "تعديل" أو "سحب" حسب الحالة.

قد يؤدي سحب العرض خلال الفترة بين الموعد المحدد لتقديم العروض وإنتهاء فترة صلاحية العرض إلى مصادرة ضمان العرض، وفقًا لمواد الإلتزام.

لا يجوز تعديل أي عرض من قبل العارض بعد الموعد المحدد لتقديم العروض.

### TENDER OPENING AND EVALUATION

### UNSUCCESSFUL TENDERERS

The Tender securities of unsuccessful Tenderers will be returned as promptly as possible, in the twenty-eight (28) days after the expiration of the period of Tender validity.

١-١٧ فتح وتقييم العروض

١-١٧-١ العارضون غير الناجحون

سيتم إعادة ضمانات العروض للعارضين غير الناجحين بأسرع ما يمكن، وذلك في غضون ثمانية وعشرين (٢٨) يومًا بعد انتهاء فترة صلاحية العرض.

### TENDER OPENING

The Tender committee will open Envelope N1 in the presence of the Tenderer’s representatives who choose to attend, at the time and date stated in the Invitation of Tenders at the address given for return of Tenders.

The Tenderers representatives who are present shall sign a register evidencing their attendance.

The Tender opening committee will take necessary and needed time to study and evaluate all administrative and technical offers, and retain only satisfactory ones.

The committee will fix a date to proceed in the opening of Envelope N2 comprising the priced financial offers of successful Tenderers.

The Tender opening and evaluation procedures may include many sessions as it may be judged by the Tender opening committee and as stated in this Tender.

١-١٧-٢ فتح العروض

ستقوم لجنة التقييم بفتح الظرف N1 بحضور ممثلي مقدمي العارضين الذين يختارون الحضور، في الوقت والتاريخ المحددين في دعوة العروض، في العنوان المحدد لإعادة العروض.

يجب على ممثلي العارضين الحاضرين التوقيع على سجل يثبت حضورهم.

ستأخذ لجنة التقييم الوقت اللازم لدراسة وتقييم جميع العروض الإدارية والفنية، وستحتفظ فقط بالعروض المقبولة.

ستحدد اللجنة موعدًا لفتح الظرف N2 الذي يحتوي على العروض المالية المقدرة للعارضين المقبولين.

قد تتضمن إجراءات فتح العروض والتقييم العديد من الجلسات حسب ما تراه لجنة فتح العروض، كما هو منصوص عليه في هذا الإلتزام.

### TENDERS EVALUATION

The Contracting Authority will appoint an Evaluation Committee, which will be in charge for the Tender’s evaluation. During the Tender opening session, the Evaluation Committee will open each Tender and list all Documents placed in the envelope.

If the information or documents provided in the offer are incomplete or incorrect, or in the absence of a specific document, the Contracting Authority may request in writing that the concerned bidder provide clarifications regarding their offer, or request the submission or completion of the relevant information or documents within a specified time period. All correspondence must be in writing, respecting the principles of transparency and equality in treatment among bidders in requests for written clarification or completion, while adhering to the provisions of paragraph 3 of Article 21 of the Public Procurement Law.

The Contracting Authority may, at any stage of the tendering process, request in writing from the bidder clarifications regarding the information related to their qualifications or their offers, in order to assist in verifying the qualifications or examining and evaluating the submitted offers.

The Contracting Authority shall correct any purely arithmetic errors discovered during its examination of the submitted offers in accordance with the provisions of the terms of reference, and shall notify the concerned bidder of the corrections immediately.

No request may be made or allowed for any substantial change in the information related to qualifications or the submitted offer, including changes aimed at qualifying a bidder who is not qualified or making a non-compliant offer compliant.

After the Tender opening session opened to public, the Evaluation Committee will take necessary time to lead and control the Tender evaluation session and ensure that all Tenders are fairly evaluated.

The Evaluation Committee will determine whether each Tender is of acceptable quality, complete and substantially responsive to the Tender Documents without material deviations, incoherencies, absurdities, objections, conditions or reservations.

Evaluation method will be an attentive analysis of each Document of the Tender.

If a Tender is not substantially responsive, it will be rejected by the Evaluation Committee and it may not subsequently be made responsive by the Tenderer by removal of the non-conformity.

The Evaluation Committee at his discretion, may ask any Tenderer for a clarification of his Tender.

This request and the response shall be in writing and shall not result in a change in the price or substance of the Tender. Comparison of these criteria will lead to establish Tenders to be acceptable. From the time of Tender opening to the time of Contract award, if any Tenderer wishes to contact the Contracting Authority on any matter related to the Tender, he should do so in writing.

Any effort by the Tenderer to influence the Contracting Authority in the Contracting Authority’s Tender evaluation, Tender comparison, or Contract award decisions may result in the rejection of the Tenderer’s tender. Information relating to the examination, clarification, evaluation and comparison of Tenders and recommendations for the award of a contract shall not be disclosed to Tenderers or any other persons not officially concerned with such process even after the award to the successful Tenderer has been announced.

Any effort by a Tenderer to influence the Contracting Authority’s processing of Tenders or award decisions may result in the rejection of the Tenderer’s Tender.

The Contracting Authority will open envelope N2 for the bids that have been identified as acceptable, and for which the return envelope N1 (administrative and technical offer) substantially meets the requirements.

After opening Envelope N2, the Contracting Authority will determine for each Tender the Evaluated Tender Price and make any correction for errors pursuant to related Clauses.

If the Tender of the successful Tenderer is seriously financially unbalanced in relation to the Contracting Authority’s estimate of work to be performed under the Contract, the Contracting Authority may require the Tenderer to produce a detailed price analysis for any or all items of the Bill of Quantities, to demonstrate the internal consistency of those prices with the construction methods and schedule proposed.

In all cases and whatever situation is, the Contracting Authority has the right to reject any submitted offer, for the reason(s) that he might find suitable.

Tenderers will have no right for any objection or claim due to the mentioned rejection.

١-١٧-٣ تقييم العروض

تقوم سلطة التعاقد بتعيين لجنة تقييم، التي ستكون مسؤولة عن تقييم العروض. خلال جلسة فتح العروض، ستقوم لجنة التقييم بفتح كل عرض وإدراج جميع الوثائق الموجودة في الظرف.

في حال كانت الـمعلومات أو الـمستندات الـمقدَّمة في العرض ناقصة أو خاطئة أو في حال غياب وثيقة معيَّنة، يَجوز لسلطة التعاقد الطلب خطّياً من العارض الـمعني توضيحات حول عرضه، أو طلب تقديم أو إستكمال الـمعلومات أو الوثائق ذات الصلة خلال فترة زمنية محدَّدة، شرط أن تكون كافة الـمراسلات خطية وإحترام مبادئ الشفافية والـمساواة في الـمعاملة بين العارضين في طلبات التوضيح أو الإستكمال الخطية، ومع مراعاة أحكام الفقرة 3 من البند الثاني من الـمادة 21 من قانون الشراء العام.

يمكن لسلطة التعاقد، في أيِّ مرحلة من مراحل إجراءات التلزيم، أن تطلب خطياً من العارض إيضاحات بشأن الـمعلومات الـمتعلِّقة بمؤهلاته أو بشأن عروضه، لـمساعدتها في التأكُّد من الـمؤهّلات أو فحص العروض الـمقدَّمة وتقييمها.
 تُصحِّح سلطة التعاقد أيَّ أخطاء حسابية محضة تكتشفها أثناء فحصها العروض الـمقدَّمة وفقاً لأحكام دفتر الشروط، وتبلِّغ التصحيحات إلى العارض الـمعني بشكل فوري.

لا يمكن طلب إجراء أو السماح بإجراء أيِّ تغيير جوهري في الـمعلومات الـمتعلِّقة بالـمؤهلات أو بالعرض الـمقدَّم، بما في ذلك التغييرات الرامية إلى جعل مَن ليس مؤهَّلاً من العارضين مؤهَّلاً أو جعل عرض غير مستوفٍ للـمتطلبات مستوفياً لها.

بعد جلسة فتح العروض التي ستكون مفتوحة للجمهور، ستأخذ لجنة التقييم الوقت اللازم لإجراء وتوجيه جلسة تقييم العروض والتأكد من أن جميع العروض قد تم تقييمها بشكل عادل.

ستحدد لجنة التقييم ما إذا كان كل عرض ذا جودة مقبولة، كامل ومطابق إلى حد كبير لوثائق الإلتزام دون أغلاط مادية، تناقضات، مغالطات، إعتراضات، شروط أو تحفظات.

طريقة التقييم ستكون تحليلًا دقيقًا لكل وثيقة من وثائق العرض تبدأ:

أولاً باستيفاء شروط المشاركة

ثانياً تحديد مدى الإستجابة للمؤهلات المطلوبة (الفنية والمالية)

ثالثاً تحديد مدى إستجابة العرض الفني للمتطلبات.

يتمّ إبلاغ العارضين بنتيجة التقييم الفني والإداري سواء كان عرضهم مستجيباً أم لا، مع تبيان الأسباب في حال عدم الإستجابة.

تضع لجنة التقييم المعايير التي ستتم على أساسها التقييم على أن تكون منسجمة مع أساس الإرساء الذي سيكون على أساس السعر الأعلى عبر إدراج جداول توضح المؤهلات الفنية والمالية وتنسيب المعايير إلى كل من المؤهلات المطلوبة وبين ما هو مطلوب فنياً مع المتطلبات الدنيا.

إذا كان العرض غير مطابق إلى حد كبير، فسيتم رفضه من قبل لجنة التقييم ولا يمكن تعديله لاحقًا ليصبح مطابقًا من قبل العارض عن طريق إزالة عدم التوافق.

قد تطلب لجنة التقييم، بحسب تقديرها، من أي عارض توضيحًا لعرضه.

يجب أن تكون هذه الطلبات والردود كتابية، ولا ينبغي أن تؤدي إلى تغيير في السعر أو جوهر العرض.

سوف يؤدي مقارنة هذه المعايير إلى تحديد العروض المقبولة. من وقت فتح العروض وحتى وقت منح العقد، إذا رغب أي عارض في التواصل مع سلطة التعاقد بشأن أي مسألة تتعلق بالعرض، يجب أن يتم ذلك كتابيًا.

أي محاولة من عارض للتأثير على تقييم سلطة التعاقد للعروض، مقارنة العروض، أو قرارات منح العقد قد تؤدي إلى رفض العرض.

يجب عدم إفشاء أي معلومات تتعلق بفحص العروض، توضيح العروض، تقييم العروض، مقارنة العروض، والتوصيات لمنح العقد للعارضين أو أي شخص آخر غير المعني رسميًا بهذا العملية حتى بعد إعلان منح العقد للعارض الناجح.

أي محاولة من عارض للتأثير على معالجة العروض أو قرارات منح العقد قد تؤدي إلى رفض العرض المقدم من قبل العارض.

ستقوم سلطة التعاقد بفتح الظرف N2 للعروض التي تم تحديدها على أنها مقبولة والتي يكون الظرف العائد N1  (العرض الإداري والفني) مستجيب جوهرياً للمتطلبات.

بعد فتح الظرف  N2، ستحدد سلطة التعاقد السعر التقييمي للعرض لكل عرض وتقوم بإجراء أي تصحيح للأخطاء وفقًا للبنود ذات الصلة.

إذا كان العرض الناجح غير متوازن ماليًا بشكل كبير بالنسبة لتقدير سلطة التعاقد للأعمال التي سيتم تنفيذها بموجب العقد، يجوز لسلطة التعاقد أن تطلب من العارض تقديم تحليل سعر مفصل لأي أو كل بنود جدول الكميات، لإثبات التناسق الداخلي لتلك الأسعار مع طرق البناء والجدول الزمني المقترح. في جميع الحالات وأيًا كانت الظروف، يحق لسلطة التعاقد رفض أي عرض مُقدّم، للأسباب التي قد تراها مناسبة. لن يكون للعارضين أي حق في الإعتراض أو المطالبة بسبب الرفض المذكور.

### CORRECTION OF ERRORS

Tenders determined to be substantially responsive will be checked by the Contracting Authority for arithmetic errors.

Errors will be corrected by the Contracting Authority as follows:

- Where there is a discrepancy between the amounts in figures and in words, the amount in words will govern; and

- Where there is a discrepancy between the unit rate and the line item resulting from multiplying the unit rate by the quantity, the unit rate as quoted will govern; and

- Where there is a discount applied to the total tender price including the contingency, the total tender price shall be retained and the discount applied to the unit prices of the Bill of Quantities items, excluding the contingency.

- If the Tenderer does not accept the corrected amount of tender, his tender will be rejected, and the Tender Security will be forfeited in accordance the related Clause.

١-١٧-٤ تصحيح الأخطاء

سيتم فحص العروض التي تم تحديد أنها متوافقة بشكل كبير من قبل سلطة التعاقد للتحقق من الأخطاء الحسابية.

سيتم تصحيح الأخطاء من قبل سلطة التعاقد على النحو التالي:

أ) عندما يكون هناك تباين بين المبالغ بالأرقام وبالكلمات، سيكون المبلغ المكتوب بالكلمات هو المعتمد؛
ب) عندما يكون هناك تباين بين سعر الوحدة والبند الناتج من ضرب سعر الوحدة في الكمية، سيكون سعر الوحدة كما تم اقتباسه هو المعتمد؛

ج) عندما يتم تطبيق خصم على إجمالي سعر العطاء بما في ذلك الإحتياطي، سيتم الاحتفاظ بالسعر الإجمالي للعرض وتطبيق الخصم على الأسعار الفردية لبنود جدول الكميات، بإستثناء الاحتياطي؛

د) إذا لم يقبل العارض المبلغ المصحح للعرض، فسيتم رفض عرضه، وسيتم مصادرة ضمان العرض وفقًا للبند المتعلق.

### AWARD AND SIGNING OF CONTRACT

### AWARD

## The Contracting Authority will award the contract to the bidder who submitted the administrative and technical proposal that substantially meets the requirements in the specifications document and offered the highest price.

Contractors have the right to submit any claim or objection, regardless of its nature, regarding the decision to award the contract, in accordance with the Public Procurement Law in Lebanon No. 244 dated 19/7/2021, Chapter Seven. The Contracting Authority has the right to evaluate any tender according to the evaluation methodology detailed in the terms of reference. The Contracting Authority reserves the right to accept or reject any Tender and to cancel the Tendering process and reject all Tenders at any time prior to award of Contract or withdrawal without thereby incurring any liability to the affected Tenderer or Tenderers or any obligation to inform them on the grounds of the Contracting Authority’s action.

After opening all the bids, in case two or more acceptable Tenderers have equal bid amount, the successful Tenderer will be chosen by a random draw.

The Contracting Authority may reject any offer if it determines that the price, combined with all other elements of the submitted offer, is unusually high compared to the subject of the purchase and its estimated value, provided that the Contracting Authority has requested from the concerned bidder in writing the details of the submitted offer in a manner that allows it to analyze the detailed information. Among the details that the Contracting Authority may request, for example, are:

A - Information and samples or similar items that prove the quality of the subject of the purchase presented in the offer;

B - Relevant manufacturing methods;

C - Selected technical solutions and/or any exceptionally favorable conditions available to the bidder for the execution of the contract.

The evaluation report includes the purchasing entity’s decision to reject an offer in accordance with the provisions of this article, the reasons for that decision, and all clarifications made with the bidders. The concerned bidder shall be promptly notified of the Contracting Authority's decision and its reasons.

١-١٨ منح العقد وتوقيعه

١-١٨-١ منح العقد

ستقوم سلطة التعاقد بمنح العقد للعارض الذي قدّم العرض الإداري والفني المستجيب جوهرياً للمتطلبات في دفتر الشروط والذي قدّم السعر الأعلى. للعارضين حق تقديم أي مطالبة أو إعتراض مهما كانت طبيعتها بشأن قرار منح العقد وفقاً لقانون الشراء العام في لبنان رقم 244 بتاريخ 19/7/2021، الفصل السابع. يحق لسلطة التعاقد تقييم أي عرض وفق منهجية التقييم المفصّلة في دفتر الشروط. تحتفظ سلطة التعاقد بالحق في قبول أو رفض أي عرض وإلغاء الإلتزام ورفض جميع العروض في أي وقت قبل منح العقد أو سحبه دون أن تتحمل أي مسؤولية تجاه العارض أو العارضين المتأثرين أو أي إلتزام بإعلامهم بأسباب تصرف سلطة التعاقد. بعد فتح جميع العروض، في حال كان للعارضين المقبولين نفس المبلغ المعروض، سيتم إختيار العارض الناجح عن طريق السحب العشوائي.

يجوز لسلطة التعاقد أن ترفض أيَّ عرض إذا قرَّرَت أنَّ السعر، مُقترناً بسائر العناصر الـمكوِّنة لذلك العرض الـمقدَّم، مُرتفع ارتفاعاً غير عاديّ قياساً إلى موضوع الشراء وقيمته التقديرية، وذلك شرط أن تكون سلطة التعاقد قد طلبت من العارض الـمعني خطياً تفاصيلَ العرض الـمقدَّم بشكل يسمح لها بتحليل الـمعلومات التفصيلية.

من التفاصيل التي يمكن أن تطلبها سلطة التعاقد، على سبيل الـمثال لا الحصر:

أ - معلومات وعيِّنات أو ما يُشابهها، تُثبت جودة موضوع الشراء الـمقدَّم في العرض؛

ب - طُرق التصنيع ذات الصلة؛

ج - الحلول التقنية الـمختارة و/أو أيّ شروط مُؤاتية بشكل إستثنائي مُتاحة للعارِض لتنفيذ العقد.

يُدرَجُ في تقرير التقييم قرارُ الجهة الشارية بِرَفض عرض ما وفقاً لأحكام هذه الـمادة، وأسبابُ ذلك القرار وكلُّ الإيضاحات التي جرت مع العارضين. ويُبلَّغ العارض الـمعني، على الفور، بقرار سلطة التعاقد وأسبابه.

### NOTIFICATION OF AWARD

The notification of award shall be done in accordance with the Public Procurement Law in Lebanon, N244 dated July 29, 2021, Article 24.

١-١٨-٢ إشعار منح العقد

سيتم إشعار منح العقد وفقًا للمادة ٢٤ من قانون الشراء العام في لبنان، رقم ٢٤٤ تاريخ ٢٩ يوليو ٢٠٢١.

### SIGNING THE CONTRACT AGREEMENT

The Contracting Authority and the successful Tenderer shall sign the Contract Agreement as per the Public Procurement Law in Lebanon, N244 dated July 29, 2021, Article 24, especially clauses (4) and (5).

١-١٨-٣ توقيع العقد

ستقوم سلطة التعاقد والعارض الناجح بتوقيع العقد وفقًا لقانون الشراء العام في لبنان، رقم ٢٤٤ تاريخ ٢٩ يوليو ٢٠٢١، المادة ٢٤، وخاصة الفقرتين (٤) و(٥).

### PERFORMANCE AND DBOT AGREEMENT SECURITY

The Tender Security of the successful Tenderer will be discharged not later than fifteen (15) days after the signing of the Contract and replaced by a Performance Security valid until the statement of reception of the Contractor’s installations in the amount of one million five hundred thousand (1,500,000) US Dollars.

The Performance Security will be discharged not later than fifteen (15) days after the signing of the statement of reception of the Contractor’s installations and replaced by a DBOT Agreement Security valid for twenty-five (25) years in the amount of three million (3,000,000) US Dollars.

١-١٨-٤ ضمان حسن التنفيذ وضمان العقد

سيتم إلغاء ضمان العرض لمقدم العطاء الناجح في موعد لا يتجاوز خمسة عشر (١٥) يومًا بعد توقيع العقد وإستبداله بضمان حسن تنفيذ بمبلغ مليون وخمسمائة ألف (١,٥٠٠,٠٠٠) دولار أمريكي صالح حتى إستلام منشآت المتعاقد.

سيتم إلغاء ضمان حسن التنفيذ في موعد لا يتجاوز خمسة عشر (١٥) يومًا بعد توقيع محضر إستلام منشآت المتعاقد وإستبداله بضمان تعاقد صالح لمدة خمسة وعشرون (٢٥) سنة بمبلغ ثلاثة ملايين (٣,٠٠٠,٠٠٠) دولار أمريكي.

- forms for Tender

٢ - نماذج العرض

FORM OF STATEMENT OF TENDER

نموذج بيان العرض

FORM OF STATEMENT OF TENDER

State the name of the Tenderer and his professional, financial, legal and engineering corporate entities. If more than one corporate entity is involved in the Tender, state the name of each of the members and the commercial relationship between them.

Name of the Tenderer		:

Name of any other corporate entity		:

Nature of commercial relationship between corporate entities		:

Names of Consultants		:

نموذج بيان العرض

اذكر إسم العارض وكياناته المهنية، المالية، القانونية والهندسية. إذا كانت هناك أكثر من كيانات قانونية واحدة تشارك في العرض، فاذكر إسم كل من الأعضاء والعلاقة التجارية بينهم.

إسم العارض:

إسم أي كيان قانوني آخر:

طبيعة العلاقة التجارية بين الكيانات القانونية:

أسماء الإستشاريين:

FORM OF INVITATION

نموذج دعوة

FORM OF INVITATION

To

Port of Tripoli

Office d'Exploitation du Port de Tripoli

Tripoli, EI Mina

LEBANON

We, the Contractor, ……………………………….… acknowledged that we have been qualified in order to prepare a Tender for a Design, Build, Operate and Transfer contract of a grain silo at the Port of Tripoli, we have taken notice of and will comply with the information listed below:

Time limit for submission of Tender

Date of the pre-Tender meeting

Tender validity                                           180 Days

Amount of Tender Security                      800,000 USD

Amount of Performance Security           1,500,000 USD

Amount of DBOT Agreement Security   3,000,000 USD

Tender fees                                                 Free of charge

No of Contracting Authority receipt

Signature by the Contractor

Signed

Name

Seal of the Contractor

نموذج الدعوة

إلى

مرفأ طرابلس

مصلحة إستثمار مرفأ طرابلس

طرابلس، الميناء

لبنان

نحن، المتعاقد، ................................... نقر بأننا مؤهلين لإعداد عرض لتصميم، بناء، تشغيل وتحويل إهراءات

حبوب في مرفأ طرابلس، وقد أخذنا علماً وسنلتزم بالمعلومات الواردة أدناه:

الموعد النهائي لتقديم العرض

تاريخ إجتماع ما قبل تقديم العرض

صلاحية العرض ١٨٠ يوم

مبلغ ضمان العرض ٨٠٠,٠٠٠ دولار أمريكي

مبلغ ضمان حسن التنفيذ ١,٥٠٠,٠٠٠ دولار أمريكي

مبلغ ضمان العقد ٣,٠٠٠,٠٠٠ دولار أمريكي

رسم الإلتزام مجاني

عدد إيصالات سلطة التعاقد

توقيع المتعاقد

التوقيع

الاسم

ختم المتعاقد

FORM FOR CONTRACTUAL COMMITMENT

نموذج الإلتزام التعاقدي

FORM FOR CONTRACTUAL COMMITMENT

Name of Project:

To:    Port of Tripoli

Office d'Exploitation du Port de Tripoli

Tripoli, EI Mina

LEBANON

I the undersigned ___________________________________________________ dully authorized to sign

on a behalf of _____________________________________________________________________

having as permanent Address in __________________________________________________________________________________

Telephone No: _________________

Facsimile    No: _________________

Acknowledge my revision and examination of the Bid Documents and contract conditions and all other documents pertaining to the execution of the above-named project.

I proclaim that after reviewing and examining the Bid Documents and contract conditions which I may not claim to ignore and after visiting, inspecting and examining the Site of the Works and its surroundings and having obtained all required information and being aware of the Contract details, difficulties and obstacles if any, of the required Works,

I pledge, if any Bid is accepted, to execute and complete all the required Works and remedy any defects therein in conformity with the said Bid Documents and contract conditions within the Time for completion stated in the Contract Documents.

I also acknowledge that I have put the prices and accepted the stipulations stated in the Conditions of Contract, taking into consideration all the Bid Documents and contract conditions and difficulties that may be encountered, if any, during the execution of the Works.

Date			:

Name of Bidder		:

Name			:

In the Capacity of	:

Signature		:

Address			:

Stamp 1,000,000.00 L.L. (One million Lebanese Pounds)

نموذج الإلتزام التعاقدي

اسم المشروع:

إلى: مرفأ طرابلس

مصلحة إستثمار مرفأ طرابلس

طرابلس، الميناء

لبنان

أنا الموقع أدناه ___________________________________________________ مفوض بالتوقيع نيابة عن

________________________________________________________________
الذي عنوانه الدائم في

___________________________________________________________________

رقم الهاتف ________________:

رقم الفاكس _________________:

أقر بمراجعة وفحص مستندات الإلتزام وشروط العقد وجميع المستندات الأخرى المتعلقة بتنفيذ المشروع المسمى أعلاه.

أعلن أنه بعد مراجعة وفحص مستندات الإلتزام وشروط العقد التي لا يمكنني الادعاء بتجاهلها، وبعد زيارة وفحص موقع الأعمال وما يحيط به، وبعد الحصول على كافة المعلومات المطلوبة وإدراكي لتفاصيل العقد والصعوبات والعوائق، إن وجدت، للأعمال المطلوبة،

أتعهد، إذا تم قبول أي عرض، بتنفيذ وإتمام جميع الأعمال المطلوبة وإصلاح أي عيوب فيها وفقاً لمستندات الإلتزام وشروط العقد ضمن الفترة الزمنية المحددة في مستندات العقد.

كما أقر أنني قد وضعت الأسعار وقبلت الشروط المنصوص عليها في شروط العقد، مع الأخذ بعين الإعتبار جميع مستندات الإلتزام وشروط العقد والصعوبات التي قد تواجه أثناء تنفيذ الأعمال.

التاريخ:
اسم العارض:

الاسم:
بصفة:
التوقيع:
العنوان:
ختم ١٬٠٠٠٬٠٠٠ ل.ل. (مليون ليرة لبناني)

Form of Tender Security (Bank Guarantee)

نموذج ضمان العرض (ضمان مصرفي)

Form of Tender Security (Bank Guarantee).

WHEREAS, [name of Tenderer] (hereinafter called "the Tenderer") has submitted his Tender dated (date) for the DBOT of a grain silo at the Port of Tripoli (hereinafter called "the Tender").

KNOW ALL PEOPLE by these presents that We [name of Bank] of [name of country] having our registered office at [address] (hereinafter called "the Bank") are bound unto [name of Contracting Authority] (hereinafter called "the Contracting Authority") in the “fresh” sum of US $800,000 (Eight hundred thousand US Dollars) for which payment well and truly to be made to the said Contracting Authority the Bank binds himself, his successors, and assigns by these presents.

SEALED with the Common Seal of the said Bank this _______________ day of _____ 20________.

THE CONDITIONS of this obligation are:

- if the Tenderer withdraws his Tender during the period of Tender validity specified in the Form of Tender; or

- if the Tenderer refuses to accept the correction of errors in his Tender; or

- if the Tenderer, having been notified of the acceptance of his Tender by the Contracting Authority during the period of Tender validity;

- fails or refuses to execute the Form of Agreement in accordance with the Instructions to Tenderers, if required; or

- fails or refuses to furnish the Performance Security, in accordance with the Instruction to Tenderers;

we undertake to pay to the Contracting Authority up to the above amount upon receipt of his first written demand, without the Contracting Authority having to substantiate his demand, provided that in his demand the Contracting Authority will note that the amount claimed by him is due to him owing to the occurrence of one of the above conditions, specifying the occurred condition or conditions.

This Guarantee will remain in force up to and including the date 28 days after the date of expiration of the Tender Validity, as stated in the Instructions to Tenderers, or as it may be extended by the Contracting Authority, notice of which extension(s) to the Bank is hereby waived.

Any demand in respect of this Guarantee should reach the Bank not later than the above date.

DATE    ________________     SIGNATURE OF THE BANK     ________________________________

WITNESS ____________________________________

SEAL _______________________________________

____________________________

[Signature, name, and address]

نموذج ضمان العرض (ضمان مصرفي)

حيث أن [اسم العارض] (المشار إليه لاحقًا بـ "العارض") قد قدّم عرضه بتاريخ (التاريخ) لتنفيذ عقد تصميم، بناء، تشغيل ونقل إهراءات حبوب في مرفأ طرابلس (المشار إليه لاحقًا بـ "الإلتزام").

نُعلِم الجميع بموجب هذا أنَّنا [اسم البنك] من [اسم الدولة] والواقع مقرنا المسجل في [العنوان] (المشار إليه لاحقًا بـ "البنك") نلتزم أمام [اسم سلطة التعاقد] (المشار إليه لاحقًا بـ "سلطة التعاقد") بمبلغ "فريش" قدره ٨٠٠,٠۰٠ دولار أمريكي (ثمانمائة ألف دولار أمريكي) والذي سيتم دفعه بشكل صحيح تمامًا إلى سلطة التعاقد، ونلتزم نحن، ومن بعدنا من خلفاء ومستحقين، بذلك.

ختم بنكنا بختمه الرسمي هذا اليوم ____________ من شهر __________ سنة __________

شروط هذا الإلتزام هي:

- إذا سحب العارض عطاءه خلال فترة صلاحية العرض المحددة في نموذج العرض؛ أو

- إذا رفض العارض قبول تصحيح الأخطاء في عرضه؛ أو

- إذا تم إعلام العارض بقبول عرضه من قبل سلطة التعاقد خلال فترة صلاحية العرض؛
أ) فشل أو رفض في توقيع نموذج العقد وفقًا لتعليمات العارضين، إذا كان ذلك مطلوبًا؛ أو
ب) فشل أو رفض في تقديم ضمان حسن التنفيذ وفقًا لتعليمات العارضين؛

نلتزم بدفع المبلغ المذكور لسلطة التعاقد عند إستلامه أول طلب خطي منه، دون الحاجة إلى إثبات سلطة التعاقد لطلبه، بشرط أن يذكر سلطة التعاقد في طلبه أن المبلغ المطلوب هو بسبب وقوع أحد الشروط المذكورة أعلاه، مع تحديد الشرط أو الشروط التي وقعت.

سيظل هذا الضمان ساريًا حتى تاريخ إنتهاء صلاحية العرض، أي ٢٨ يومًا بعد تاريخ إنتهاء صلاحية العرض، كما هو مذكور في تعليمات العارضين، أو كما قد يتم تمديده من قبل سلطة التعاقد، ويتم التنازل بموجب هذا عن أي إشعار بهذا التمديد للبنك.

يجب أن تصل أي مطالبة بشأن هذا الضمان إلى البنك في موعد لا يتجاوز التاريخ المذكور أعلاه.

التاريخ        _________________

توقيع البنك   _________________

الشاهد       __________________

الختم         __________________

FORM OF PERFORMANCE SECURITY

(UNCONDITIONAL)

نموذج ضمان حسن التنفيذ (غير مشروط)

Performance Bank Guarantee (Unconditional)

To

Port of Tripoli

Office d'Exploitation du Port de Tripoli

Tripoli, El Mina

LEBANON

WHEREAS [name and address of the signatory of the DBOT Agreement] (hereinafter called "the Contractor") has undertaken, in pursuance of Contract No. ____ dated ____________ to execute the DBOT of a grain silo at the Port of Tripoli (hereinafter called "the Contract.");

AND WHEREAS it has been stipulated by you in the said Contract that the Contractor shall furnish you with a Bank Guarantee by a recognized bank for the “fresh” sum specified therein as security for compliance with his obligations in accordance with the Contract;

AND WHEREAS we have agreed to give the Contractor such a Bank Guarantee;

NOW THEREFORE we hereby affirm that we are the Guarantor and responsible to you, on behalf of the Contractor, up to a total of US$ 1,500,000 (one million five hundred thousand US dollars) such “fresh” sum being payable in the types and proportions of currencies in which the Contract Price is payable, and we undertake to pay you, upon your first written demand and without cavil or argument, any sum or sums within the limits of US $1,500,000 (one million five hundred thousand US dollars) as aforesaid without your needing to prove or to show grounds or reasons for your demand for the sum specified therein.

We hereby waive the necessity of your demanding the said debt from the Contractor before presenting us with the demand.

We further agree that no change or addition to or other modification of the terms of the Contract or of the Works to be performed thereunder or of any of the Contract Documents that may be made between you and the Contractor shall in any way release us from any liability under this guarantee, and we hereby waive notice of any such change, addition, or modification.

This guarantee shall be valid until a date 28 days from the date of issue of the statement of reception of the Contractor's installations.

SIGNATURE AND SEAL OF THE GUARANTOR

______________________________________________________________________________________

Name of Bank :

Address            :

Date                  :

ضمان حسن التنفيذ من البنك (غير مشروط)

إلى

مرفأ طرابلس

مصلحة استثمار مرفأ طرابلس

طرابلس، الميناء

لبنان

حيث أن [اسم وعنوان موقّع إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية] (المشار إليه فيما بعد بـ "المتعاقد") قد تعهد، وفقًا للعقد رقم ____ بتاريخ ____________، بتنفيذ مشروع تصميم، بناء، تشغيل ونقل (DBOT) إهراءات حبوب في مرفأ طرابلس (المشار إليه فيما بعد بـ "العقد")؛

وحيث أنه قد تم الإشتراط من قبلكم في العقد المشار إليه أن يقدم المتعاقد لكم ضمانًا بنكيًا من بنك معترف به بمبلغ "فريش" محدد هناك كضمان للإمتثال لإلتزاماته بموجب العقد؛

وحيث أننا قد وافقنا على تقديم هذا الضمان البنكي للمتعاقد؛

لذلك نؤكد بموجب هذا أننا الكفيلون والمسؤولون أمامكم نيابة عن المتعاقد، بمبلغ إجمالي قدره ١,٥٠٠,٠٠٠ دولار أمريكي (مليون وخمسمائة ألف دولار أمريكي) وهذا المبلغ "الفريش" قابل للدفع بالعملات وبالنسب التي يُدفع بها سعر العقد، ونتعهد بدفع أي مبلغ أو مبالغ ضمن حدود ١,٥٠٠,٠۰٠ دولار أمريكي (مليون وخمسمائة ألف دولار أمريكي) عند طلبكم الأول المكتوب ودون إعتراض أو نقاش، دون الحاجة لإثبات أو إظهار الأسباب أو المبررات لمطالبتكم بالمبلغ المحدد فيه.

نحن نتنازل بموجب هذا عن ضرورة مطالبتكم بالديون من المتعاقد قبل تقديم المطالبة لنا.

ونتفق أيضًا على أن أي تغيير أو إضافة أو تعديل آخر لشروط العقد أو للأعمال التي يجب تنفيذها بموجب العقد أو أي من مستندات العقد التي قد تُجرى بينكم وبين المتعاقد لن يعفي بأي حال من الأحوال من مسؤوليتنا بموجب هذا الضمان، ونحن نتنازل عن إشعار أي تغيير أو إضافة أو تعديل من هذا القبيل.

سيظل هذا الضمان ساريًا حتى تاريخ ٢٨ يومًا من تاريخ إصدار محضر إستلام منشآت المتعاقد.

التوقيع والختم من الكفيل

اسم البنك:

العنوان:

التاريخ:

FORM OF DBOT AGREEMENT SECURITY

(UNCONDITIONAL)

نموذج ضمان العقد (غير مشروط)

DBOT Agreement Bank Guarantee (Unconditional)

To

Port of Tripoli

Office d'Exploitation du Port de Tripoli

Tripoli, El Mina

LEBANON

WHEREAS [name and address of the signatory of the DBOT Agreement] (hereinafter called "the Contractor ") has undertaken, in pursuance of Contract No. ____ dated ____________ to execute the DBOT of a grain silo at the Port of Tripoli (hereinafter called "the Contract.");

AND WHEREAS it has been stipulated by you in the said Contract that the Contractor shall furnish you with a Bank Guarantee by a recognized bank for the “fresh” sum specified therein as security for compliance with his obligations in accordance with the Contract;

AND WHEREAS we have agreed to give the Contractor such a Bank Guarantee;

NOW THEREFORE we hereby affirm that we are the Guarantor and responsible to you, on behalf of the Contractor, up to a total of US$ 3,000,000 (three million US Dollars) such “fresh” sum being payable in the types and proportions of currencies in which the Contract Price is payable, and we undertake to pay you, upon your first written demand and without cavil or argument, any sum or sums within the limits of US$ 3,000,000 (three million US Dollars) as aforesaid without your needing to prove or to show grounds or reasons for your demand for the sum specified therein.

We hereby waive the necessity of your demanding the said debt from the Contractor before presenting us with the demand.

We further agree that no change or addition to or other modification of the terms of the Contract or of the Works to be performed thereunder or of any of the Contract Documents that may be made between you and the Contractor shall in any way release us from any liability under this guarantee, and we hereby waive notice of any such change, addition, or modification.

This guarantee shall be valid until ………. (DBOT Agreement duration being 25 years).

SIGNATURE AND SEAL OF THE GUARANTOR

______________________________________________________________________________________

Name of Bank:

Address            :

Date                  :

ضمان العقد من البنك (غير مشروط)

إلى

مرفأ طرابلس

مصلحة إستثمار مرفأ طرابلس

طرابلس، الميناء

لبنان

حيث أن [اسم وعنوان موقّع إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية] (المشار إليه فيما بعد بـ "المتعاقد") قد تعهد، وفقًا للعقد رقم ____ بتاريخ ____________، بتنفيذ مشروع تصميم، بناء، تشغيل ونقل (DBOT) إهراءات حبوب في مرفأ طرابلس (المشار إليه فيما بعد بـ "العقد")؛

وحيث أنه قد تم إشتراط من قبلكم في العقد المشار إليه أن يقدم المتعاقد لكم ضمانًا بنكيًا من بنك معترف به بمبلغ "فريش" محدد هناك كضمان للإمتثال لإلتزاماته بموجب العقد؛

وحيث أننا قد وافقنا على تقديم هذا الضمان البنكي لسلطة التعاقد؛

لذلك نؤكد بموجب هذا أننا الكفيلون والمسؤولون أمامكم نيابة عن المتعاقد، بمبلغ إجمالي قدره ٣,٠٠٠,٠٠٠ دولار أمريكي (ثلاثة ملايين دولار أمريكي) وهذا المبلغ "الفريش" قابل للدفع بالعملات وبالنسب التي يُدفع بها سعر العقد، ونتعهد بدفع أي مبلغ أو مبالغ ضمن حدود ٣,٠٠٠,٠٠٠ دولار أمريكي (ثلاثة ملايين دولار أمريكي) عند طلبكم الأول المكتوب ودون إعتراض أو نقاش، دون الحاجة لإثبات أو إظهار الأسباب أو المبررات لمطالبتكم بالمبلغ المحدد فيه.

نحن نتنازل بموجب هذا عن ضرورة مطالبتكم بالديون من المتعاقد قبل تقديم المطالبة لنا.

ونتفق أيضًا على أن أي تغيير أو إضافة أو تعديل آخر لشروط العقد أو للأعمال التي يجب تنفيذها بموجب العقد أو أي من مستندات العقد التي قد تُجرى بينكم وبين المتعاقد لن يعفينا بأي حال من الأحوال من مسؤوليتنا بموجب هذا الضمان، ونحن نتنازل عن إشعار أي تغيير أو إضافة أو تعديل من هذا القبيل.

سيظل هذا الضمان ساريًا حتى ………. (مدة العقد ٢٥ سنة).

التوقيع والختم من الكفيل

اسم البنك:

العنوان:

التاريخ:

FORM OF TENDER

For Design, Build, Operate and Transfer

of a grain silo at the Port of Tripoli

Address:                                    			 Date:

Dear. Sir,

- Having examined the Tender conditions, the DBOT Agreement, the form of Tender, the Contract Agreement, the form of Tender security, the Contracting Authority's requirements and all Bid Documents forming part of the Contract Documents and having visited the site and having obtained all information necessary to submit a Tender, we the undersigned, hereby offer to Design, Build, Operate and Transfer a grain silo at the Port of Tripoli for a period of 25 (twenty five) years for the following conditions and remuneration to be paid by the Contractor under the Contract requirements:

Annual rental Fee: ………………% (of the storage and handling Fees)

- We acknowledge receipt of, and have considered, the following addenda issued do Tenderers during the Tender period: ……………………………………………………………

- We agree to tender by this Tender for a period of one hundred eighty (180) days from the latest date for receipt of Tender ……………………… and it may be accepted by you at any time before the expiry of that period. As a guarantee of our Tender, we have submitted a Tender security issued by: ……………………………………………………………………………………… for the sum of US$ 800,000 (Eight hundred thousand US Dollars).

- Until a formal Agreement is prepared and executed, this Tender, together with your written acceptance thereof, shall constitute a binding Contract between us, subject to the Law and procedures of the government of Lebanon.

- We understand that you are not bound to accept the highest of any Tender that you might receive.

Dated this ……………………………………………………………………………………

Signature …………………………………………………………………………………….

Name:

In the capacity of…………………………………………………………………………

Duly authorized to sign Tenders on behalf of:

Witness signature…………………………………………………….………………….

Name:

Address:

نموذج تعهد

لتصميم، بناء، تشغيل ونقل

إهراءات حبوب في مرفأ طرابلس

العنوان: 										التاريخ:

حضرة................................................................،

- بعد مراجعة شروط الإلتزام، إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية، نموذج التعهد، العقد، نموذج ضمان العرض، متطلبات سلطة التعاقد وجميع مستندات الإلتزام التي تشكل جزءًا من مستندات العقد، وبعد زيارة الموقع والحصول على جميع المعلومات اللازمة لتقديم العرض، نحن الموقعون أدناه، نعرض تقديم عرض لتصميم، بناء، تشغيل ونقل إهراءات حبوب في مرفأ طرابلس لمدة ٢٥ (خمسة وعشرون) سنة وفقًا للشروط والأجر التالي الذي سيدفعه المتعاقد بموجب متطلبات العقد:

الرسم السنوي للإيجار ………………% (من رسوم التخزين والمناولة)

- نقر بإستلامنا، وأخذنا بعين الاعتبار، الإضافات التالية التي تم إصدارها للمشتركين في الإلتزام خلال فترة العرض: ……………………………………………………………

- نوافق على تقديم العرض لمدة مئة وثمانين (١٨٠) يومًا من آخر تاريخ إستلام العرض ……………………… ويمكن قبوله من قبلكم في أي وقت قبل إنتهاء هذه الفترة. وكضمان لعرضنا، قدمنا ضمان عطاء صادر عن : ……………………………………………………………………………………… بمبلغ ٨٠٠,٠٠٠ دولار أمريكي (ثمانمائة ألف دولار أمريكي).

- حتى يتم إعداد وتنفيذ إتفاقية رسمية، يعتبر هذا العرض، مع قبولكم المكتوب له، بمثابة عقد ملزم بيننا، وفقًا للقانون والإجراءات المتبعة في لبنان.

- نفهم أنه ليس ملزمًا بقبول أعلى أو أي عطاء قد تتلقونه.

بتاريخ …………………………………………………………………………………

التوقيع ………………………………………………………………………………….

الإسم:

بصفة …………………………………………………………………………

مخول بالتوقيع على العروض نيابة عن: ……………………………………………

توقيع الشاهد

الإسم:

العنوان:

- LETTER OF ACCEPTANCE

٣ - كتاب القبول

Port of Tripoli

Office d'Exploitation du Port de Tripoli

Tripoli, El Mina

LEBANON

Dear Sirs,

Tender for Design, Build, Operate and Transfer

of a grain silo at the Port of Tripoli

With reference to your submitted Tender dated………………………. in respect of the above project and with reference to Tender conditions of the Tender Documents,

We hereby inform you by this LETTER OF ACCEPTANCE that your Tender for the above-named project is accepted.

Please acknowledge by signing the attached duplicate of this letter, the receipt of this LETTER OF ACCEPTANCE and your full agreement to its contents, without any reserve.

Yours faithfully,

For

Port of Tripoli

Office d’Exploitation du Port de Tripoli

Tripoli, El Mina

LEBANON

Port Director

We hereby acknowledge receipt for the LETTER OF ACCEPTANCE and accept the contents thereof without any reserve.

Named of Authorized Signatory

………………………………………………..

Title: ……………………………………….

Company: ……………………………….

Signature: ……………………………….

Date: ………………………………………

مرفأ طرابلس
مصلحة إستثمار مرفأ طرابلس
طرابلس، الميناء
لبنان

السادة المحترمين،

عرض لتصميم، بناء، تشغيل ونقل إهراءات حبوب في مرفأ طرابلس

بالإشارة إلى عرضكم المقدم بتاريخ …………………………. بخصوص المشروع المشار إليه أعلاه، وبالرجوع إلى شروط الإلتزام الواردة في مستندات الإلتزام:

نحيطكم علمًا من خلال كتاب القبول هذا بأن عرضكم للمشروع المذكور قد تم قبوله.

يرجى التأكيد بتوقيع النسخة المرفقة من هذا الكتاب، مع العلم بإستلامكم كتاب القبول ومواءمتكم التامة مع محتوياتها، دون أي تحفظ.

وتفضلوا بقبول فائق الاحترام،
عن
مرفأ طرابلس
مصلحة إستثمار مرفأ طرابلس
طرابلس، الميناء
لبنان

مدير المرفأ

نحن بموجب هذا نقر بإستلام كتاب القبول ونقبل محتوياتها دون أي تحفظ.

إسم الموقع المخول …………………………….

الوظيفة ………………………………………

الشركة ………………………………

التوقيع ………………………………

التاريخ ………………………………………

- Contract Agreement

٤ - عقد إتفاق

CONTRACT AGREEMENT

For Design, Build, Operate and Transfer

of a grain silo at the Port of Tripoli

THIS AGREEMENT is made the ………………………………………………….……………………………………..…………… day of…………………………….……………..…between…………………………………………………...…………………………...

whose offices are…………………… (herein after called "The Contracting Authority") on the one part, and (herein after called "The Contractor”) the other part.

WHEREAS the Contracting Authority has accepted a Tender from the Contractor for the Design, Build, Operate and Transfer (DBOT) contract of a grain silo at the Port of Tripoli for a period of 25 (twenty-five) years (herein after called "The DBOT Agreement duration").

NOW HE PARTIES HEREBY AGREE AS FOLLOWS:

- In this Agreement, words and expressions shall have the respective meanings assigned to them in the DBOT Agreement.

- The following Documents shall be read and construed as forming part of this Contract Agreement.

- The Minutes of Meetings (if any)

- Letter of acceptance

- The Addendum (if any)

- The mentioned Tender document

- The DBOT Agreement thereto

- The Contracting Authority’s Requirements

- The Contractor undertakes to Design, Build, Operate and Transfer of a grain silo at the Port of Tripoli all in accordance with the Contract.

- In consideration of the Design, Build, Operate and Transfer of a grain silo at the Port of Tripoli, the Contracting Authority undertakes to grant the Contractor the Contract for the DBOT Agreement duration and the Contractor undertakes to pay the Contracting Authority the fees stipulated in the Contract.

- IN, WITNESS WHEREOF, the Parties hereto have caused this Agreement to be made on the date first mentioned above.

SIGNED, SEALED AND DELIVERED

By the Contracting Authority           	by the Contractor

Signed                    	Signed

Name                    	Name

In the capacity of              	In the capacity of

Witness signature               	Witness signature

Name                     	Name

Address                    	Address

SEAL OF CONTRACTING AUTHORITY              	SEAL OF CONTRACTOR

AFFIXED BELOW                	AFFIXED BELOW

عقد إتفاق

لتصميم، بناء، تشغيل ونقل إهراءات حبوب في مرفأ طرابلس

تم عقد هذا الاتفاق في يوم

……………………………………………….……………………………………..…………… من شهر……………………………….……………..… بين

……………………………………………………...…………………………... (والتي يشار إليها لاحقاً بـ "سلطة التعاقد") من جهة، و (موقّع إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية") (والذي يشار إليه لاحقاً بـ "المتعاقد") من جهة أخرى.

حيث أن سلطة التعاقد قد قبل عرضا من المتعاقد لتنفيذ عقد التصميم، البناء، التشغيل والنقل (DBOT) لإهراءات الحبوب في مرفأ طرابلس لمدة ٢٥ (خمسة وعشرون) سنة (والتي يشار إليها لاحقاً بـ "مدة العقد").

يتفق الطرفان على ما يلي:

١ - في هذه الإتفاقية، سيكون للكلمات والتعابير المعاني المحددة لها في إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية.

٢ - تعتبر الوثائق التالية جزءاً من هذه الاتفاقية ويتم قراءتها وتفسيرها كجزء من عقد الاتفاق:

أ - محاضر الاجتماعات (إن وجدت)

ب - كتاب القبول

ج - التعديلات (إن وجدت)

د - مستندات العطاء المشار إليها

هـ - إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية

و - متطلبات سلطة التعاقد

٣ - يتعهد المرخص له بتصميم، بناء، تشغيل ونقل إهراءات الحبوب في مرفأ طرابلس وفقًا للعقد.

٤ - في مقابل تصميم، بناء، تشغيل ونقل إهراءات الحبوب في مرفأ طرابلس، يتعهد سلطة التعاقد بمنح المتعاقد العقد لمدة العقد، ويتعهد المتعاقد بدفع الرسوم المحددة في العقد.

٥ - تحت شهادة الشهود، قام الطرفان بعقد هذه الإتفاقية في التاريخ المذكور أولاً أعلاه.

تم التوقيع والختم من قبل

| سلطة التعاقد التوقيع………………………… الاسم ………………………… الوظيفة………………………… | المتعاقد التوقيع………………………… الاسم ………………………… الوظيفة………………………… |
| --- | --- |

- DBOT AGREEMENT

٥ – إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية

Development and operation Agreement

for a grain silo in the port of Tripoli

(LEBANON)

DBOT AGREEMENT

PREAMBLE

PRIOR TO THIS DBOT AGREEMENT, THE FOLLOWING WAS EXPLAINED:

The Port of TRIPOLI wanted to supplement its facilities by installing a grain silo within the port domain for the storage of all sort of grains listed in this document.

The required installations should be a steel silo plant with all necessary and auxiliary equipment and infrastructure, able to ensure a safe and adequate storage of grains in conformity with local and international standards.

AFTER WHICH, THE PROVISIONS OF THIS AGREEMENT ARE GIVEN BELOW:

Between:

hereinafter called the “Contracting Authority” First part,

And:

The signatory of the DBOT Agreement hereinafter called the “Contractor” Second part

The following was agreed:

إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية

لتطوير وتشغيل إهراءات حبوب في مرفأ طرابلس

(لبنان)

عقد إتفاق

مقدمة

قبل هذه الاتفاقية، تم شرح ما يلي:

كان مرفأ طرابلس يرغب في تعزيز مرافقه عن طريق تركيب إهراءات حبوب ضمن نطاق المرفأ لتخزين جميع أنواع الحبوب المدرجة في هذا الدفتر.

يجب أن تكون المنشآت المطلوبة عبارة عن صوامع معدنية مع جميع المعدات والبنية التحتية اللازمة والملحقة بها، قادرة على ضمان تخزين الحبوب بشكل آمن وكافٍ وفقًا للمعايير المحلية والدولية.

بعد ذلك، تم تحديد أحكام هذه الاتفاقية على النحو التالي:

بين:

(اسم سلطة التعاقد)، الذي يُشار إليه فيما بعد بـ "سلطة التعاقد"، الطرف الأول،

و:

(اسم موقّع إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية)، الذي يُشار إليه فيما بعد بـ "المتعاقد"، الطرف الثاني.

تم الاتفاق على ما يلي:

- PART ONE

Subject and nature of the DBOT Agreement

Article 1: Subject of the DBOT Agreement

The subject of this DBOT Agreement is to define the conditions for design, implementation and operation of the following installations, equipment and tools, assigned to grain storage using, in particular, a grain silo;

The below mentioned documents should be considered as a part of the DBOT Agreement:

- INSTRUCTIONS TO TENDERERS

- FORMS FOR TENDER

- LETTER OF ACCEPTANCE

- CONTRACT AGREEMENT

- CONTRACTING AUTHORITY’S REQUIREMENTS

- DBOT AGREEMENT

- ADDENDUM (IF ANY)

- APPENDIX 1: DRAWINGS

- MINUTES OF MEETING (IF ANY)

Installations funded, designed, installed and maintained by the Contractor (refer to attached drawings in Appendix 1 and to technical specifications) are, but not limited to:

- A water body maintained and dredged to a depth of -15.50m NGL in the silo location along 200m of the existing quay wall, ensuring an adequate berthing post for the grain ship

- A quay wall maintained during the DBOT Agreement period (200m length, 15.50m depth) for ships mooring and silo access.

- Grain silo, with the following minimum characteristics, dimensions and capacities:

● 120,000T total capacity

● 5,000T capacity per steel bin with flat bottom

● Possibility for storage of wheat, corn, barley, sunflower and other

The grain silo must be completely operational and ready for use, according to international rules and standards and fully classified by recognized classification societies.

- One ship loader (Minimum loading capacity 400T/hr).

- Two ship unloaders (Minimum unloading capacity 200T/hr).

- 9 flat bottom bins (1,500T each).

- Several loading trucks stations.

- One unloading truck station.

- Auxiliary civil and building works that must be executed on the same land area of the silo, which will be occupied by the Contractor.

- Water and electricity supply utilities.

- Environmental protection facilities.

- All equipment and tools needed to operate the silo according to international recognized standards.

In this regard, it is not allowed the use of land-based cranes for ships loading/unloading operations.

- The Contractor will have the opportunity to occupy a land adjacent to the container terminal and the main quay wall for a total area of approximate 36,000m2 (refer to Appendix N1) which will be used for installing the silo, storage of materials and equipment, construction of offices and workshop, in addition to miscellaneous buildings needed to operate the facility

No fees for the above-mentioned occupancy shall be asked from the Contractor during the DBOT Agreement period.

- Maintenance of all areas occupied by the Contractor, and all other Port facilities used by him

- The Contractor shall make available at any time a strategic stock of wheat, for a minimum quantity up to 20% of the total silo capacity, within the bins, for the sole usage of the Public Authority at any time.

- The minimum unloaded quantity of grains shall be not be less than 250,000 T/year, with a minimum of 20% of this mentioned quantity stored within the silo.

- The Contractor shall receive all wheat cargo ships accessing the Port of Tripoli

The minimum DWT of grain cargo ships (other than wheat) received by the Contractor shall be greater than 10,000 DWT. The maximum ship size for loading/unloading operations have to meet the infrastructure capacities of the berth:

- Berth depth@-15.50m MSL

- Berth length: 200m

- The DBOT Agreement period will be set for a period of 25 years, to be counted from the date of the Contracting Authority temporary reception of all executed works.

The maximum estimated duration to execute the above-mentioned works will be 24 months, counted from the date of the contract signature, and during which no fees are to be paid by the Contractor regarding the DBOT Agreement annual rental fee.

No fees for land occupation should be paid by the Contractor, except for using other miscellaneous facilities (like electricity, water…) and which will be paid according to regulations.

Each of the Contractor installations will comply with the international recognized standards and the ones set by the Contracting Authority in the technical specifications enclosed with this Agreement, and shall be approved by the Contracting Authority nominated consultant before and during execution till their handing over.

The initial development work for these installations will be performed in compliance with Article 3 below.

The drawings showing the perimeter of the DBOT Agreement is appended to this DBOT Agreement (Appendix 1).

It includes a conceptual overall mass plan for the facility and the open areas and the water body which belong to the Contracting Authority and made available to the Contractor.

The water body shall be available for the temporary berth of ships for grains loading/unloading; nevertheless, the Contracting Authority can use this berth for general cargo handling operations.

The listed land, open spaces and water bodies, stating their surface area, the list of facilities, equipment and tools shall be confirmed by a statement produced jointly by the Contracting Authority and the Contractor.

Any modification will be confirmed by a statement produced in the same way.

Roles and authorities of different parties

- The Consultant Engineer is empowered to represent the Contracting Authority, to take all actions and give all instructions and Approvals in the Contracting Authority’s name and on its behalf, under the Contract, following a close review and study with the Contracting Authority of all the mentioned instructions, except where otherwise provided under the Contract, or where the Contracting Authority find it appropriate: All communications of any kind between the Contractor  and the Contracting Authority shall be addressed in writing through the Consultant Engineer, which will be responsible for making and/or obtaining the necessary reply.

- All communications of any kind between the Contractor and the Consultant Engineer shall be addressed in writing, with a copy to the Contracting Authority.

- The Contractor will appoint a representative, referred herein as “The Contractor Representative” who is empowered to act at all times for and on behalf of and to bind the Contractor, in all matters relating to the Contract.

Such appointment and any revocation thereof shall be notified in writing by the Contractor to the Contracting Authority.

The Contractor shall submit for the Contracting Authority approval, the nomination of the “The Contractor Representative” with all supported Documents (CV – experience…), within one week, following the Contract signature.

The “The Contractor Representative” shall be:

- A qualified civil engineer with a maritime experience for not less than 20 years.

- Member of the Lebanese Order of Engineers (at Beirut or Tripoli).

The Contracting Authority can, at any time, ask for the revocation of the Contractor Representative without any objection of the Contractor.

- No Approval or oral approval or commitment given by the Contracting Authority or on behalf of the Contracting Authority by the Consultant Engineer shall:

- change, relieve, discharge or release the Contractor from any of his obligations, responsibilities and undertakings under the Contract, except by way of a Change Order signed by a duly authorized officer of the Contracting Authority;

- constitute a representation, warranty or Agreement that the subject matter of any such Approval or oral approval or commitment (including without limitation, the Detailed Program, the Contractor’s design, Construction Materials or construction methods), for which the Contractor alone shall at all times remain responsible, is feasible, appropriate or meets or complies with the design intent, purpose intended or other Requirements of the Contract.

- The Contractor agrees that all disputes with the Contracting Authority relating to the Contract shall be resolved amicably, if not, by arbitration pursuant to Lebanese laws.

- الجزء الأول

موضوع وطبيعة العقد

المادة الأولى: موضوع العقد

موضوع إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية هذه هو تحديد الشروط المتعلقة بتصميم، تنفيذ، تشغيل المنشآت

والمعدات والأدوات التالية والمخصصة لتخزين الحبوب، بإستخدام بشكل خاص إهراءات الحبوب؛

يجب إعتبار الوثائق المذكورة أدناه جزءًا من إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية:

- تعليمات العارضين.

- نماذج للعرض.

- كتاب القبول

- اتفاقية العقد

- متطلبات سلطة التعاقد

- إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية

- الملاحق (إن وجدت)

- الملحق ١: الرسومات

- محاضر الإجتماعات (إن وجدت)

المنشآت الممولة، المصممة، المنفذة والمصانة من قبل المتعاقد (يرجى الرجوع إلى الرسومات المرفقة في الملحق ١ والمواصفات الفنية) هي، ولكن ليست محدودة بـ:

١ - مسطح مائي يتم صيانته وحفره إلى عمق -١٥,٥٠ متر من مستوى سطح البحر في موقع الإهراءات على طول ٢٠٠ متر من حائط الرصيف الحالي، لضمان وجود مرسى مناسب لسفن الحبوب.

٢ - حائط رصيف يتم صيانته خلال فترة العقد (طول ٢٠٠ متر، عمق ١٥,٥٠ متر) لرسو السفن والوصول إلى الإهراءات.

٣ - إهراءات حبوب، مع الخصائص والأبعاد والقدرات الإستيعاب الدنيا التالية:

- القدرة إستيعابية إجمالية ١٢٠,٠٠٠ طن.

- قدرة ٥,٠٠٠ طن لكل صومعة معدنية مع قاع مسطح.

- إمكانية تخزين القمح، الذرة، الشعير، دوار الشمس والحبوب الأخرى.

يجب أن تكون إهراءات الحبوب قابلة للتشغيل بالكامل وجاهزة للإستخدام وفقًا للقواعد والمعايير الدولية ومصنفة بالكامل من قبل هيئات التصنيف المعترف بها.

٤ - جهاز واحد لتحميل السفن (قدرة تحميل لا تقلّ عن ٤٠٠ طن/ساعة).

٥ - جهازان لتفريغ السفن (قدرة تفريغ لا تقلّ عن ٢٠٠ طن/ساعة لكل منهما).

٦ - صومعة ذات قاع مسطح عدد ٩ (سعة ١,٥٠٠ طن لكل منها).

٧ - عدة محطات شحن للشاحنات.

٨ - محطة واحدة لتفريغ الشاحنات.

٩ - أعمال إنشائية وبناء ملحقة يجب تنفيذها على نفس المساحة المخصصة للإهراءات، والتي سيشغلها المتعاقد.

١٠ - مرافق إمداد المياه والكهرباء.

١١ - منشآت حماية البيئة.

١٢ - جميع المعدات والأدوات اللازمة لتشغيل الإهراءات وفقًا للمعايير المعترف بها دوليًا.
وفي هذا الصدد، يُمنع استخدام الرافعات البرية لعمليات تحميل/تفريغ السفن.

١٣ - سيتمكن المتعاقد من إشغال أرض مجاورة لمحطة الحاويات وحائط الرصيف الرئيسي بمساحة تقريبية تبلغ ٣٦,٠٠٠ متر مربع (يرجى الرجوع إلى الملحق رقم ١) والتي ستستخدم لتركيب الإهراءات، تخزين المواد والمعدات، بناء المكاتب وورش العمل، بالإضافة إلى المباني المتنوعة اللازمة لتشغيل المنشأة.

لن تُفرض رسوم على المتعاقد خلال فترة العقد بالنسبة لإستخدام الأرض المذكورة أعلاه.

١٤ - صيانة جميع المساحات التي يشغلها المتعاقد، وجميع مرافق المرفأ الأخرى التي يستخدمها.

١٥ - يجب على المتعاقد أن يوفر في أي وقت مخزونًا إستراتيجيًا من القمح، يعادل بحد أدنى ٢٠% من إجمالي سعة الإهراءات، داخل الصوامع، لإستخدام السلطات العامة في أي وقت.

١٦ - يجب ألا تقل كمية الحبوب غير المُفرغة في الحد الأدنى عن ٢٥٠,٠٠٠ طن سنويًا، مع تخزين ٢٠% من هذه الكمية كحد الأدنى داخل الإهراءات.

١٧ - يجب على المتعاقد إستقبال جميع سفن شحن القمح التي تصل إلى مرفأ طرابلس.

يجب أن تكون الحمولة في الحد الأدنى لسفن شحن الحبوب (بخلاف القمح) التي يستقبلها المتعاقد أكبر من ١٠,٠٠٠ DWT. يجب أن تتوافق أقصى أحجام السفن لعمليات التحميل/التفريغ مع قدرات بنية الرصيف:

- عمق الرصيف ١٥,٥٠ متر من مستوى سطح البحر

- طول الرصيف: ٢٠٠ متر

١٨ - ستكون فترة العقد لمدة ٢٥ سنة، بدءًا من تاريخ إستلام سلطة التعاقد المؤقت لجميع الأعمال المنفذة.

المدة القصوى المقدرة لتنفيذ الأعمال المذكورة أعلاه ستكون ٢٤ شهرًا، بدءًا من تاريخ توقيع العقد، وخلال هذه الفترة لن تُدفع الرسوم من قبل المتعاقد والعائدة لرسوم الإيجار السنوي للتعاقد.

لا يجب دفع رسوم لإستخدام الأراضي من قبل المتعاقد، بإستثناء إستخدام المنشآت الأخرى (مثل الكهرباء والمياه...) والتي ستُدفع وفقًا للقوانين.

يجب أن تتوافق كل من منشآت المتعاقد مع المعايير المعترف بها دوليًا ومع تلك المحددة من قبل سلطة التعاقد في المواصفات الفنية المرفقة بهذه الاتفاقية، ويجب أن يوافق عليها استشاري سلطة التعاقد قبل وأثناء التنفيذ حتى تسليمها.

يتم تنفيذ الأعمال الإنشائية الأولية لهذه المنشآت وفقًا للمادة ٣ أدناه.

الرسومات التي توضح محيط العقد مرفقة بهذه الاتفاقية (الملحق رقم ١).

تتضمن مخطط إجمالي تصوري للمرافق والمساحات المفتوحة والمسطح المائي المملوكة من سلطة التعاقد والتي تم توفيرها للمتعاقد.

يجب أن يكون المسطح المائي متاحًا للرسو المؤقت للسفن لتحميل/تفريغ الحبوب؛ ومع ذلك، يمكن لسلطة التعاقد استخدام هذا الرصيف لعمليات مناولة البضائع العامة.

يجب تأكيد الأرض المدرجة والمساحات المفتوحة والمسطح المائي، مع تحديد مساحتها، وقائمة المنشآت والمعدات والأدوات، من خلال محضر مشترك بين سلطة التعاقد والمتعاقد.

سيتم تأكيد أي تعديل من خلال محضر يتم صياغته بنفس الطريقة.

الأدوار والسلطات للأطراف المختلفة

١ - يُمكّن المهندس الإستشاري من تمثيل سلطة التعاقد، وإتخاذ كافة الإجراءات وإعطاء جميع التعليمات والموافقات باسم سلطة التعاقد ونيابة عنه، بموجب العقد، بعد مراجعة ودراسة دقيقة مع سلطة التعاقد لجميع التعليمات المذكورة، ما لم ينص العقد على خلاف ذلك، أو حيثما يراه سلطة التعاقد مناسبًا. يجب أن تكون جميع الإتصالات من أي نوع بين المتعاقد وسلطة التعاقد موجهة كتابيًا من خلال المهندس الإستشاري، الذي سيكون مسؤولاً عن إعداد والحصول على الرد اللازم.

٢ - يجب أن تكون جميع الإتصالات من أي نوع بين المتعاقد والمهندس الإستشاري موجهة كتابيًا، مع نسخة إلى سلطة التعاقد.

٣ - سيقوم ممثل، يُشار إليه هنا بـ "ممثل المتعاقد" الذي يُمكّن من التصرف في جميع الأوقات نيابة عن المتعاقد وملزم بالربط بينه وبين سلطة التعاقد في جميع الأمور المتعلقة بالعقد.
يجب أن يتم إخطار سلطة التعاقد كتابيًا من قبل المتعاقد بتعيين هذا الممثل أو إلغاء تعيينه. يجب على المتعاقد تقديم الموافقة من سلطة التعاقد لتعيين "ممثل المتعاقد" مع جميع المستندات الداعمة (السيرة الذاتية - الخبرة ...) خلال أسبوع من توقيع العقد.

يجب أن يكون "ممثل المتعاقد":

  - مهندس مدني مؤهل ذو خبرة بحرية لا تقل عن ٢٠ عامًا.

  - عضو في نقابة المهندسين اللبنانيين (في بيروت أو طرابلس). يمكن لسلطة التعاقد، في أي وقت، أن يطلب إلغاء تعيين ممثل المتعاقد دون إعتراض من المرخص له.

٤ - لا يُعتبر أي موافقة أو موافقة شفهية أو إلتزام يُعطى من قبل سلطة التعاقد أو من قبل المهندس الإستشاري بإسم سلطة التعاقد:

  - تغيير أو إعفاء أو تفريغ أو إطلاق المتعاقد من أي من إلتزاماته أو مسؤولياته أو تعهداته بموجب العقد، إلا عن طريق أمر تغيير موقّع من قبل موظف مفوض من سلطة التعاقد؛

  - تمثيل أو ضمان أو إتفاق بأن موضوع أي من هذه الموافقات أو الموافقات الشفهية أو الإلتزامات (بما في ذلك، دون حصر، البرنامج التفصيلي، تصميم المتعاقد، مواد البناء أو طرق البناء)، الذي يظل المتعاقد المسؤول الوحيد عنه في جميع الأوقات، قابل للتنفيذ أو مناسب أو يتوافق مع الهدف أو الغرض أو المتطلبات الأخرى للعقد.

٥ - يوافق المتعاقد على أنه سيتم حل جميع النزاعات مع سلطة التعاقد المتعلقة بالعقد وديًا، وإذا لم يكن ذلك ممكنًا، من خلال التحكيم وفقًا للقوانين اللبنانية.

Article 2: Nature of the DBOT Agreement

Use of the installations and equipment will always be optional for the public and subject to the needs of the general port services.

The Contractor will have no grounds for complaint should an invitation to Tenders be launched by the Contracting Authority for the creation and operation of additional silos or any other installations.

The DBOT Agreement is granted on a strictly personal basis Intuitu personae.

The Contractor is personally liable for the performance of all the obligations stipulated in these specifications and may not therefor subcontract all or part of these DBOT Agreement or dispose of his company other than in the conditions stipulated in Article 13 bellow.

The Contractor is required to ensure continuity of the public service licensed, equal treatment of the users of the public service licensed and to adapt the installations, equipment and tools under the DBOT Agreement to the needs of said public service.

The Tenderers shall have to bid for an annual rental fee (as a % of the storage and handling operations fees set by the Port Authority).

The contract will be awarded to the highest acceptable bid.

The minimum bid value shall not be less than 15%.

المادة الثانية: طبيعة العقد

إستخدام المنشآت والمعدات سيكون دائمًا إختياريًا للسلطة العامة ويخضع لإحتياجات الخدمات العامة في المرفأ.

لن يكون للمتعاقد أي أساس للتذمر في حال تم إطلاق دعوة للمناقصات من قبل سلطة التعاقد لإنشاء وتشغيل صوامع إضافية أو أي منشآت أخرى.

يتم منح العقد على أساس شخصي بحت Intuitu personae.

المتعاقد مسؤول شخصيًا عن تنفيذ جميع الإلتزامات المنصوص عليها في هذه المواصفات، ولا يمكنه، لذلك، التعاقد من الباطن على كامل أو جزء من هذا العقد أو التصرف في شركته إلا وفقًا للشروط المنصوص عليها في المادة ١٣ أدناه.
المتعاقد ملزم بضمان إستمرارية الخدمة العامة المرخصة، مع معاملة متساوية لمستخدمي الخدمة العامة المرخصة، وتكييف المنشآت والمعدات والأدوات في إطار العقد لتلبية إحتياجات تلك الخدمة العامة.

يجب على العارضين تقديم عروضهم مقابل رسوم إيجار سنوية (كنسبة مئوية من رسوم التخزين والمناولة التي تحددها سلطة المرفأ).

سيتم منح العقد لأعلى عرض مقبول.

يجب ألا تقل قيمة العرض عن ١٥%.

- PART TWO

Performance of the work and maintenance of the facilities and installations of the DBOT Agreement

- الجزء الثاني

تنفيذ العمل وصيانة المنشآت والمرافق الخاصة بالعقد

Article 3: Execution plans

Before the beginning of any work, the Contractor is required to submit to the Contracting Authority all designs, stability calculation notes of miscellaneous facilities, of civil and MEP works and buildings, topographic surveys, bathymetric survey covering the ship berth, geotechnical investigations to be undertaken for all foundations and piling system and any other necessary data for design, plans for construction, execution, acquisition or modification of all the works, the silo detailed plans and characteristics, buildings and facilities, and all the equipment to be installed.

These plans will comprise all drawings, sketches and explanatory documents needed or which can be requested by the Engineer for complete determination of the constructions and installations to be build, along with the equipment operating conditions.

The Contractor cannot begin the execution of any work, installation of any equipment or machinery (including the bins), supplying of any material… before getting the writing approval of the Contracting Authority on the related stability calculation notes, structural and MEP design reports, drawings and technical specifications, method statements for execution, according to the Contracting Authority requirements. All above mentioned documents are to be provided for the Contracting Authority in 3 copies (hard) and one soft copy.

In any way, the Contractor will be the only responsible regarding the stability and the right choice of any equipment, facility, and material… related to this Contract, even if he has obtained the approval of the Contracting Authority and/or the Engineer on the mentioned documents.

The Contracting Authority and/or the Engineer can request any document of any kind, and therefore can pause the launching of works or stop the works execution as much as it will be needed, without any right for the Contractor to submit any claim or objection of any nature.

If the Contracting Authority instructs that further construction Documents are necessary for caring out the relevant works, the Contractor shall, upon receiving the Contracting Authority instructions, prepare such construction Documents within requested details and time for submission set by the Contracting Authority.

The Contracting Authority will be entitled to specify any modifications it feels to be suitable to ensure the correct use and conservation of the public domain works and, in general, the correct running of all the services.

Exercise this power of supervision and specification of modifications will be granted to the Contracting Authority. Approval or failure to approve by the Contracting Authority will neither imply its own liability nor relieve the Contractor of its liability with regard to users and third parties, the possible consequences of performance of the work or operation of the installations.

Errors, omissions, ambiguities, inconsistencies, inadequacies and other defects in the design and/or in the construction Documents shall be rectified both as to the Documents and, as the case may be, the relevant structures by the Contractor at its own cost notwithstanding that the same may have been approved by the Contracting Authority.

المادة الثالثة: خرائط التنفيذ

قبل بدء أي عمل، يُطلب من المتعاقد تقديم جميع التصاميم، ملاحظات حسابات الإستقرار للمرافق المتنوعة، الأعمال الإنشائية والكهربائية والميكانيكية (MEP) والمباني، المسوحات الطبوغرافية، المسح البحري الذي يغطي رصيف السفن، التحقيقات الجيوتقنية التي سيتم إجراؤها لجميع الأساسات ونظام الحفر، وأي بيانات أخرى ضرورية للتصميم، خطط البناء، التنفيذ، الإستحواذ أو تعديل جميع الأعمال، تفاصيل خطط الإهراءات وخصائصها، المباني والمرافق، وجميع المعدات التي سيتم تركيبها، إلى سلطة التعاقد.

يجب أن تتضمن هذه الخطط جميع الرسومات، والرسوم التوضيحية، والوثائق التفسيرية اللازمة أو التي يمكن طلبها من المهندس لتحديد جميع المنشآت والمرافق التي سيتم بناؤها، جنبًا إلى جنب مع شروط تشغيل المعدات.
لا يمكن للمتعاقد البدء في تنفيذ أي عمل، أو تركيب أي معدات أو آلات (بما في ذلك الصوامع)، أو توريد أي مواد، قبل الحصول على موافقة خطية من سلطة التعاقد على ملاحظات حساب الإستقرار ذات الصلة، تقارير التصميم الإنشائي والكهربائي والميكانيكي (MEP)، الرسومات والمواصفات الفنية وبيانات طريقة التنفيذ وفقًا لمتطلبات سلطة التعاقد. يجب تقديم جميع الوثائق المذكورة لسلطة التعاقد في ٣ نسخ (ورقية) ونسخة إلكترونية واحدة.
على أي حال، سيكون المتعاقد هو المسؤول الوحيد عن الإستقرار والإختيار الصحيح لأي معدات أو منشأة أو مادة تتعلق بهذا العقد، حتى إذا حصل على موافقة سلطة التعاقد و/أو المهندس على الوثائق المذكورة.
يمكن لسلطة التعاقد و/أو المهندس طلب أي وثيقة من أي نوع، وبالتالي يمكنهم وقف بدء الأعمال أو إيقاف تنفيذ الأعمال حسب الحاجة، دون أي حق للمتعاقد في تقديم أي مطالبة أو إعتراض من أي نوع.

إذا وجه سلطة التعاقد بأن هناك مستندات بناء إضافية ضرورية لتنفيذ الأعمال المعنية، يجب على المتعاقد، عند تلقي تعليمات سلطة التعاقد، إعداد هذه المستندات ضمن التفاصيل والوقت المطلوبين لتقديمها وفقًا لتوجيهات سلطة التعاقد.
سيكون لسلطة التعاقد الحق في تحديد أي تعديلات يراها مناسبة لضمان الإستخدام السليم وصيانة الأعمال العامة، وبشكل عام، تشغيل جميع الخدمات بشكل صحيح.

سيتم منح سلطة التعاقد سلطة ممارسة هذا الإشراف وتحديد التعديلات. لن يعني قبول أو رفض سلطة التعاقد للموافقة تحميله أي مسؤولية ولن يعفي المتعاقد من مسؤوليته تجاه المستخدمين والأطراف الثالثة، أو العواقب المحتملة لأداء العمل أو تشغيل المنشآت.

يجب على المتعاقد تصحيح الأخطاء، الغيابات، الغموض، التناقضات، النواقص والعيوب الأخرى في التصميم و/أو مستندات البناء، سواء في المستندات أو، حسب الحالة، الهياكل ذات الصلة، على نفقته الخاصة على الرغم من أنه قد تم الموافقة عليها من قبل سلطة التعاقد.

Article 4: Performance of work

All the work will be performed in accordance with the approved plans, using materials of good quality employed in compliance with the rules of professional good practice and international standards.

In addition to obtaining the Contracting Authority’s consent regarding the subcontracting of any part of the Works, the Contractor shall, within 14 days of signing the Contract, and before commencing the Works on site, or any parts of the Works he intends to subcontract for which approval of the Contracting Authority is required, and for each subcontract, provide:

- The name, address and telephone and fax numbers of the subcontractor.

- The nature and scope of the Works to be subcontracted.

- Information of the Subcontractor’s experience of similar works and details of the Subcontractor’s site supervision, sources of labor, and Equipment, and financial capabilities, in sufficient detail to enable the Contracting Authority to determine of the subcontractor is able to undertake and complete the subcontract works within the time and to the standards required by the Contract.

- The approximate value of the subcontract works based on the Contact prices.

- The percentage of (4) above and the overall percentage of subcontracted works as a proportion of the Contract price.

- Confirmation that the subcontract includes terms and conditions and all obligations and responsibilities contained in the Contract, in so far as these apply to the subcontract.

- Details of all subcontracts (Equipment, labor, names…) to be mentioned in monthly reports, in addition to any other information that might be asked by the Contracting Authority regarding subcontracting.

Before beginning any work on site, the Contractor shall submit for the Contracting Authority’s approval three (3) hard copies and one soft copy of the program of Works (works schedule).

This program shall indicate all the activities of the Contract in their correct sequence and order with a minimum detail at the Bill of Quantities level, and shall be computerized and prepared using a professional management software package approved by the Engineer.

A revised program, shall be submitted for approval by the Contracting Authority within 14 days of a request, at any time and for any reason that he sees it.

The Contractor shall promptly report in writing to the Contracting Authority the occurrence of any event or condition that might delay or prevent completion of the works in accordance with the program and indicate steps being taken to meet the situation.

The drawings, lines and levels in the original Tendering Documents shall be treated as baseline information and the Contractor shall, without any additional cost and time be responsible for verifications, surveys, preparation of revised drawings and obtaining approval of the Contracting Authority before the start of the works.

The Contractor shall also be responsible for the verification of the survey control points, benchmarks and such other information provided to him, and any errors or delays in the works caused by the Contractor’s failure to verify the accuracy for such data will not be an acceptable cause for extension of time and additional cost to the Contracting Authority.

The Contractor shall protect the Works during inclement weather and shall use all reasonable means to proceed with performance of the Works during such weather and to prevent or minimize delays resulting there from.

The Contractor shall use reasonable efforts to keep the Site clear of unnecessary obstruction so as to avoid danger to persons and provide fencing, lighting, guarding and watching of the works until completion and taking over.

The Contractor shall comply with specific safety, health and environmental requirements as requested by the Contracting Authority.

Following the contract signature, which date is considered to be the starting date of works, the Contractor will have a maximum period of 24 months counted from the mentioned date, during which he must execute and deliver to the Contracting Authority all requested civil works, supply the silo equipment and install it, ready for operation.

In this respect, a temporary handing over certificate will be issued by the Contracting Authority at the delivery of these works, followed by a final handing over after one year, in case all works are executed in conformity with specifications.

No annual rental fee shall be paid by the Contractor at the contract signature, neither during the works execution period.

The Contractor will have a period of 25 years to operate the facility, during which he should pay the Tender annual rental fee according to payment modality as per Article 16.

The occupancy for the land area will be free of charge during all the DBOT Agreement period.

المادة الرابعة: تنفيذ العمل

سيتم تنفيذ جميع الأعمال وفقًا للخطط المعتمدة، بإستخدام مواد عالية الجودة وبما يتوافق مع قواعد الممارسة المهنية الجيدة والمعايير الدولية.

بالإضافة إلى الحصول على موافقة سلطة التعاقد فيما يتعلق بالتعاقد من الباطن لأي جزء من الأعمال، يجب على المتعاقد، في غضون ١٤ يومًا من توقيع العقد، وقبل بدء الأعمال في الموقع، أو أي جزء من الأعمال التي يعتزم التعاقد من الباطن عليها والتي تتطلب موافقة سلطة التعاقد، ولأي عقد من الباطن، تقديم ما يلي:

١ - اسم المقاول من الباطن، عنوانه، وأرقام الهاتف والفاكس.

٢ - طبيعة ومدى الأعمال التي سيتم التعاقد من الباطن عليها.

٣ - معلومات عن خبرة المقاول من الباطن في الأعمال المماثلة وتفاصيل إشرافه على الموقع، مصادر العمل، والمعدات، وقدراته المالية، بالتفصيل الكافي لتمكين سلطة التعاقد من تحديد ما إذا كان المقاول من الباطن قادرًا على تنفيذ الأعمال المكلف بها وإكمالها ضمن الوقت والمعايير المطلوبة بموجب العقد.

٤ - القيمة التقريبية لأعمال المقاول من الباطن إستنادًا إلى أسعار العقد.

٥ - النسبة المئوية لـ (٤) أعلاه والنسبة الإجمالية للأعمال المتعاقد عليها من الباطن كنسبة من قيمة العقد.

٦ - تأكيد أن العقد الثانوي يتضمن الشروط والأحكام وكافة الإلتزامات والمسؤوليات الواردة في العقد، بقدر ما تنطبق على العقد الثانوي.

٧ - تفاصيل جميع العقود الثانوية (المعدات، العمل، الأسماء...) التي يجب ذكرها في التقارير الشهرية، بالإضافة إلى أي معلومات أخرى قد يطلبها سلطة التعاقد بشأن التعاقد من الباطن.

قبل بدء أي عمل في الموقع، يجب على المرخص له تقديم ثلاثة (٣) نسخ ورقية ونسخة إلكترونية واحدة من برنامج العمل (جدول الأعمال) لموافقة سلطة التعاقد.

يجب أن يوضح هذا البرنامج جميع الأنشطة المتعلقة بالعقد بترتيبها الصحيح، مع الحد الأدنى من التفاصيل على مستوى الكميات المعتمدة، ويجب أن يكون مُعدًا بإستخدام برنامج إدارة محترف معتمد من قبل المهندس.

يجب تقديم برنامج محدث لموافقة سلطة التعاقد في غضون ٧٤ يومًا من الطلب، في أي وقت ولأي سبب يراه سلطة التعاقد مناسبًا.

يجب على المتعاقد الإبلاغ عن أي حدث أو حالة قد تؤدي إلى تأخير أو منع إتمام الأعمال وفقًا للبرنامج المحدد، ويجب عليه تحديد الإجراءات المتخذة لمواجهة هذه الحالة.

يجب معاملة الرسومات والخطوط والمستويات في مستندات الإلتزام الأصلية كبيانات أساسية، ويجب على المتعاقد، دون أي تكلفة إضافية أو تأخير في الوقت، التحقق من دقة هذه البيانات، إجراء المسوحات، إعداد الرسومات المعدلة والحصول على موافقة سلطة التعاقد قبل بدء الأعمال.

يجب على المتعاقد أيضًا التحقق من نقاط المسح، والنقاط المرجعية وأي بيانات أخرى تم توفيرها له، وأي أخطاء أو تأخيرات في الأعمال ناتجة عن فشل المتعاقد في التحقق من دقة هذه البيانات لن تكون سببًا مقبولًا لتمديد الوقت أو تكلفة إضافية على سلطة التعاقد.

يجب على المتعاقد حماية الأعمال أثناء الظروف الجوية السيئة، ويجب عليه إستخدام جميع الوسائل المعقولة لمتابعة تنفيذ الأعمال خلال هذه الظروف الجوية ولمنع أو تقليل التأخيرات الناتجة عنها.

يجب على المتعاقد بذل جهد معقول للحفاظ على الموقع خاليًا من العوائق غير الضرورية لتجنب الخطر على الأشخاص، توفير الأسوار، الإضاءة، الحراسة ومراقبة الأعمال حتى إكتمالها وإستلامها.

يجب على المتعاقد الإمتثال لمتطلبات السلامة والصحة والبيئة الخاصة التي يطلبها سلطة التعاقد.

بعد توقيع العقد، الذي يُعتبر تاريخ بدء الأعمال، سيكون لدى المتعاقد فترة أقصاها ٢٤ شهرًا من التاريخ المذكور، يجب عليه خلالها تنفيذ وتسليم جميع الأعمال الإنشائية المطلوبة لسلطة التعاقد، وتوريد معدات الإهراءات وتركيبها جاهزة للعمل.

في هذا الصدد، سيتم إصدار محضرإستلام مؤقت من قبل سلطة التعاقد عند تسليم هذه الأعمال، يليه محضر إستلام نهائي بعد سنة، في حالة تنفيذ جميع الأعمال بما يتوافق مع المواصفات.

لن يتم دفع أي رسوم إيجار سنوية من قبل المتعاقد عند توقيع العقد، ولا خلال فترة تنفيذ الأعمال.
سيكون لدى المرخص له فترة ٢٥ عامًا لتشغيل المنشأة، خلال هذه الفترة يجب عليه دفع رسوم الإيجار السنوية للمزايدة وفقًا لطريقة الدفع المنصوص عليها في المادة ١٦.

سيكون اشغال المساحة الأرضية مجانيًا طوال فترة العقد.

Article 5: Maintenance of works

All works executed by the Contractor will be maintained in good condition by him, so that they are at all times suitable for their intended purpose.

The work made available to the Contractor (if any) will be maintained in good condition by the Contractor, so that they are at all times suitable for their intended purpose.

The Contractor will take the necessary steps to ensure that the installations and equipment as well as their immediate vicinity, are kept clean.

In the event of negligence on its part, and no reply is received to formal notification, the necessary work will be performed by the Contracting Authority as it sees fit.

The amount of the sums advanced by the Contracting Authority will be recovered through statements made enforceable by the Contracting Authority's representative, in the conditions laid down in Article 37.

المادة الخامسة: صيانة الأعمال

سيتم صيانة جميع الأعمال التي ينفذها المتعاقد بشكل جيد من قبله، بحيث تظل صالحة في جميع الأوقات لأغراضها المحددة.

سيتم صيانة الأعمال المتاحة للمتعاقد (إن وجدت) بشكل جيد من قبله، بحيث تظل صالحة في جميع الأوقات لأغراضها المحددة.

يتعين على المتعاقد اتخاذ الخطوات اللازمة لضمان أن المنشآت والمعدات وكذلك محيطها المباشر، تظل نظيفة.

في حالة الإهمال من جانبه، وعدم تلقي رد على الإشعار الرسمي، سيتم تنفيذ الأعمال اللازمة من قبل سلطة التعاقد كما يراه مناسبًا.

سيتم إسترداد المبالغ التي يقدمها سلطة التعاقد من خلال البيانات التي تصبح قابلة للتنفيذ من قبل ممثل سلطة التعاقد، وفقًا للشروط المنصوص عليها في المادة ٣٧.

Article 6: Construction and maintenance costs

The renovation, modification and renewal work on the installations, equipment and tools will be made available by the Contractor at his latter's expense.

They will be performed according to the procedures defined in a separate agreement.

All works will be at the expense of the Contractor and performed in the conditions laid down in Article 3 above.

All maintenance costs will be at the expense of the Contractor.

المادة السادسة: تكاليف البناء والصيانة

سيتم تنفيذ أعمال التجديد والتعديل والتحديث على المنشآت والمعدات والأدوات التي يوفرها المتعاقد على نفقته الخاصة.
سيتم تنفيذ هذه الأعمال وفقًا للإجراءات المحددة في إتفاقية منفصلة.

جميع الأعمال ستكون على حساب المتعاقد ويتم تنفيذها وفقًا للشروط المنصوص عليها في المادة ٣ أعلاه.
جميع تكاليف الصيانة ستكون على حساب المتعاقد.

Article 7: Performance times

Starting from the commencement date stated bellow, the Contractor will - within the time stipulated in the Letter of Tender, complete the initial development work on the installations and equipment.

Confirmed by a statement, the commencement date of the grain silo operation shall be the date of the handover of the installations made available by the Contractor mentioned in Article 1.

Prior to any operating activities, the initial development work on the installations and equipment has to be completed and confirmed.

A statement will be produced jointly by the Contracting Authority and the Contractor.

المادة السابعة: أوقات التنفيذ

ابتداءً من تاريخ البدء المحدد أدناه، يجب على المتعاقد ضمن المدة المحددة في كتاب العرض إتمام أعمال التطوير الأولية الخاصة بالمنشآت والمعدات. يُؤكد محضر تاريخ بدء تشغيل إهراءات الحبوب على أنه تاريخ تسليم المنشآت التي يوفرها المتعاقد والمذكورة في المادة الأولى. بل البدء بأي أنشطة تشغيلية، يجب إتمام وتأكيد أعمال التطوير الأولية الخاصة بالمنشآت والمعدات. سيتم إصدار محضر مشترك من قبل سلطة التعاقد والمتعاقد.

Article 8: Supervision of construction and maintenance

In the event of acquisition, prior technical inspections will be performed by the Contractor and the Contracting Authority.

The deeds of acquisition of equipment, installations and facilities may only be signed by the Contractor after approval by the Contracting Authority.

The initial design, development, execution, modification and maintenance work will be performed under the supervision of the Contracting Authority, through a nominated staff (the Engineer / Consultant).

المادة الثامنة: الإشراف على البناء والصيانة

في حال الشراء، يجب إجراء الفحوصات الفنية المبدئية من قبل المُنَفِّذ وصاحب المشروع.

لا يمكن توقيع عقود شراء المعدات، المنشآت، والمرافق إلا بعد موافقة صاحب المشروع.

سيتم تنفيذ التصميم الأولي، التطوير، التنفيذ وأعمال التعديل والصيانة تحت إشراف صاحب المشروع، من خلال فريق مختار (المهندس / الاستشاري).

Article 9: Additional installations and equipment

Whenever requested, the Contractor will be required to deploy additional installations and equipment as determined by the Contracting Authority, provided that this entails no fundamental change to the content of the DBOT Agreement.

المادة التاسعة: المنشآت والمعدات الإضافية

كلما طُلب ذلك، يجب على المُنَفِّذ نشر منشآت ومعدات إضافية كما يحددها صاحب المشروع، بشرط أن لا يؤدي ذلك إلى تغيير جوهري في محتوى العقد.

- PART THREE

Operation

- الجزء الثالث

التشغيل

Article 10: Docking and port activity

This DBOT Agreement gives the Contractor no right to intervene, either to place (pilotage and towing) the ships and boats on the docks or the berth fitted out by itself or to move these ships and boats, or to police traffic and use of the roads.

The Contractor, must all times, comply with the Port navigation requirements, schedules and plans, especially regarding ships navigation and berthing, and therefore make necessary adjustments to his own schedules and plans accordingly, so that the Port navigation won’t be affected in any way.

المادة العاشرة: الترصيف وحركة المرفأ

لا تمنح هذه الاتفاقية المتعاقد أي حق في التدخل، سواء لوضع (التوجيه والسحب) السفن والقوارب على الأرصفة أو الرصيف الذي قام بتركيبه بنفسه، أو لتحريك هذه السفن والقوارب، أو لمراقبة حركة المرور واستخدام الطرق.

يجب على المتعاقد، في جميع الأوقات، الامتثال لمتطلبات الملاحة في المرفأ والجداول الزمنية والخطط، خاصة فيما يتعلق بملاحة السفن وربطها، وبالتالي إجراء التعديلات اللازمة على جداول أعماله وخططه وفقًا لذلك، حتى لا تتأثر حركة الملاحة في المرفأ بأي شكل من الأشكال.

Article 11: Operation of licensed installations

The Contractor will be required to make the equipment and installations available to the public, in the conditions stipulated in the operating regulations.

Prior to any operating activities, the operating regulations have to be drawn up by the Contractor and approved by the Contracting Authority.

The Contractor performs technical and commercial operation of the installations, in particular the silo equipment.

In this respect:

- He is responsible for all the operations performed on the installations, in particular the silo equipment.

He is required to provide the appropriate personnel, equipment and technical resources. He is in charge of coordination with regard to the customers.

- He constructs all facilities, buildings, equipment, tools and as applicable the open necessary spaces

- He maintains during all the DBOT Agreement period open areas, facilities, levels of water bodies, marine infrastructures, buildings, executed civil works, equipment and tools as stipulated above

- He assumes responsibility as operator of the dock and its installations with respect to all legislative and regulatory provisions in force, in all areas and respects

المادة الحادية عشر: تشغيل المنشآت المرخصة

يُطلب من المتعاقد جعل المعدات والمنشآت متاحة للسلطة العامة، وفقًا للشروط المحددة في قوانين التشغيل.

قبل أي أنشطة تشغيلية، يجب على المتعاقد إعداد قوانين التشغيل والحصول على موافقة سلطة التعاقد عليها.

يقوم المتعاقد بتنفيذ العمليات الفنية والتجارية للمنشآت، وخاصة معدات الإهراءات.

في هذا الصدد:

أ - هو المسؤول عن جميع العمليات المنفذة على المنشآت، وخاصة معدات الإهراءات. ويُطلب منه توفير الموظفين والمعدات والموارد التقنية المناسبة. وهو المسؤول عن التنسيق مع العملاء.

ب - يقوم بإنشاء جميع المنشآت والمباني والمعدات والأدوات، وكذلك المساحات المفتوحة اللازمة حسب الاقتضاء.

ج - يُحافظ على المساحات المفتوحة والمنشآت ومستويات المسطحات المائية والبنية التحتية البحرية والمباني والأعمال الإنشائية المنفذة والمعدات والأدوات طوال مدة العقد كما هو محدد أعلاه.

د - يتحمل المسؤولية كتشغيل للرصيف ومنشآته وفقًا لجميع الأحكام التشريعية والتنظيمية السارية، في جميع المجالات والجوانب.

Article 12: Port regulations

The Contractor will be subject to port regulations. He will in particular abide by the provisions of the regulations concerning use of the hangars, open areas and land of the Port of Tripoli.

He will comply with any decisions made by the Contracting Authority, in the interests of public safety, and of port operations.

He will be required temporarily to move its equipment, whenever so asked by the port authorities, either for the purposes of the working of the port, or for repairs to be made to public works, or for any other reason given by the Contracting Authority.

المادة الثانية عشر: قوانين المرفأ

يخضع المتعاقد لقوانين المرفأ. وعليه أن يلتزم على وجه الخصوص بأحكام القوانين المتعلقة باستخدام المخازن والمساحات المفتوحة وأراضي مرفأ طرابلس.

سوف يلتزم بأي قرارات يتخذها سلطة التعاقد من أجل مصلحة السلامة العامة وتشغيل المرفأ.

وسيُطلب منه تحريك معداته مؤقتًا، كلما طلبت منه سلطات المرفأ ذلك، سواء لأغراض تشغيل المرفأ أو لإجراء إصلاحات على الأعمال العامة أو لأي سبب آخر يقدمه سلطة التعاقد.

Article 13: Assignment and Subletting

The Contractor shall not assign any part or the whole of the DBOT Agreement to another party.

The Contractor shall not sublet all or any part of the DBOT Agreement to others without approval by the Contracting Authority of the extent of the functions to be sublet according to article 30 of the Public Procurement Law.

The provisions of the above-mentioned paragraph shall not apply to Sub-Contractors for minor day-to-day services required for the efficient management and operation of the DBOT Agreement.

Prior to any activities, any subcontractor shall be identified in the list of authorized Subcontractors, which has to be drawn up by the Contractor and approved by the Contracting Authority.

The Contracting Authority will be entitled to demand any modifications in the list it feels suitable for the proper and efficient operation of the DBOT Agreement.

المادة الثالثة عشر: التعيين والتأجير من الباطن

لا يجوز للمتعاقد تحويل أي جزء أو الكل من العقد إلى طرف آخر.

لا يجوز للمتعاقد تأجير كل أو جزء من العقد للآخرين دون موافقة سلطة التعاقد على نطاق الوظائف التي سيتم تأجيرها وفقاً لأحكام المادة 30 من قانون الشراء العام.

لا تنطبق أحكام الفقرة المذكورة أعلاه على المقاولين الثانويين الذين يقدمون خدمات يومية بسيطة تُطلب لإدارة لتشغيل العقد بشكل فعال.

قبل البدء بأي أنشطة، يجب تحديد أي مقاول ثانوي في قائمة المقاولين الثانويين المعتمدين، التي يجب أن يعدها المتعاقد وتوافق عليها سلطة التعاقد.

يحق لسلطة التعاقد طلب أي تعديلات في القائمة التي يرى أنها مناسبة لضمان التشغيل السليم والفعال للتعاقد.

Article 14: Supervisor of operations

The installations and equipment of the DBOT Agreement will be operated under the direct supervision of the Contracting Authority.

The Contractor will comply with all Contracting Authority’s directives.

Exercise of this power of supervision granted to the Contracting Authority will however, remain optional and can in no case be considered binding upon it.

المادة الرابعة عشر: مشرف العمليات

سيتم تشغيل المنشآت والمعدات الخاصة بالعقد تحت إشراف سلطة التعاقد المباشر.

يجب على المتعاقد الامتثال لجميع توجيهات سلطة التعاقد.

ومع ذلك، فإن ممارسة هذا الحق في الإشراف الممنوح لسلطة التعاقد ستظل اختيارية ولا يمكن اعتبارها ملزمة له في أي حال من الأحوال.

- PART FOUR

Fees

- الجزء الرابع

الرسوم

Article 15: Duties

The duties charged for miscellaneous services provided by the port like potable water, electricity…, are those applicable and in use by the Contracting Authority.

The Contractor cannot object on any modification made to the charges by the Contracting Authority.

Payment of these duties shall be done according to Contracting Authority regulations.

المادة الخامسة عشر: الرسوم

الرسوم المفروضة على الخدمات المتنوعة المقدمة من قبل المرفأ مثل المياه الصالحة للشرب، الكهرباء... هي الرسوم المعمول بها والتي يحددها سلطة التعاقد.

لا يحق للمتعاقد الاعتراض على أي تعديل يتم إجراؤه على الرسوم من قبل سلطة التعاقد.

يتم دفع هذه الرسوم وفقًا لقوانين سلطة التعاقد.

Article 16: Rights and obligations

- The Tender Annual Rental Fee for the silo installation and operation shall be a percentage of the storage and handling (loading and unloading) fees set by the Port Authority.

- The Contractor shall pay to the Port Authority all miscellaneous services provided by the Port as electricity, water, permits… in conformity with applicable charges.

- The Port Authority should be responsible to collect the whole fees amount as per set tariffs related to all the listed silo operations as per the Port regulations (Storage, handling…), whereas the Port Authority agrees that at all times during the DBOT Agreement period, the Port Authority shall retain only the annual rental fee percentage as per the agreement and pay to the Contractor the remaining amount without any deduction for the entire works executed per trimester within a period of 15 days of the beginning the following trimester

- Occupancy fees of any water body within the port basins, berthing along quay walls, channel usage by ships, and all other maneuvering actions… shall be paid directly to the Port Authority. For the avoidance of doubt, the Contractor shall not request any percentage of the above-mentioned fees.

- The Contractor shall respect at any time the Contracting Authority in law fees, and any related modification as per the demand and the Contracting Authority decision.

المادة السادسة عشر: الحقوق والواجبات

- رسوم الإيجار السنوية الخاصة بتركيب وتشغيل إهراءات الحبوب ستكون نسبة مئوية من رسوم التخزين والمناولة (التحميل والتفريغ) التي تحددها إدارة المرفأ.

- يجب على المتعاقد دفع جميع الرسوم المتعلقة بالخدمات المتنوعة التي يقدمها المرفأ مثل الكهرباء، المياه، التصاريح… وفقًا للرسوم المعمول بها.

- يجب على إدارة المرفأ أن تكون مسؤولة عن جمع كامل قيمة الرسوم وفقًا للتعريفات المعتمدة لجميع العمليات المتعلقة بالإهراءات بحسب قوانين المرفأ (التخزين، المناولة…) حيث أن إدارة المرفأ توافق على أنه طوال فترة العقد، ستحتفظ إدارة المرفأ فقط بنسبة الإيجار السنوي وفقًا للاتفاق، وتدفع للمتعاقد المبلغ المتبقي دون أي خصم عن الأعمال المنفذة خلال كل ربع سنة، وذلك في غضون ١٥ يومًا من بداية الربع التالي.

- يجب دفع رسوم إشغال أي مسطح مائي داخل أحواض المرفأ، الرسوم على حوائط الأرصغة، استخدام القنوات من قبل السفن وجميع عمليات المناورة الأخرى مباشرة إلى إدارة المرفأ. ولإزالة أي لبس، لا يحق للمتعاقد المطالبة بأي نسبة من الرسوم المذكورة أعلاه.

- يجب على المتعاقد احترام دائمًا رسوم سلطة التعاقد القانونية وأي تعديلات ذات صلة حسب الطلب وقرار سلطة التعاقد.

Article 17: Collect of charges

The Contracting Authority is responsible to collect all charges resulting from operating the silo, according to the way that he may find suitable and adequate for his cash flow.

Nevertheless, the Contracting Authority is not responsible for any financial disturbance regarding this cash flow.

المادة السابعة عشرة: جمع الرسوم

تكون سلطة التعاقد مسؤولًة عن جمع جميع الرسوم الناتجة عن تشغيل الإهراءات، وفقًا للطريقة التي تراها مناسبة وملائمة لتدفقه النقدي

ومع ذلك، لا تكون سلطة التعاقد مسؤولًة عن أي اضطراب مالي يتعلق بتدفقه النقدي.

Article 18: Duty collection

The sums collected by the Contracting Authority are recorded in a book with stubs, with details of all sums received entered on the stub and on the detached receipt.

This book will have to be presented to the Contractor at the time of each payment made to the Contractor.

المادة الثامنة عشر: جمع الرسوم

تُسجل المبالغ التي تجمعها سلطة التعاقد في دفتر يحتوي على قسائم، حيث يتم تدوين تفاصيل جميع المبالغ المستلمة في القسيمة وفي الإيصال المنفصل.

يجب تقديم هذا الدفتر إلى المتعاقد في وقت كل دفعة تُدفع له.

Article 19: Publicity of prices

The applicable prices will be made known to the public by means of clearly displayed posters, as close as possible to the installations and equipment, and at locations specified by the Contracting Authority.

The Contracting Authority will be responsible for the conversations of these posters and will replace them whenever necessary.

المادة التاسعة عشر: الإعلان عن الأسعار

سيتم الإعلان عن الأسعار السارية للعامة من خلال لوحات واضحة المعالم، بالقرب من المنشآت والمعدات، وفي المواقع التي يحددها سلطة التعاقد.

سيكون سلطة التعاقد مسؤولاً عن الحفاظ على هذه اللوحات واستبدالها عند الحاجة.

Article 20: Book of complaints

A book will be kept in the Contractor's office for receiving complaints either against the Contractor, or against its personnel.

The results of the Contracting Authority examination of each complaint will also be entered in the book.

This book will be initialed by the Contracting Authority. It will be presented whenever requested by the public.

As soon as a complaint has been entered in it, the Contractor must notify the Contracting Authority.

المادة العشرون: سجل الشكاوى

يجب أن يتم الاحتفاظ بسجل في مكتب المتعاقد لاستقبال الشكاوى ضد المتعاقد أو ضد موظفيه.

يتم أيضًا إدخال نتائج فحص سلطة التعاقد لكل شكوى في السجل.

سيتم توقيع هذا السجل من قبل سلطة التعاقد، ويجب أن يُعرض عند الطلب من قبل العامة.

بمجرد تسجيل شكوى فيه، يجب على المتعاقد إبلاغ سلطة التعاقد بذلك.

Article 21: Responsibility – Insurance

The Contractor is responsible for any damage resulting from operation of his installations, in particular the silo equipment, whether caused by itself or by one of its Contractors, equipment, personnel… and whether suffered by itself, by the port, by a third party or by a user.

He agrees to repair any damage of any kind.

He agrees to repair the damage caused by third parties within the limits of the DBOT Agreement and is responsible for any action taken against them.

He takes out all the insurance policies corresponding to its responsibilities under application of this Agreement, in particular for the development and maintenance work as stated in the copies of insurance policies appended to this Agreement.

The scope and amount of insurances shall be revised annually to reflect any change in the risks, the value of additions or improvements to the facilities and the effects of inflation.

The Contractor shall, prior to any operating activities, submit copies of the insurance policies effected by him to the Contracting Authority for approval and he shall provide evidence, whenever requested by the Contracting Authority, that the current insurance premiums have been paid.

المادة الواحدة والعشرون: المسؤولية – التأمين

يكون المتعاقد مسؤولاً عن أي ضرر ناتج عن تشغيل منشآته، وبشكل خاص معدات الإهراءات، سواء كان الضرر ناتجًا عن نفسه أو عن أحد مقاولي الباطن أو معداته أو موظفيه، سواء كان الضرر قد لحق به هو أو بالمرفأ أو بطرف ثالث أو بالمستخدم.

يوافق المتعاقد على إصلاح أي ضرر من أي نوع.

يوافق المتعاقد على إصلاح الأضرار التي تسبب بها أطراف ثالثة ضمن حدود العقد وهو مسؤول عن أي إجراء يتم اتخاذه ضدهم.

يأخذ المتعاقد جميع وثائق التأمين التي تتوافق مع مسؤولياته بموجب هذا الاتفاق، وبشكل خاص بالنسبة للأعمال الإنشائية وأعمال الصيانة كما هو موضح في نسخ وثائق التأمين المرفقة بهذا الاتفاق

يجب مراجعة نطاق وقيمة التأمينات سنويًا لتعكس أي تغييرات في المخاطر أو قيمة الإضافات أو التحسينات في المنشآت وأثر التضخم.

يجب على المتعاقد، قبل أي أنشطة تشغيلية، تقديم نسخ من وثائق التأمين التي قام بإبرامها إلى سلطة التعاقد للموافقة عليها ويجب عليه تقديم دليل، كلما طلب سلطة التعاقد ذلك، على أن الأقساط التأمينية الحالية قد تم دفعها.

- PART FIVE

Financial conditions

- الجزء الخامس

شروط مالية

Article 22: Annual Accounts

Before 31 March of each year, the Contractor will provide the Contracting Authority with the detailed accounts, based on his books and containing the following, for the previous year:

- The gross income of all kinds derived from operation of the DBOT Agreement

- The maintenance and operating costs, including financial costs (reimbursement of Loans and financial costs inherent in the DBOT Agreement)

Whenever requested, the Contractor will be required to present accounts Documents books, correspondences and other Documents that the Contracting Authority considers necessary for supervision of operation.

مادة الثانية والعشرون: الحسابات السنوية

قبل ٣١ آذار من كل عام، يجب على المتعاقد تقديم الحسابات التفصيلية لسلطة التعاقد، استنادًا إلى دفاتره وتشمل ما يلي للسنة السابقة:

١ - الدخل الإجمالي من جميع الأنواع الناتج عن تشغيل العقد.

٢ - تكاليف الصيانة والتشغيل، بما في ذلك التكاليف المالية (سداد القروض والتكاليف المالية المتعلقة بالعقد).

عند الطلب، يجب على المتعاقد تقديم المستندات المحاسبية والكتب والمراسلات وغيرها من المستندات التي يعتبرها سلطة التعاقد ضرورية للإشراف على التشغيل.

Article 23: First payment to the Contracting Authority

The first payment to the Contracting Authority, after the handover of the silo works by the Contracting Authority and putting the silo in operation, shall be per trimester, within 15 days of the beginning of the following trimester, knowing that the financial year begins in the 1st of January.

المادة الثالثة والعشرون : الدفعة الأولى لصالح سلطة التعاقد

تكون الدفعة الأولى لسلطة التعاقد، بعد تسليم أعمال الإهراءات من قبل سلطة التعاقد وتشغيل الإهراءات، على أساس فصلي، ويتم دفعها خلال ١٥ يومًا من بداية الفصل التالي، مع العلم أن السنة المالية تبدأ في ١ يناير.

Article 24: profit sharing

No profit sharing to be paid for the Contracting Authority.

المادة الرابعة والعشرون: توزيع الأرباح

لا يتم دفع أي مشاركة في الأرباح لسلطة التعاقد.

- PART SIX

DBOT Agreement period – Buy back – Expiry – renewal

- الجزء السادس

مدة العقد - لشراء - الانتهاء - التجديد

Article 25: DBOT Agreement period

The DBOT Agreement period shall be twenty-five (25) years from the commencement date, which is the temporary handing over and receipt date by the Contracting Authority, of all executed works and installations, unless the DBOT Agreement is terminated under the provisions of this DBOT Agreement.

المادة الخامسة والعشرون: فترة العقد

تكون فترة العقد خمسة وعشرون (٢٥) سنة اعتبارًا من تاريخ البدء، وهو تاريخ تسليم الأعمال والمنشآت المنفذة مؤقتًا واستلامها من قبل سلطة التعاقد، ما لم يتم إنهاء العقد بموجب أحكام إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية هذه.

Article 26: Effect of expiry of the DBOT Agreement

On expiry of the time set in the previous article and by the sole fact of this expiry, the Contracting Authority will assume all the rights of the Contractor and receive all revenue from the DBOT Agreement.

It will immediately take possession of the assets of the DBOT Agreement and will not assume responsibility for the DBOT Agreement's debts duly contracted by the Contractor, without compensation for whatsoever reason.

A closing balance of the DBOT Agreement accounts will be drawn up within a maximum of six (6) months from expiry of the DBOT Agreement.

المادة السادسة والعشرون: أثر انتهاء فترة العقد

عند انتهاء الفترة المحددة في المادة السابقة ومن خلال انتهاء هذه الفترة فقط، سيتولى سلطة التعاقد جميع حقوق المُستفيد من العقد ويستلم جميع الإيرادات الناتجة عن العقد.

ستتولى سلطة التعاقد فورًا حيازة أصول العقد ولن تتحمل مسؤولية ديون العقد التي تم التعاقد عليها من قبل المتعاقد، دون تعويض لأي سبب كان.

سيتم إعداد رصيد إغلاق لحسابات العقد في موعد أقصاه ستة (٦) أشهر من تاريخ انتهاء العقد.

Article 27: Renewal of DBOT Agreement

The Contractor shall not have any exclusive right to renew the DBOT Agreement on expiry of the DBOT Agreement period.

The Contracting Authority shall not be under obligation to renew the DBOT Agreement or let a DBOT Agreement to any other party.

In the event that the Contracting Authority decides to let a further DBOT Agreement expiry of the DBOT Agreement period and notwithstanding provisions under Article 26, he shall give the Contractor a right of first refusal to renew the DBOT Agreement, providing that the terms offered by the Contractor are, in the opinion of the Contracting Authority, no less favorable than those offered by any other potential The Contractor.

Any Agreement for renewal as aforesaid shall be made at least 1 (one) year before the expiry of the DBOT Agreement period.

المادة السابعة والعشرون: تجديد العقد

لا يحق للمتعاقد الحصول على حق حصري لتجديد إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية عند انتهاء فترة العقد.

لن تكون سلطة التعاقد ملزمًة بتجديد العقد أو منح تعاقد لأي طرف آخر.

في حال قرر سلطة التعاقد منح تعاقد آخر بعد انتهاء فترة العقد، وبغض النظر عن الأحكام الواردة في المادة ٢٦، فإنه يجب أن يمنح المتعاقد حق الأولوية في تجديد العقد، بشرط أن تكون الشروط المقدمة من قبل المتعاقد في رأي سلطة التعاقد، لا تقل فائدة عن تلك المقدمة من قبل أي صاحب تعاقد آخر محتمل.

يجب إبرام أي اتفاقية للتجديد وفقًا لذلك قبل سنة واحدة على الأقل من انتهاء فترة العقد.

Article 28: Earlier termination – Forfeiture

Termination of the DBOT Agreement shall be in accordance with the Public Procurement Law in Lebanon, N244 dated July 29, 2021, Article 33.

المادة الثامنة والعشرون: إنهاء مبكر – مصادرة

يجب أن يتم إنهاء إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية وفقًا لقانون الشراء العام رقم 244/2021، تاريخ 19/7/2021، وتعديلاته.

Article 29: Partial closure of the installations

Should the Contracting Authority at any time decide - after hearing the Contractor- that in the public interest, it is necessary to close part of the installations, either temporarily or permanently, the Contractor when requested will evacuate the premises and restore them to their original condition.

Should it fail to meet this obligation within the time allotted, the necessary work will be carried out in its place.

If this concerns installations, the closure of which would lead to the cessation of all or part of the services provided by the Contractor, this closure would be announced in the manner utilized for this DBOT Agreement, unless resulting from work declared to be in the public interest in a law or administrative order.

If application of this article leads to a prejudice for the Contractor, it would be entitled to compensation which, if an amicable arrangement cannot be reached, would be set by the competent Lebanese court.

المادة التاسعة والعشرون: الإغلاق الجزئي للمنشآت

في حال قرر سلطة التعاقد في أي وقت - بعد سماع رأي المتعاقد- أنه من الضروري إغلاق جزء من المنشآت، سواء بشكل مؤقت أو دائم، فإن المتعاقد سيتعين عليه عند الطلب إخلاء المكان وإعادته إلى حالته الأصلية.

إذا فشل في الوفاء بهذا الالتزام خلال المدة المحددة، سيتم تنفيذ الأعمال اللازمة بدلاً منه.

إذا كان هذا يتعلق بمنشآت يؤدي إغلاقها إلى توقف جميع أو جزء من الخدمات التي يقدمها المتعاقد، فيجب الإعلان عن هذا الإغلاق بالطريقة المعتمدة لهذا العقد، ما لم يكن نتيجة لأعمال منصوص عليها في قانون أو أمر إداري على أنها من المصلحة العامة.

إذا أدى تطبيق هذه المادة إلى ضرر للمتعاقد، فإنه سيكون له الحق في تعويض يتم تحديده من قبل المحكمة اللبنانية المختصة إذا لم يتم التوصل إلى تسوية ودية.

Article 30: Force majeure

Force majeure shall mean any event or circumstance or combination of events and circumstances which are wholly outside the control of the parties, and which gives rise to force majeure as recognized by Lebanese law, but not limited to:

- war and other hostilities (whether war be declared or not), invasion, other act of foreign enemies in Lebanon.

- civil war, rebellion, insurrection, riot, commotion and civil disorder in Lebanon.

- ionizing radiation or contamination from nuclear fuel or waste or radioactive, toxic or explosive effects of any nuclear explosive device in Lebanon.

- pressure waves caused by aircraft or other aerial devices travelling at sonic or supersonic speed in Lebanon.

- riot, commotion on civil disorder unless solely restricted to employees of the Contractor on his sub-Contractor and arising from the conduct of the works.

- Earthquake and any operation of the forces of nature against which an experienced Contractor could not reasonably have been expected to take precautions.

Neither the Contracting Authority nor the Contractor shall be considered in default or in contractual breach to the extent that performance of its obligations is prevented by force majeure which arises after the date hereof.

If, as a result of any happening of force majeure, the Contractor is prevented from performing its obligations hereunder, the parties shall use their best endeavors to modify the same in the most practical manner, having regard to the circumstances in order to achieve an end result which is the same or as closely similar to that envisaged by this DBOT Agreement as may be acceptable to the Contracting Authority and the Contractor.

Any costs or losses incurred by either party in consequence thereof shall be borne by the party which suffers such costs or losses.

In the event that operation of the DBOT Agreement is suspended or is seriously modified or curtailed for a continuous period of one year due to the effects of force majeure, either party may, by giving one month's written notice to the other party, terminate this DBOT Agreement.

المادة الثلاثون: القوة القاهرة

تعني القوة القاهرة أي حدث أو ظرف أو مجموعة من الأحداث والظروف التي تكون خارجة تمامًا عن سيطرة الأطراف، والتي تترتب عليها القوة القاهرة كما هو معترف بها بموجب القانون اللبناني، ولكن ليس على سبيل الحصر:

- الحرب وأعمال العدوان الأخرى (سواء تم إعلان الحرب أم لا)، الغزو، أو أي عمل من أعمال الأعداء الأجانب في لبنان.

- الحرب الأهلية، التمرد، الثورة، الشغب، الاضطرابات المدنية والفوضى في لبنان.

- الإشعاع المؤين أو التلوث الناتج عن الوقود النووي أو النفايات أو الآثار السامة أو المتفجرة لأي جهاز نووي انفجاري في لبنان.

- موجات الضغط الناتجة عن الطائرات أو الأجهزة الجوية الأخرى التي تسير بسرعة الصوت أو أسرع من الصوت في لبنان.

- الشغب أو الفوضى أو الاضطراب المدني إلا إذا كان مقتصرًا فقط على موظفي المتعاقد أو مقاولي الباطن التابعين له ويحدث بسبب تصرفات أعماله.

- الزلازل وأي عملية من قوى الطبيعة التي لا يمكن للمتعاقد الخبير اتخاذ احتياطات معقولة ضدها.

لن يُعتبر المتعاقد أو سلطة التعاقد في حالة تقصير أو خرق تعاقدي إلى الحد الذي يمنع فيه أداء التزاماته بسبب القوة القاهرة التي تحدث بعد تاريخ توقيع هذا العقد.

إذا حالت أي من حالات القوة القاهرة دون قدرة المتعاقد على الوفاء بالتزاماته بموجب هذا العقد، يجب على الطرفين بذل أفضل جهودهما لتعديل نفس الالتزامات بأكثر الطرق العملية، مع مراعاة الظروف، من أجل الوصول إلى نتيجة مماثلة قدر الإمكان لتلك التي تم تصورها في هذا العقد بما يتوافق مع موافقة سلطة التعاقد والمتعاقد.

أي تكاليف أو خسائر يتكبدها أي من الطرفين نتيجة لذلك ستتحملها الجهة التي تتحمل هذه التكاليف أو الخسائر.

في حال تم تعليق عملية العقد أو تعديلها بشكل كبير أو تقليصها لمدة مستمرة تزيد عن عام بسبب تأثيرات القوة القاهرة، يمكن لأي من الطرفين، من خلال إعطاء إشعار خطي مدته شهر واحد للطرف الآخر، إنهاء إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية هذا.

- PART SEVEN

Miscellaneous clauses

- الجزء السابع

مواد مختلفة

Article 31: Warranties

The Contractor warrants that he is a legal entity, properly established under Lebanese law and that he has the legal powers to enter into the DBOT Agreement and that he has the financial and other resources and the expertise to perform faithfully and diligently his obligations under this DBOT Agreement.

المادة الواحدة والثلاثون: الضمانات

يضمن المتعاقد أنه كيان قانوني تم تأسيسه بشكل صحيح بموجب القانون اللبناني وأنه يمتلك الصلاحيات القانونية للدخول في إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية وأنه يمتلك الموارد المالية والموارد الأخرى والخبرة اللازمة لأداء التزاماته بموجب هذه إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية هذه بأمانة واجتهاد.

Article 32: Language and Law

This DBOT Agreement shall be construed in accordance with Lebanese law.

This DBOT Agreement has been drawn-up in Arabic and English. In the event of any dispute as to the meaning between the Arabic and English texts, the Arabic version shall govern. However, since the DBOT Agreement between the DBOT Agreement authority and the Contractor were carried out in Arabic and English as per the Clause 4 of the Public Procurement Law, the English text should be taken into consideration while trying to interpret the meaning of the Arabic clauses in the DBOT Agreement.

المادة الثانية والثلاثون: اللغة والقانون

تُفسر إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية هذه وفقًا للقانون اللبناني.

تم إعداد إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية هذه باللغتين العربية والإنجليزية.

في حال وجود أي نزاع بشأن المعنى بين النصين العربي والإنجليزي، فإن النسخة العربية هي التي تحكم. ومع ذلك، بما أن إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية بين سلطة العقد والمتعاقد تم إتمامها باللغتين العربية والإنجليزية وفقاً لأحكام المادة 4 من قانون الشراء العام، يجب أخذ النص الإنجليزي في الاعتبار عند محاولة تفسير معاني البنود العربية في إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية.

Article 33: Matters affecting execution of the works

The Contractor shall be deemed to have obtained all necessary information as to risks, contingencies and all other circumstances which may affect the carrying out of the works which the Contractor is required to perform hereunder, and on the project as a whole. The Contractor shall be deemed to have satisfied itself as to the correctness and sufficiency of its business plan, estimated construction costs and construction period. The Contractor's revenues during the DBOT Agreement period shall cover the cost of all its obligations under this DBOT Agreement and all things necessary for the proper completion of the operation of the installations, in particular the silo equipment, the remedying of all defects and the maintenance and insurance of the operation of the installations, in particular the silo equipment, throughout the DBOT Agreement period.

المادة الثالثة والثلاثون: الأمور التي تؤثر على تنفيذ الأعمال

يُعتبر المتعاقد أنه قد حصل على كافة المعلومات اللازمة حول المخاطر، الاحتمالات وجميع الظروف الأخرى التي قد تؤثر على تنفيذ الأعمال التي يُطلب من المتعاقد تنفيذها بموجب هذه الاتفاقية، وكذلك على المشروع ككل. يُعتبر المتعاقد أنه قد تأكد من صحة وكفاية خطته التجارية، التكاليف التقديرية للبناء وفترة البناء. يجب أن تغطي إيرادات المتعاقد خلال فترة العقد تكاليف جميع التزاماته بموجب هذه الاتفاقية، وكل ما هو ضروري لإتمام تشغيل المنشآت بشكل صحيح، لا سيما معدات الإهراءات، إصلاح جميع العيوب، صيانة وتأمين تشغيل المنشآت، لا سيما معدات الإهراءات، طوال فترة العقد.

Article 34: Election of domicile

The Contractor will elect domicile in Tripoli. He must have offices close to the DBOT Agreement installations and, if so requested, will employ an agent who will live in the building assigned to said offices. This agent will be authorized to receive all administrative notifications on behalf of the Contractor.

المادة الرابعة والثلاثون: اختيار محل الإقامة

يجب على المتعاقد اختيار محل إقامته في طرابلس. يجب أن يكون لديه مكاتب قريبة من منشآت العقد، وإذا طُلب منه ذلك، يجب عليه تعيين وكيل يقيم في المبنى المخصص لتلك المكاتب. يكون هذا الوكيل مفوضًا لاستلام جميع الإشعارات الإدارية نيابةً عن المتعاقد.

Article 35: Notices

All notices, demands, requests, consents, approvals or other instruments required or permitted under this DBOT Agreement shall be made in writing and shall either be delivered by hand against written acknowledge of or on behalf of the addresses, or sent by recorded delivery mail or by facsimile transmission to the address specified by each of the parties for that purpose, recorded and appended to this DBOT Agreement or to such other address as may from time to time be notified by either party to the other for this purpose. In the event of service by mail or fax, service shall be deemed to have occurred upon receipt by the sender of an appropriate confirmation of delivery.

المادة الخامسة والثلاثون: الإشعارات

يجب أن تكون جميع الإشعارات أو المطالبات أو الطلبات أو الموافقات أو أي مستندات أخرى مطلوبة أو مسموح بها بموجب هذا الاتفاق مكتوبة، ويجب تسليمها يدويًا مقابل إشعار خطي بالاستلام من قبل المرسل إليه أو على نفقته، أو إرسالها بالبريد المسجل أو عن طريق الفاكس إلى العنوان المحدد من قبل كل طرف لهذا الغرض، المسجل والمرفق بهذا الاتفاق أو إلى أي عنوان آخر قد يُعلم به أي من الطرفين للطرف الآخر لهذا الغرض.

في حال خدمة الإشعار عبر البريد أو الفاكس، يُعتبر الإشعار قد تم بمجرد استلام المرسل تأكيدًا مناسبًا بالتسليم

Article 36: Operating statistics

In the first three months of each year, the Contractor will be required to submit a statistical statement of operations to the Contracting Authority.

المادة السادسة والثلاثون: إحصائيات التشغيل

في الأشهر الثلاثة الأولى من كل عام، يُطلب من المتعاقد تقديم بيان إحصائي عن العمليات إلى سلطة التعاقد.

Article 37: Bond

Before signing the DBOT Agreement, the Contractor will set up a performance security of US$ 1,500,000 (One million five hundred thousand US Dollars) in the conditions laid down by the laws and regulations concerning public works surety bonds. The performance security will be discharged after the signing of the statement of reception of Contractor's installations and replaced by a DBOT Agreement security of US$ 3,000,000 (Three million US Dollars). Any costs arising from measures implemented at the expense of the Contractor, under implementation of these specifications, would be taken from this bond. Whenever a sum is taken from the bond, the Contractor will be required to top up the amount within two weeks from the date of formal notice sent to it accordingly.

The bond will be returned to the Contractor at the end of the DBOT Agreement. However, in the event of forfeiture, the unreturned part of the bond will remain the property of the Contracting Authority.

المادة السابعة والثلاثون: الضمان

قبل توقيع إتفاقية التصميم والإنشاء والتشغيل ونقل الملكية، يجب على المتعاقد أن يقدم ضمانًا لحسن التنفيذ بمبلغ ١,٥٠٠,٠٠٠ دولار أمريكي (مليون وخمسمائة ألف دولار أمريكي) وفقًا للشروط المنصوص عليها في القوانين المتعلقة بكفالات الأعمال العامة. يتم إلغاء ضمان حسن التنفيذ بعد توقيع محضر استلام منشآت المتعاقد واستبداله بضمان تعاقد بمبلغ ٣,٠٠٠,٠٠٠ دولار أمريكي (ثلاثة ملايين دولار أمريكي). سيتم خصم أي تكاليف ناتجة عن التدابير التي يتم تنفيذها على نفقة المتعاقد، وفقًا لمتطلبات هذه المواصفات، من هذا الضمان. كلما تم سحب مبلغ من الضمان، يجب على المقترض إعادة المبلغ إلى المستوى المطلوب في غضون أسبوعين من تاريخ الإشعار الرسمي الذي يُرسل إليه وفقًا لذلك. سيتم إرجاع ضمان العقد إلى المتعاقد في نهاية فترة العقد. ومع ذلك، في حالة مصادرة الضمان، سيظل الجزء غير المعاد من الضمان ملكًا لسلطة التعاقد.

Article 38: Settlement of Disputes

The parties shall use their best efforts to settle amicably all disputes arising out of or in connection with this Agreement. A special Committee nominated by the Contracting Authority will give necessary assistance and support in order to reach an amicable settlement. Any dispute between the Parties as to the matters arising pursuant to this Agreement which cannot be settled amicably within 30 (thirty) days after receipt by one party of the other party’s request for such amicable settlement may be submitted by either party to the competent Court in the Republic of Lebanon. Lebanese Law shall apply to the interpretation of this Agreement and to any jurisdiction which may be invoked thereunder.

المادة الثامنة والثلاثون: تسوية النزاعات

يجب على الأطراف بذل أقصى جهدهم لتسوية جميع النزاعات الناشئة عن أو المتعلقة بهذه الاتفاقية بشكل ودي. ستقوم لجنة خاصة يعينها سلطة التعاقد بتقديم الدعم والمساعدة اللازمة من أجل الوصول إلى تسوية ودية. أي نزاع بين الأطراف بشأن الأمور الناشئة بموجب هذه الاتفاقية والذي لا يمكن تسويته وديًا خلال ٣٠ (ثلاثين) يومًا من استلام إحدى الأطراف طلب الطرف الآخر لتسوية ودية يمكن تقديمه من قبل أي طرف إلى المحكمة المختصة في لبنان. سيطبق القانون اللبناني على تفسير هذه الاتفاقية وعلى أي اختصاص يمكن الاستناد إليه بموجبها.

Article 39: Publication, printing, stamp duty registration costs

The publication, printing, stamp duly and registration costs of these specifications and appended items will be borne by the Contractor.

المادة التاسعة والثلاثون: تكاليف النشر والطباعة ورسوم الدمغة والتسجيل

يتحمل المتعاقد تكاليف النشر والطباعة ورسوم الدمغة والتسجيل لهذه المواصفات والمرفقات المتعلقة بها.

Signed in

Date

In two copies,

THE CONTRACTING AUTHORITY                                                                                             THE CONTRACTOR

Appended to this DBOT Agreement:

- Instructions to Tenderers

- Drawing of the DBOT Agreement

- List of installations made available by the Contracting Authority and statement

- List of installations put in place by the Contractor and statement

- Technical specifications (including the Contracting Authority's Requirements) For the installations but in place by the Contractor

- List of tariffs (in the date of signature of the Agreement)

- Operating regulations

- Insurance

- Notices

# PROJECT REPORT

### GENERAL

The Port of Tripoli wishes to supplement its facilities by installing a grain silo within the port domain.

The strategy adopted to accomplish this is to install the above-mentioned silo under a DBOT Agreement. The Contractor, who, will be both investor and developer, will develop the site by fitting out the platform and installing the silo with all related civil and MEP works, facilities and buildings and will then operate it for a predetermined time, on expiry of which the facilities will be transferred to the Port authority or the DBOT Agreement renewed.

This section describes the technical elements necessary for installing the grain silo and is designed to give the Contractor a clear view of the context in order to integrate the related investments into its proposal.

The following subjects will be examined successively:

- Specifications of the ships that can use the berth.

- Description of marine infrastructure.

- The design definition of the civil and MEP engineering structures required for the installation of the silo

- The technical characteristics of the silo, its equipment and utilities.

- The rules to be adopted for the operation and maintenance of the facilities.

- The technical specifications for the marine infrastructure.

### PROJECT ASSUMPTION: MINIMUM SILO SIZE

Refer to related Clauses specifying minimum dimensions and characteristics of the silo.

### PORT ENGINEERING

The Contractor must refer to attached drawings Reference: ST-HP-2030-MAR-TD-001/002 A, dated January 2023, showing a proposed conceptual:

- Overall area Mass plan of the facility.

- Grain silo mass plan.

The silo will be located in the western end of the container terminal of the Port, which is considered to be a sheltered one regarding harbor agitation and waves attack.

Dredging and debris removal shall be executed whatever seabed nature is, along the 200m berth length and for 30m width, using adequate dredgers and following environmental regulations in order to reach -15.50m MSL. Dredged material will be dumped in selected and approved dumping areas. These areas could be an offshore one, using split barges or another suitable system, in predefined and approved dumping areas by the relevant Public Authorities with minimum water depths not less than 50m, or a land reclaimed one, following dredged materials nature, Contracting Authority need…

Clearance of seabed from miscellaneous debris, wrecks… must be also achieved.

Dumping of salvaged debris cannot be done offshore, but in selected onshore discharges, according to in law regulations. The works execution by the Contractor regarding all installations to be made prior to the silo operating must be conform to the International and Lebanese standards and regulations and to all relevant public authorities’ requirements.

Regarding this issue, the Contractor must prepare and submit an Environmental Impact Assessment (EIA) study covering all aspects of the project (Preparation, works execution, installation, operation…) to the relevant Public Authorities, especially the Ministry of Environment, and obtain all necessary approvals necessary to launch the construction on site and later on to operate the facility. Furthermore, operating the silo during the DBOT Agreement period must be conform to International and Lebanese standards in addition to in law regulations and relevant Public Authorities requirements.

- CONTRACTING AUTHORITY’S REQUIREMENTS

### NATURAL CONDITIONS

All installations will be designed in order to resist and to be adapted to the site natural conditions, especially ground, winds and marine environment.

Any site surveys and geotechnical investigations relevant to the design of installations will be carried out. General level of Lebanon (NGL) is taken as level reference for this project.

Sea level, which is influenced by tides, winds effects and atmospheric pressure, could vary between -0,45m NGL and +0,80m NGL.

All directions mixed, maximum winds speed is:

- 26 m/s for a period of return of 10 years.

- 18 m/s for a period of return of 1 year.

- 10 m/s for a wind occurring 10 times a year.

### THE CONTRACTOR OBLIGATIONS

In respect with the DBOT Agreement, all installations will be designed, supplied, executed, installed, operated and maintained by the Contractor, except if Contractual Documents stipulate otherwise. In particular, the Contractor will have to fund and install all necessary installations for bringing water and electrical power from town (connections, network…) to the silo.

Following specifications are indicated as minimum and better solutions are encouraged, provided that they respect these specifications and keeping in mind that financial aspects are essential.

In all way, all actions and works to be done by the Contractor must be approved by the Contracting Authority prior to execution.

### PROJECT OBLIGATIONS AND CONSTRAINTS

The project zone shall be constructed, operated and maintained during all the DBOT Agreement period as per related International and Lebanese Standards.

### SILO CHARACTERISTICS

Reference is made to paragraph 1.1;

It shall include 24 grain bins (5,000T capacity per steel bin with flat bottom) with a 120,000T total capacity and the possibility for storage of wheat, corn, barley, sunflower and other.

The grain bins and all other miscellaneous equipment shall be new and unused and must be completely operational and ready for use, according to international rules and standards and fully classified by recognized classification societies.

Furthermore, 9 flat bottom bins (1,500T each) are to be provided and installed adjacent to the 5,000T ones.

The overall structural stability of the silo shall be ensured regarding soil bearing capacity through:

- Geotechnical investigation

- Soil improvement

- Piling system

- Slabs on grade

- Concrete foundations

- Excavation / filling…

A technical visit to the manufacture must be achieved by the Contracting Authority/Engineer and the Contractor in order to review all technical specifications before taking any action regarding supplying it to Tripoli Port, with all related electrical and mechanical equipment.

All further supplying negotiations, above mentioned visits, mobilization fees… will be on the Contractor’s full charge and responsibility.

### PRINCIPLE OF OPERATION AND UTILITIES

Operation of the silo shall be fully computerized, covering all aspects, taking in consideration necessary and approved health, safety and environmental measures.

### EQUIPMENT

Mechanical and electrical equipment shall be designed and approved by the Contracting Authority before its supplying by the Contractor.

All these equipment’s shall be conformed to International Standards like US and/or European ones, and provided from reliable and well-known manufacturers and shall include in minimum:

- One ship loader (Minimum 400T/hr).

- Two ship unloaders (Minimum 200T/hr each).

- Several loading truck stations.

- One unloading truck station.

- Handling/expedition tower.

- 4 weight bridges (~80T capacity each).

- Loading chain conveyor.

- Unloading chain conveyors.

- Belt conveyors.

- anchor bolts, level sensors, outside and inside ladders, roof handrail and radar level sensors.

- Catwalks, sweep augers, aeration systems, temperature systems, fumigation systems, Intake pit grids, Rotary drum cleaners with aspiration, weigher with aspiration, loading bellows with filters for the truck loading silos, sets of installation tools, installation cables, cleaner elevator, 2 main elevators with magnets, loading and unloading chain conveyors, slides, valves, ductworks and pipes, Central aspiration system, spare parts list and compressor units.

### ELECTRICAL POWER SUPPLIES

The silo must be equipped in minimum with electrical power supplies comprising:

- A main power supply.

- An emergency power supply.

The main power supply shall be provided from a transformer station.

### UTILITIES

The silo area shall include (in addition to bins, unloaders, loader and all necessary MEP equipment):

- Building offices (2 stories ~ 270m2 each) housing all the managerial and operating staff.

- Workshop (~160m2).

- Technical building (~75m2), for electrical installation comprising at minimum:

- A medium-voltage connection to the town grid network.

- A Transformer and main low-voltage switchboard equipped with all the outgoing lines necessary for the operation of the equipment, utilities and lighting. The construction of this transformer station will comply with the standards and regulations in effect.

- Guard house.

- 24-hour-per-day guard service.

- Storage area for storing the necessary tools, supplies and spare parts.

- Car parking (~3,200m2).

- Truck parking (~5,300m2).

- Any other structure needed to operate the silo.

The Contractor shall provide for the Contracting Authority all detailed drawings regarding the above-mentioned structures including necessary architectural, structural and MEP calculations reports and details, in addition to the infrastructures and overall layout of the mentioned leased area.

The Contractor shall not begin the construction or the installation of any structures before obtaining the final approval of the Contracting Authority on the listed documents and drawings.

### OPERATION AND MAINTENANCE

Operation of the silo includes functioning and maintenance which will be ensured by personal, equipment, tools and consumable resources, according to International Standards like US and/or European ones in addition to the recognized regulations.

### PERSONNEL

The personal resources (who’s CVs are to be approved by the Conceding Authority prior to employment with its full right to ask the Contractor to cancel any employee contract during operation for the reason that the Conceding Authority find suitable) shall comprise at minimum:

- General Manager   			: 1 post

- Head of Department		: 5 posts

- Team leader 			: 6 posts

- Head of Team 			: 5 posts

- Head of Delivery Unit   		: 4 posts

- Electromechanical Specialist 	: 6 posts

- Principal Machine Operator 		: 10 posts

- Machine Operator   			: 20 posts

- Accountant     			: 2 posts

- Administrative Employee 		: 7 posts

- Management 			: 4 posts

- Maintenance operator	 	: 12 posts

### MAINTENANCE AND REPAIR

Preventive maintenance and routine work interventions shall be carried out by the permanent personnel and will require maintenance supplies and consumables (grease, oil, standard tools, etc.). Repair operations may be carried out by specialized external The Contractors with respect to the DBOT Agreement.

### REGULATIONS

All installations shall comply with International Standards like US and/or European ones in addition to the recognized Laws. In particular, all installations shall comply with the safety rules and environmental protection norms set by Lebanese and International recognized Laws (eventual Environmental Impact Assessment will be carried out according to the Requirements of the Ministry of Environment).

The equipment provided shall comply with the applicable International Standards like US and/or European ones from all aspects.

The silo must be completely operational and ready for use, according to international rules and Standards like the US and/or European ones, newly built and fully operational. More specifically, the safety rules for personnel protection shall be applied.

The electrical and mechanical equipment shall comply also with International Standards like the US and/or European ones. The silo shall moreover be equipped with fire-fighting equipment. The alleyways will be equipped with standard fire extinguishers.

- TECHNICAL SPECIFICATIONS

### GENERAL

This specification details the requirements for the supply of material and execution of works in a marine environment.

The Drawings (Appendix 1) shall be considered in conjunction with this specification.

### STANDARDS, CODES OF PRACTICE AND REFERENCES

The following Standards and Codes of Practice are referenced within the Specification.

All materials, workmanship and testing shall conform to the requirements of the latest editions of the following Standards and Codes of Practice except as explicitly varied by this specification, or to any other requirements that can be requested by the Engineer:

- BS 6349 Maritime Structures;

- CUR/CIRIA Recommendations of Construction Industry Research and Information Association (CIRIA) C683, “The Rock Manual”, 2007;

- CUR/CIRIA Recommendations of Construction Industry Research and Information Association (CIRIA) C683, “The Rock Manual”, 1991;

- ISRM International Society for Rock Mechanics. Suggested Methods for Determining Point Load Strength, 1985;

- ACI 313-97: Standard practice for design and construction of concrete silos and stacking tubes for storing granular materials

- ANSI/ASAE EP433 DEC1988 (R2011) “Loads exerted by free-flowing grain on bins”

- AS 3774-1996 “Loads on bulk solids containers”

- BS EN 1991-4:2006: Eurocode 1 - Actions on structures – Part 4: Silos and Tanks

- BS EN 1993-4-1: Eurocode 3 - Design of steel structures - Part 4-1: Silos

- BS 812-111: Testing aggregates. Methods for determination of ten per cent fines value (TFV);

- BS 812-112: Testing aggregates. Method for determination of aggregate impact value

- BS 812-117: Testing aggregates. Method for determination of soundness;

- ASTM C 131-81: Test for Resistance to Abrasion of Small size Coarse Aggregate by Use of Los Angles Machine;

- ASTM D2938: Unconfined compression strength;

- BS 1367.2: Resistance to weathering;

- BS 1377-3:1990 Methods of test for soils for civil engineering purposes. Chemical and electro-chemical tests;

- BS 4027:1996 - Specification for sulphate-resisting Portland cement;

- BS 5930:1990 - Code of practice for site investigations;

- BS EN 196-2:2005 - Methods of testing cement. Chemical analysis of cement;

- BS EN 196-9:2003 - Methods of testing cement. Heat of hydration. Semi-adiabatic method;

- BS EN 196-21:1992 - Methods of testing cement. Determination of the chloride, carbon dioxide and alkali content of cement;

- BS EN 197-1: 2001- Cement - Part 1: Composition, specifications and conformity criteria for common cements;

- BS EN 450 -1: 2007 - Fly ash for concrete - Part 1: Definition, specifications and conformity criteria;

- BS EN 933-1:1997 - Tests for geometrical properties of aggregates. Determination of particle size distribution. Sieving method;

- BS EN 933-3:1997 - Tests for geometrical properties of aggregates. Determination of particle shape. Flakiness index;

- BS EN 933-4:2000 - Tests for geometrical properties of aggregates. Determination of particle shape. Shape index;

- BS EN 934-4:2001 Admixtures for concrete, mortar and grout. Admixtures for grout for prestressing tendons. Definitions, requirements, conformity, marking and labelling;

- BS EN 934-6:2001 Admixtures for concrete, mortar and grout. Sampling, conformity control and evaluation of conformity;

- BS EN 993-7:2000 - Methods of test for dense shaped refractory products. Determination of modulus of rupture at elevated temperatures;

- BS EN 1008:2002 - Mixing water for concrete. Specification for sampling, testing and assessing the suitability of water, including water recovered from processes in the concrete industry, as mixing water for concrete;

- BS EN 1097-2:1998 - Tests for mechanical and physical properties of aggregates. Methods for the determination of resistance to fragmentation;

- BS EN 1097-5:1999 - Tests for mechanical and physical properties of aggregates. Determination of the water content by drying in a ventilated oven;

- BS EN 1097-6:2000 - Tests for mechanical and physical properties of aggregates. Determination of particle density and water absorption;

- BS EN 12390-1:2000 - Testing hardened concrete. Shape, dimensions and other requirements for specimens and molds;

- BS EN 12390-2:2000 - Testing hardened concrete. Making and curing specimens for strength tests;

- BS EN 12390-3:2002 - Testing hardened concrete. Compressive strength of test specimens;

- BS EN 12390-6:2000 Testing hardened concrete. Tensile splitting strength of test specimens;

- BS EN 12390-7:2000 Testing hardened concrete. Density of hardened concrete;

- BS EN 1367-4:1998 - Tests for thermal and weathering properties of aggregates. Determination of drying shrinkage;

- BS EN 1744-1:1998 - Tests for chemical properties of aggregates. Chemical analysis;

- BS 1881-208:1996 - Testing Concrete: Recommendations for the determination of the initial surface absorption of concrete;

- ASTM C40-73C40-04 Standard Test Method for Organic Impurities in Fine Aggregates for concrete;

- ASTM C88-99a Standard Test Method for Soundness of Aggregates by Use of Sodium Sulphate or Magnesium Sulphate;

- ASTM C142-97(2004) Standard Test Method for Clay Lumps and Friable Particles in Aggregates;

- ASTM C295-03 Standard Guide for Petrographic Examination of Aggregates for concrete;

- ASTM C494/C494M-04 Standard Specification for Chemical Admixtures for concrete;

- ASTM C1260-01 Standard Test Method for Potential Alkali Reactivity of Aggregates (Mortar-Bar Method);

The design and construction shall comply with the requirements set by the local regulations (LIBNOR), the International Standards like US and/or European ones and with the requirements of the Contracting Authority and/or the Engineer.

The Contractor shall make an agreement with an international well-known entity, capable of issuing certificates of compliance for the design and execution phase, including procurement of equipment, erection of buildings, and products which must be subject to regulatory or non-regulatory checks, testing, verifications or inspections, based on norms and standards.

Safety and protection rules are to be been taken into account during installation and commissioning.

### RESPONSIBILITIES OF THE DBOT AGREEMENTNAIRE FOR THE WORKS EXECUTION

The selected Contractor shall be responsible for:

- Obtaining and maintaining all permits/clearances necessary to access the site and carry out the Works safely, including the compliance with the requirements of relevant Authorities and the instructions of the Engineer.

- Communicating only with the Engineer. No other external communication is authorized without notification and prior approval from the Engineer.

- Provision of all materials, plants and equipment, labor, consumables, site supervision, and all necessary requirements to complete the Works.

- Maintaining the site environmental conditions throughout the Works period, including provision of discharge or disposal procedure compliant with the requirements of relevant Authorities and/or the Engineer.

- Restoration of the fieldwork locations, where required, including reinstating access roads and other site areas to the satisfaction of the relevant Authorities and/or the Engineer.

- Full access and cooperation with any other third-party and Contractors that may be working on the site.

- Adherence to the Engineer's specifications.

- Compliance to any relevant environmental measures.

- Compilation of all required Method Statements and Health & Safety Plans.

- Providing insurances as per Engineer's requirements.

- Not proceeding with ordering any materials until approval in writing from the Engineer.

- Preparing and submitting to the approval to the Engineer a fully detailed program which shall be updated on a regular basis.

- Providing sufficient staff and equipment to perform the duties in executing the Contract.

- The full-time supervision of the whole of the Works.

- Liaising and co-ordination with any third-party Contractors or suppliers.

- Taking all precautions to prevent the outbreak of fire and providing adequate firefighting appliances on site to the approval of the Engineer.

- The Contractor and all persons employed by him and his Sub-Contractors, to comply with the Provisions of all Acts, Orders, Laws and Regulations concerning the safety, health or welfare of persons working on site as per Lebanese regulations.

- The Contractor and all persons employed by him and his Sub-Contractors, to comply with the provisions of the safety regulations at site.

- Ensuring that accidents shall be properly documented, reported immediately to the KCT Engineer and assistance given by way of medical help and transportation to hospital.

- Removal from site of all redundant plant on completion of the Works.

- Only modern, properly maintained equipment of the appropriate capacity and suitability for the duty required, shall be used for the Works.

- Protection of the Works from damage by his own workmen performing subsequent operations.

- Provision and maintenance of signs, red warning lamps and barricades as necessary.

- Immediately prior to handing over the works the Contractor shall thoroughly clean the whole of the works and surrounding area all to the approval of the Engineer.

- Ensure that excessive noise and pollution does not occur during the execution of the Works.

- Provision of all necessary attendance as may be required for the completion of the Works.

- Ensuring the proper adoption of QA / QC procedures.

- The working areas on site are subject to the limitations imposed by the Engineer. A temporary stockpile area will be made available to the selected Sub-Contractor (location and area to be approved by the Engineer).

- Temporary stockpile area will be made available to the selected Sub-Contractor (location and area to be approved by the Engineer).

- Supplying of all lighting and power required for the Works, the work of his Sub-Contractors, including supplying, operating and maintaining temporary switches, cables and leads. The Contractor shall pay for all charges for the current consumed and connections.

- Provision of temporary aids to navigation (if applicable) ensuring safe marine navigation at all times in accordance with the requirements of the concerned authorities.

- Preparation and submittal of "Shop", "Fabrication" and "As-Built" drawings.

- The Contractor to ensure the existing road to the project opened recently by the Contracting Authority is intact from any damage caused by construction traffic, and to maintain it periodically to the Engineer satisfaction.

- The Contractor has to provide periodical (weekly or as needed) interim survey report to the Engineer to enable proper monitoring of works, and to provide pre-surveys and a post-survey of every completed area, along with volume calculations as required and to the satisfaction of the Engineer.

- Submitting of a program of work in accordance with the Client requirements.

- Working the necessary shifts to complete the works in accordance to agreed program, while taking into consideration the local Statutory Authorities' requirements and limitations.

### PROVISIONS

The following items will be provided by the Contractor:

- Site offices on land for the Contractor and Engineer.

- Provision of utilities for the Contractor and Engineer site offices (power, water, high-speed internet connection) via temporary services.

- Maintenance of the Contractor and Engineer site offices (cleaning, consumables, etc.).

- Site offices will be watertight, lockable, air conditioned if necessary and provided with adequate lighting.

### INSPECTION

The Contractor, including any subordinate organizations engaged by him, shall be subject to surveillance inspection and audit by the Engineer.

Inspection of works will be performed by the Engineer while the Works are in progress to verify compliance with the specified technical and quality requirements, to observe the equipment and procedures, and to review the program.

Inspection or audit by the Engineer shall not relieve the Contractor of his responsibility for complying with the requirements of this specification.

### SCHEDULE FOR THE WORKS

The Contractor shall provide a detailed schedule for the Works in a chart format within two weeks from the starting date, and shall receive the approval of the Engineer on it, before starting Works execution on site.

### SUBMITTALS

The Contractor shall submit within two weeks from the starting date, and shall receive the approval of the Engineer on it, before starting Works execution on site, the following minimum information:

- Detailed proposed Method Statement discussing proposed phasing, Works by land, Works by water, HSE issues, minimization of environmental impact, disposal methodology on land, production/day expected, etc.

- The experience and qualifications of the Sub-Contractors in similar works (year, location, client, scope of Works, duration and value of the contract).

- The experience and qualifications of the Sub-Contractor project team.

- List of equipment to be assigned to carry out the Works.

- Work schedule.

- Compliance to the specifications.

### SITE SPECIFICATION

### GENERALITY

The required facility shall include in general:

- A grain silo, with the following minimum characteristics, dimensions and capacities:

● 120,000T total capacity of bins.

● 5,000T capacity per steel bin with flat bottom.

● Possibility for storage of wheat, corn, barley, sunflower and other.

- One ship loader (Minimum 400T/hr).

- Two ship unloaders (Minimum 200T/hr each).

- Several loading truck stations.

- One unloading truck station.

- Necessary mooring and quay accessories for ship berthing in front of the mentioned silo along 200m of quaywall line @-15.50 NGL.

- Handling/expedition tower.

- 4 weight bridges (~80T capacity each).

- Loading chain conveyor.

- Unloading chain conveyors.

- Belt conveyors.

- Anchor bolts, level sensors, outside and inside ladders, roof handrail and radar level sensors.

- Catwalks, sweep augers, aeration systems.

- Temperature systems, fumigation systems, Intake pit grids, Rotary drum cleaners with aspiration, weigher with aspiration.

- Loading bellows with filters for the truck loading silos, sets of installation tools, installation cables, cleaner elevator, 2 main elevators with magnets.

- Valves, ductworks and pipes, Central aspiration system, spare parts list and compressor units.

- Power and water supply systems.

- Auxiliary civil, MEP and building works that must be executed on the same land area of the silo.

- Car parking.

- Truck parking.

- CCTV system.

- Guard and security.

- …

### CLIMATIC DATA

in Tripoli, the summers are warm, muggy, arid, and clear and the winters are cool, wet, windy, and mostly clear. Over the course of the year, the temperature typically varies from 52°F to 86°F and is rarely below 46°F or above 88°F.

TEMPERATURE

The hot season lasts for 3.6 months, from June 19 to October 8, with an average daily high temperature above 81°F. The hottest day of the year is August 9, with an average high of 86°F and low of 77°F.  The cool season lasts for 3.4 months, from December 8 to March 21, with an average daily high temperature below 66°F. The coldest day of the year is January 23, with an average low of 52°F and high of 61°F.

PRECIPITATION

A wet day is one with at least 0.04 inches of liquid or liquid-equivalent precipitation. The chance of wet days in Tripoli varies significantly throughout the year.

The wetter season lasts 5.2 months, from October 24 to April 1, with a greater than 16% chance of a given day being a wet day. The chance of a wet day peaks at 33% on January 17. The drier season lasts 6.7 months, from April 1 to October 24. The smallest chance of a wet day is 0% on July 20. Among wet days, we distinguish between those that experience rain alone, snow alone, or a mixture of the two. Based on this categorization, the most common form of precipitation throughout the year is rain alone, with a peak probability of 33% on January 17.

RAINFALL

To show variation within the months and not just the monthly totals, we show the rainfall accumulated over a sliding 31-day period centered around each day of the year. Tripoli experiences significant seasonal variation in monthly rainfall. The rainy period of the year lasts for 6.8 months, from October 2 to April 27, with a sliding 31-day rainfall of at least 0.5 inches. The most rain falls during the 31 days centered around January 6, with an average total accumulation of 3.0 inches. The rainless period of the year lasts for 5.2 months, from April 27 to October 2. The least rain falls around July 30, with an average total accumulation of 0.0 inches.

HUMIDITY

Humidity comfort level is based on the dew point, as it determines whether perspiration will evaporate from the skin, thereby cooling the body. Lower dew points feel drier and higher dew points feel more humid. Unlike temperature, which typically varies significantly between night and day, dew point tends to change more slowly, so while the temperature may drop at night. A muggy day is typically followed by a muggy night.

Tripoli experiences extreme seasonal variation in the perceived humidity. The muggier period of the year lasts for 4.5 months, from May 30 to October 14, during which time the comfort level is muggy, oppressive, or miserable at least 23% of the time. The muggiest day of the year is August 4, with muggy conditions 91% of the time. The least muggy day of the year is December 28, when muggy conditions are essentially unheard of.

WIND

This section discusses the wide-area hourly average wind vector (speed and direction) at 10 meters above the ground. The wind experienced at any given location is highly dependent on local topography and other factors, and instantaneous wind speed and direction vary more widely than hourly averages. The average hourly wind speed in Tripoli experiences significant seasonal variation over the course of the year. The windier part of the year lasts for 4.2 months, from December 20 to April 25, with average wind speeds of more than 8.6 miles per hour. The windiest day of the year is February 7, with an average hourly wind speed of 10.1 miles per hour. The calmer time of year lasts for 7.8 months, from April 25 to December 20. The calmest day of the year is October 14, with an average hourly wind speed of 7.0 miles per hour.

Predominant average hourly wind direction in Tripoli is from the west throughout the year.

### SOME GRAINS DATA FOR BULK STORAGE

### CONFORMITY TO ATEX REGULATIONS

All facility, components and equipment shall be designed, built and operated in conformity to ATEX regulations, classified as follows:

| Area classification | Area inclusions |
| --- | --- |
| ATEX 20 | Inside volume of filter equipment and ducts Inside volume of vacuum cleaning system equipment and pipes |
| ATEX 21 | Inside volume of grain handling equipment Inside volume of storage cells Grain outlet discharge points on belt or chain conveyors Screw handling devices |
| ATEX 21 | Grain reception hopper at Truck Inside volume of hopper and buffer tank |
| No classified risk | Head House building Pits and cellars Galleries under cells Outside Catwalk Trucks and train reception building |

### HANDLING EQUIPMENT AND WEIGHT BRIDGES

CAPACITY OF THE INSTALLATION

Based on an average grain specific weight of 0.75, the Capacity of the installation shall be from 400 to 600 tons/hr.

TURNOVER OF STORAGE INSTALLATION PER YEAR

10 to 20 times per year.

CATWALKS LOADING

250kgs/m3

GALVANIZATION

All outside equipment shall be galvanized, in conformity with N.F. A 36.321 and A 46.321 NF A 91-121 and 91 - 122.

Damaged part will be painted with zinc rich paint, thickness 100 microns.

Galvanized spray paint is prohibited.

HANDLING EQUIPMENT AND WEIGHT BRIDGES

The mechanical equipment for handling equipment and weigh bridges shall include in minimum:

- Handling equipment (conveyor, elevator, valve, ducting, etc.).

- Over band.

- Scale / upstream and downstream hoppers.

- Cleaner.

- Expedition bins.

- Connexion ducting.

- Weight bridges.

- Sampling equipment.

- Compressed air equipment.

- Vacuum centralized cleaner.

- Security features (grid, casing, etc.).

UNLOADING SHIPPER

The mechanical equipment for the unloading shipper shall include in minimum:

- Gantry on rails.

- Turret / Driving cabin.

- Engine room.

- Ship unloading system (vacuum or chain conveyor).

- Chain conveyor.

- Telescopic loading spouts.

- Anchoring devices.

- Winch for Bobcat.

### STEEL COMPONENTS

Bins will be made of steel with a yield strength ranging from 350 MPa for wall rings up to 600 MPa for stiffeners.

Innovative metal coating can be applied, talking in consideration achieving the highest resistance to corrosion in aggressive environment (marine environment in this case).

Z-600 galvanized steel can be used also, containing 600 gr/m2 of Zinc, equivalent to a thickness of 42 microns per side which makes it longer-lasting even in the most aggressive of atmospheres (as established in Standard ISO 9223).

Minimum galvanization for stiffeners is Z-600. This option goes for stiffeners up to 3,5 mm, thicker stiffeners are hot-dip galvanized.

Silos will feature an inspection door for maintenance in the second wall ring as well as another one on the roof.

Access stairs to the silo, roof and gateways are to be designed to be safe and ergonomic, according to UNE-EN ISO 14122-2 standard and manufactured in highly resistant galvanized steel offering the best solution for silo inspection and maintenance.

Roof vents must guarantee proper bin ventilation preventing the entrance of water even in the most adverse of conditions.

For easier visual control of the silo and the material stored inside, inspection holes are included on the roof which, due to the quick-close system and easy access from the stairs, are an important handling tool.

Wind-Ring allows bin to maintain the bin’s cylindrical shape without any effect from wind loads which offers rigidity and safety.

They may be installed on the roof or on the wall ring depending on the diameter and height of the bin. Ventilation for proper grain conditioning and conservation must be ensured.

### MATERIALS

### CEMENT

A)  CEMENT GENERAL

1) The cement to be used throughout the Works shall be obtained from manufacturers approved in writing and shall be as described under one of the following headings and shall comply with the latest editions of the mentioned Standards:

a) Portland Cement (PC). Cement Class 42.5 N complying with BS EN 197-1 but containing not less than 4% and not more than 13% proportion by mass of tri-calcium aluminates.

b) Moderate Sulphate Resisting Portland Cement (MSRPC). Cement Class 42.5 N complying with BS EN 197-1 but containing not less than 4% and not more than 8% proportion by mass of tri- calcium aluminates, or cement complying with ASTM C150 Type II. In either case the cement shall not contain more than 2.7% proportion by mass of Sulphur trioxide.

c) Sulphate Resisting Portland Cement (SRPC). Cement complying with either BS 4027 Class 42.5N or ASTM C150 Type V.

d) High Slag Blast Furnace Cement (HSBC). Combination manufactured in the concrete mixer from Portland cement to BS EN 197-1 and ground granulated blast furnace slag (GGBS) conforming to BS 6699. The combination shall contain a minimum of 60% slag by mass of nucleus.

e) Portland Pulverized - Fuel Ash / Fly Ash Cement (PFAC).

Combinations manufactured in the concrete mixer from Portland cement to BS EN 197-1 and pulverized fuel ash (Fly ash) conforming to BS EN 450 (category S).

The combination shall contain at least 30% pulverized - fuel ash by mass of nucleus.

- Cement shall comply with the additional requirements in Table 1.

Table 1 - Additional requirements for cement

| Property | Test method | Limits |
| --- | --- | --- |
| Acid soluble alkali as Na2O equivalent | BSEN 196: Part 21 | Not higher than 0.6% |
| Heat of hydration | BS 4550: Part 3 | Not higher than 315 kJ/kg at 7 days |
| Specific surface | BSEN 196: Part 6 | 225 - 350m2/kg |

- The temperature of the cement shall not exceed 65ºC at the time of incorporation into a concrete mix.

Cement type for each mix shall be as shown in the table of concrete mixes.

B) CEMENT TESTING

- Cement shall be certified by the manufacturer as complying with the requirements of the appropriate specification.

- Before ordering cement, the Contractor shall submit details of the proposed supplier and information on the proposed methods of transport, storage and certification for approval and show that the quantity and quality required can be attained and maintained throughout the construction period.

Representative samples of the proposed cement are to be taken and forwarded to an independent laboratory approved by the Superintendent for analysis before the source is approved.

- Having obtained approval, the Contractor shall not change the agreed arrangements without permission.

- Each consignment of cement shall be accompanied by a certificate showing the place of manufacture and the results of standard tests carried out on each day's bulk production included in the consignment.

- Additionally, tests shall be carried out on each consignment of cement on arrival, and also at monthly intervals during storage.

- The Contractor shall store the cement so that separate consignments can be identified until the results of the testing are available.

- Tests should be carried out for the properties listed in Table 2 with test methods and limits to BS or ASTM as appropriate:

Table 2 - Tests to be carried out on cement

Properties to be tested

Strength

Fineness

Heat of hydration

Setting time

Soundness

Reactive alkali level as NA2 O equivalent

Chloride content

Loss on ignition

Insoluble residue

Tri-calcium aluminates content

SiO2, MgO, Ai2O3, Fe2O3, CaO contents

SO3

- The cementitious materials like GGBS, fly ash and MS to be used throughout the works shall be obtained from manufacturers approved in writing.

Testing and delivery of these materials shall be as per related Clauses.

### AGGREGATE

A)  AGGREGATE GENERAL

- Aggregate shall comply in all respects with BS 882 / BS EN 12620

- The Contractor shall obtain approval of proposed aggregate sources, and shall select suitable aggregate and samples of sand and stone for specified testing before obtaining aggregate

- Laboratory tests according to BS 812 shall be made at regular intervals to confirm the suitability of aggregate

B)  QUALITY AND TESTING

- Aggregate shall be free from earth, clay, loam and soft, clayey, shale or decomposed stone, organic matter, other impurities or any matter likely to affect durability of concrete and shall be hard and dense.

The percentage of hollow shells shall not exceed 3% by mass retained on 2.36mm sieve to BS 410 as determined by BS 812: Part 103.1

- Aggregate to be used in the construction of structures for retaining aqueous liquids shall comply with BS 8007.

Aggregate shall comply in all respects with BS 882 / BS EN 12620

- The following shall apply when tested in accordance with BS 812:

a) Water absorption shall not exceed 2%

- The following criteria shall be ascertained by testing in accordance with BS 812 and BS 882/ BS EN 12620, unless indicated otherwise, to assess the suitability of aggregate

- The results shall comply with the limits given in BS 882 / BS EN 12620 or as otherwise stated herein:

- Grading (BS 812: Part 103, BS EN 933-1)

- Magnesium sulphate soundness (BSEN 1367-2, ASTM C88)

- Specific gravity and water absorption (BS 812: Part 2)

- Clay, silt and dust content (BS 812: Part 103)

- Clay lumps and friable particles (ASTM C142)

- Organic impurities (ASTM C40, ASTM C87)

- Acid soluble sulphate and chloride content (BS 812: Parts 118 & 119, BS1377-3, and BS EN 1744-1)

- 10% fines value for coarse aggregates (BS 812: Part 111)

- Elongation (BS 812: Part 105.2) and flakiness (BS 812: Parts 105.1 & 105.2)

- Potential alkali reactivity (BSI/WG10, ASTM C289 and ASTM C227)

- Los Angeles abrasion test, if required (ASTM C131 & ASTM C535)

- Drying shrinkage in accordance with BS 812: Part 120

- Moisture content (BS 812: Part 109)

- The frequency of the above tests (a) to (m) shall be in accordance with Table 3.

| Test Reference | Frequency once per period of days |  |
| --- | --- | --- |
|  | Aggregate | Sand |
| (b) (c) (d) (e) (f) (g) (h) (i) (j) (k) (l) (m) | 1 30 7 1 7 30 3.5 7 3.5 Initial only Initial only Initial only 2/day | 1 - 7 1 - 30 1 - - Initial only - Initial only 3/day |

Table 3 - Test to be carried out on aggregates

- Tests (a), (d) and (m) shall be carried out as indicated or per 100m3 whichever is the more frequent.

- The combined grading of aggregate shall be determined on a daily basis.

The percentage passing any sieve size as determined by approved trial mixes shall be the target grading for all concrete of that type.

- The combined grading of the works concrete shall not vary by more than 4% on any sieve from that target.

C)  FINE AGGREGATE

- As defined in BS 882 BS EN 12620 fine aggregate shall be mainly capable of passing through a 5mm BS test sieve and shall be graded so that when mixed with the coarse aggregate and cement a concrete of maximum density is produced.

- Crushed sand may be added to natural sand in approved proportions in order to achieve the required grading. Crushed sand alone may not be used without approval.

- The amount of material passing a 75-micron BS 410 fine test sieve when tested in accordance with BS 812: Part 103 table 4 (wet-sieving method) shall not exceed 3% by mass.

- When subjected to five cycles of the soundness test specified in ASTM C88, fine aggregate shall show a loss not exceeding 10% when magnesium sulphate solution is used.

- The use of washed beach sand as fine aggregate may be permitted providing that the grading and salt content are maintained within the limits given herein.

Each sand shall be dug from above high-water level and washed thoroughly.

Before taking sand from any area, the salt content shall be established and only sand from approved areas shall be used.

- Sand for mortar shall comply with BS 1199 and 1200 / BS EN 13139.

D) COARSE AGGREGATE

- As defined in BS 882 / BS EN 12620 Coarse aggregate shall be mainly retained on a 5.00mm BS 410 test sieve; the grading shall be within the limits prescribed in BS 882 / BS EN 12620 so that when mixed with the approved fine aggregate and cement a workable concrete of maximum density is produced.

The densities of the classes of concrete shall be as approved after tests have been carried out on the site.

- The amount of material passing a 75micron BS 410 fine test sieve when tested in accordance with BS 812: Part 103 (wet-sieving method) shall not exceed 1% by mass.

- Drying shrinkage shall not 0.075 percent when tested in accordance with BS 812: Part 120.

- When subjected to five cycles of the soundness test specified in ASTM C88, coarse aggregate shall not show a loss exceeding 10% when magnesium sulphate solution is used.

- The flakiness and elongation indices of the predominant size fractions in each single-sized coarse aggregate, determined in accordance with BS 812, shall not exceed 30% by mass.

E) ALKALI - REACTIVITY POTENTIAL

- Aggregate shall not contain any materials that are reactive with alkalis in the aggregate itself or in the cement, the mixing water or in water in contact with the finished concrete or mortar in amounts sufficient to cause excessive localized or general expansion of the concrete or mortar.

- The Contractor shall initially assess an aggregate source by testing in accordance with ASTM C289.

If potential reactivity is indicated, then mortar bar tests in accordance with ASTM C227 shall be carried out and the results shall comply with the limits given in ASTM C33 for overall effect on the permanent works mixes, before use of the aggregate is approved.

F) SULPHATE AND CHLORIDE CONTENT

- The acid-soluble sulphate (SO3) content shall not exceed 0.4% by mass.

- The acid-soluble chloride content as chloride ion in aggregate as a percentage by mass shall not exceed the limits given in Table 4.

Table 4 - Sulphate and chloride content limits for aggregate

| Concrete type | Coarse aggregate | Fine aggregate |
| --- | --- | --- |
| Mass (unreinforced) | 0.06% | 0.05% |
| Reinforced | 0.03% | 0.03% |

Note: The concrete mix shall also comply with the overall limits for chloride and sulphate content given elsewhere in this specification.

### REINFORCEMENT

A) STEEL FOR REINFORCEMENT

- Reinforcement for use in reinforced concrete shall comply where appropriate with BS 4449, 4466, 4482, 4483, BS 8110 and 8666.

- Reinforcement for structures shall comply with the requirement in BS 5400.

- High-yield deformed bars shall be Type 2 complying with BS 4449, unless otherwise approved.

- Modulus of elasticity for any steel reinforcing bar shall be not less than 200kN/mm².

- Steel for pre-stressing shall be separately approved.

B)  TYING WIRE

- Tying wire shall be galvanized mild steel of diameter approximately 1.5mm.

- Tying wire shall have the same minimum concrete cover as detailed for reinforcement.

- Tying wire for use with epoxy coated steel reinforcement shall be nylon coated.

The ends of the tying wire shall be turned into the main body of the concrete.

C) TESTING

- Prior to approval the Contractor shall provide copies of the manufacturer's certificates of test results relating to the steel reinforcement to be supplied, and shall additionally provide independent test results obtained from an approved laboratory.

- Samples for testing shall be selected randomly from each consignment of reinforcement delivered to the site representing different days' production at the manufacturer's works.

- One test shall be carried out per 5,000m length of bars delivered to site.

D) STAINLESS STEEL

- Where specified and shown on the Drawings stainless steel reinforcement shall be hot rolled austenitic stainless-steel Type 316S33 to BS 6744, with a minimum tensile strength of 510N/mm2.

- Test certificates and product analysis certificates shall be provided. Note that Type 304 is strictly excluded.

E) EPOXY COATED STEEL

- Where specified and shown on Drawings, fusion bonded epoxy coated steel reinforcement shall comply with BS ISO 14654.

- Epoxy coating for reinforcement shall comply with BS ISO 14656.

### WATER

A) WATER GENERAL

- The Contractor shall make his own arrangements and obtain approval for the provision of fresh water for the manufacture and curing of concrete.

B) QUALITY

- Water to be used for mixing and curing concrete and mortar shall be fresh and free from sediment and dissolved or suspended matter which may be harmful and shall comply with the requirements of BS 3148 or BS EN 1008.

- Water samples from the intended source of supply shall be taken for analysis before any concrete work is commenced, and at intervals throughout the duration of the Contract.

- If the samples are unacceptable the Contractor shall either change to a new supply or take steps to improve the existing source, as approved.

C) SULPHATE AND CHLORIDE CONTENT

- The sulphate (SO3) content shall not exceed 500mg/l, and the chloride ion content shall not exceed 350mg/l.

Tests to establish the contents shall be carried out at monthly intervals.

Note: Each concrete mix shall also comply with the overall limits for chloride and sulphate given elsewhere in this specification.

### ADMIXTURES

A) ADMIXTURES

- Where approved or specified, admixtures complying with BS 5075, BS EN 934-2 or ASTM C494 shall be used as a means of:

a) Achieving workability with the lowest appropriate water-cement ratio in order to achieve durability;

b) Controlling and retarding setting;

c) Reducing bleeding and associated plastic settlement and cracking.

- Set-retarding and water-reducing admixtures shall normally be limited to those based on ligno-sulphonates or modified ligno-sulphonates or sodium naphthalene sulphates.

Air entraining agents shall be based on neutralized vinsol or other resin.

- Unless approved by the Superintendent admixtures from different manufacturers shall not be used.

Compatibility of different admixtures with binders like Fly ash and GGBS shall be tested and approval shall be obtained from the Superintendent before the trial mix approval.

- Admixtures other than those specified herein shall only be used provided that the prior approval of the Superintendent has been obtained and any information or test results required by him have been provided and are to his satisfaction.

- Admixtures containing chlorides or other corrosive agents shall not be used.

- In the event of admixtures being used, the concrete tests described in the Specification shall be incorporated to ensure the specified strengths are achieved and comparison shall be made with concrete manufactured without the admixture to prove that the density has not been reduced.

- When admixtures are used in the Works very strict control is to be maintained to ensure that the correct quantity of admixture is used at all times.

- The equipment to be used for dispensing and the method of incorporating the admixture in the concrete shall be to the approval of the Superintendent.

- The dispensing unit shall be translucent such that the operator can see the discharge of the admixture. Calibration of dispensing unit shall be checked and approved.

B) POLYPROPYLENE FIBERS

- Fibrous concrete reinforcement shall be 100% virgin polypropylene fibers specifically manufactured for use as a concrete reinforcement and so certified by the manufacturer. It shall contain no reprocessed olefin materials.

- The fiber dosage shall provide a minimum surface area of 200m2 of fibers per cubic meter of concrete. The length of each fiber shall be between 10mm and 50mm. Fibers may be monofilament or fibrillated.

C) CORROSION

- Corrosion inhibitor admixtures (Hycrete X1000 or equivalent) shall be added to reinforced concrete mix design and the Sub-Contractor should provide either by itself or through the admixture supplier the warranty that no side effects would occur and the minimum required compressive strength of the mix design will be achieved.

### DELIVERY AND STORAGE

A) CEMENT

- Cement shall be delivered in bulk or in sealed and marked bags, and shall be protected from the weather by enclosed transfer systems or other approved coverings.

- The Contractor shall provide approved silo to store sufficient bulk cement for continuity of work, and the cement shall be placed therein upon delivery.

Approved precautions shall be taken to prevent cement dust causing a nuisance.

- Alternatively, the Contractor shall provide ventilated sheds with raised floors for the storage of sufficient bagged cement for continuity of work.

- Each consignment shall be stacked separately for inspection, testing and approval before use.

- The cement shall be placed in the sheds upon delivery and shall be used in the order in which it was delivered.

B) AGGREGATE

- Aggregate shall be stored in concrete-based bins with the base laid to falls to provide drainage away from the storage area or on stages to prevent intermixing and the inclusion of dirt and foreign materials.

- Each size of aggregate shall be stored separately.

- Storage bins shall be emptied and cleaned regularly.

C) REINFORCEMENT

- Steel reinforcement shall be stored in an approved manner above ground, at a location covered by a concrete ground slab, under cover and racked as necessary for protection from aggressive elements.

- Canvas or similar covers shall be provided for the reinforcing bars to prevent the deposit and accumulation of deleterious materials on the bars. Similar provision shall be made to cover the bars in the reinforcement bending area.

### CONCRETE GRADE

- MIXES GENERAL

1) Concrete shall be as shown in the Table of Mixes below (Table 5). The number of mixes may require to be increased.

2) The criteria given in Table 5 are designed to produce concrete of the required strength and durability.

B)  TABLE OF MIXES

Table 5 - Table of mixes

| Concrete grade | Type of cement | Maximum size of aggregates | Minimum quantity of cement | Maximum quantity of cement | Maximum free water/cement ratio | Specified Characteristic Strength (Cubic) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mm | kg/m3 | kg/m3 |  | 7-day N/mm² | 28-day N/mm² |
| 30/20/S | SRPC | 20 | 350 | 400 | 0.47 | 20 | 30 |
| 40/20/M | MSRPC | 20 | 370 | 400 | 0.42 | 26 | 40 |
| 40/20/S | SRPC | 20 | 370 | 400 | 0.42 | 26 | 40 |
| 40/20 | PC | 20 | 370 | 400 | 0.42 | 26 | 40 |
| 40/20/H or 40/20/P | HSBC or PFAC | 20 | 380 | 420 | 0.42 | 26 | 40 |
| 50/20/H or 50/20/F | HSBC or PFAC | 20 | 380 | 420 | 0.4 | 33 | 50 |

Notes on table of mixes:

PC           Portland cement

SRPC       Sulphate resisting Portland cement

MSRPC   Moderately sulphate resisting Portland cement

COMB    Combination manufactured in the mixer at the time of concrete production

- Grade 50/20/F shall contain polypropylene fibers as specified.

- The Characteristic Strengths are for concrete which has been cured at a temperature of 20°C ±1°C, and are the values below which no more than 5% of the test results fall. The 7-day strengths shall be used only as a guide.

- HSBC shall contain at least 60% GGBS complying with BS 6699 and concrete shall comply with the additional limits as indicated in related clause.

- PFAC shall contain minimum 30% Pulverized Fuel Ash complying with BSEN 450 and shall comply with the additional limits as indicated in related clause.

- Concrete containing HSBC or PFAC may contain Silica Fume/micro silica complying with EN13263 or equivalent approved by the Superintendent up to 5% by weight of binder.

- Concrete with HSBC, PFAC and Micro silica requires particular attention concerning protection and curing. The Sub-Contractor shall provide a comprehensive method statement with information on material, incorporation and special precautions when using micro silica and other minerals.

C) QUALITY AND TESTING

- Concrete shall comply with BS 8500-1 and BS 8110 except where BS 8007 or this specification differs.

- Sampling for test purposes shall comply with BS 1881 Part 101.

- The making and curing of specimens shall comply with BS 1881 Parts 108, 109, 110 and 111.

- If air-entrainment is specified the average air content at the time of placing measured in accordance with BS 1881 shall be:

- Concrete containing 20mm maximum size aggregate 5% ±1%.

- Concrete for water-retaining structures shall be watertight and shall comply with the recommendations of BS 8007.

- Mass concrete for paving shall be tested to BS 1881 Part 118 and shall have a characteristic flexural strength of 4.0N/mm2 at 28 days.

Characteristic flexural strengths are for concrete which has been cured at a temperature of 20°C ±1°C and are values below which no more than 5% of the test results fall.

- If concrete specimens are cured at higher temperatures or for longer periods than BS 1881 Part 111 requires, the adjusted Characteristic Strength shall be calculated as follows:

100 f'/f = A + B log [24D (T+12)/1000]

f'   = adjusted Characteristic Strength

f    = specified Characteristic Strength

T   = curing temperature

D  = age at testing in days

A & B are coefficients given in Table 6.

The above equation applies only to PC, MSRPC and SRPC.

| Record cube strength N/mm2 | A | B |
| --- | --- | --- |
| Less than 15 15 to 35 Greater than 35 | 10.0 20.0 30.0 | 67.5 20.0 52.5 |

Table 6 - Coefficients for the calculation of the concrete’s adjusted characteristic strength

This calculation may be applied for curing at temperatures up to 27°C.

- Before placing concrete, the Sub-Contractor shall obtain approval of the mixes proposed for each class of concrete and the average target strengths.

The mixes shall be designed to achieve the minimum workability for the Contractor to place and compact the concrete with the equipment proposed for use.

- The mean strength shall exceed the Characteristic Strength by a margin of 1.64 times the standard deviation expected from the concreting plant, except that no standard deviation less than 3.5N/mm2 shall be used as a basis for designing a mix.

### TRIAL MIXES

A) LABORATORY TRIAL MIXES

- Preliminary laboratory tests shall be carried out to determine the mixes to satisfy the specification with the available materials.

- Trial mixes shall be tested to determine the following properties of mixes proposed for initial field tests:

a) Bleeding in accordance with ASTM C232 (non-vibrating) Nil/negligible.

b) Drying shrinkage in accordance with BS 812-120 / BRE Digest 357.

c) Air content if applicable.

d) Free water/cement ratio.

e) Workability.

f) Fresh and hardened concrete densities.

g) Absorption in accordance with BS 1881: Part 122.

- The upper target limit for absorption after 30 minutes shall be 2% for reinforced concrete and 2.3% for unreinforced concrete.

- The following tests should commence when the concrete specimens are 28 days old:

a) Initial Surface Absorption Tests (ISAT) shall be carried out in accordance with Test 6 of BS 1881: Part 5, 1970 and BS 1881: Part 208, 1996.

b) The target limits shall be as in Table 7:

Table 7 - Target limits for ISAT

| Time after starting test | 10 min | 30 min | 1 hour |
| --- | --- | --- | --- |
| ISAT results ml/m2/s | 0.25 | 0.17 | 0.10 |

- For Reinforced Concrete only:

a) Penetration of water in accordance with DIN 1048. The target limit for penetration at 4 days shall be 30mm.

b) Chloride permeability shall be in accordance with ASTM 1202. The target limit shall be 2000 Coulombs.

- For Concrete with HSBC or PFAC only:

a) Penetration of water in accordance with DIN 1048.

b) The target limit for penetration at 4 days shall be 10mm.

c) Chloride permeability shall be in accordance with ASTM 1202.

d) The target limit shall be 1000 Coulombs

e) Chloride diffusion test shall be carried out as per Nord NT build 492

- Maximum diffusion coefficient shall be 1.75x10-12m2/s at 90 days at 20ºC or as approved by the Superintendent.

- If any of the values obtained for properties (a) to (h) for unreinforced concrete or (a) to (m) for reinforced concrete that were mentioned in clause 2.2.B.4 are unacceptable, the mixes shall be re-designed.

B) INITIAL FIELD TESTS

- Trial mixes shall be prepared under full-scale site conditions and tested in accordance with BS 1881.

- Samples of concrete incorporating the reinforcing details to be used shall be cast and examined, before hardening using hand tools, and after hardening by coring to assess the mixes.

Cores shall be 150mm diameter by 200mm long.

- Trial mixes shall be made on each of three days; the workability shall equate to the designed target value. Six cubes from each mix shall be taken, three for test at 28 days.

Further trial mixes shall be made if the range (the maximum minus the minimum of the three cube results in any batch) exceeds 15% of the average of that batch, or if the range of the three batch averages exceeds 20% of the overall average of the batches.

- The mixes shall be tested to determine the following properties:

a) Bleeding in accordance with ASTM C232.

b) Air content if applicable.

c) Free water/cement ratio.

d) Flexural strength in accordance with BS 1881: Part 118.

- The average 28-day Characteristic Strength of the three trial mixes shall not be less than the designed mean strength, and the results of the above tests shall be acceptable before the mix is approved. Otherwise, the mix shall be re-designed.

### QUALITY CONTROL

A) TEST SPECIMENS

- Test specimens shall be manufactured in an on-site laboratory, specially equipped for the purpose, in controlled conditions.

- They shall be made, cured, stored, transported and tested to BS 1881. The method of compacting test specimens shall be as approved.

- The testing machine shall be housed in a laboratory and calibrated to BS EN 10002-3 when delivered; the calibration shall include the use of the Foote strain gauge and be verified at 3-monthly maximum intervals.

B) SAMPLING: COMPRESSIVE STRENGTH

- A sample of concrete shall be taken at random on eight separate occasions during each of the first five days of using a mix and tested for compressive strength. The standard deviation shall be calculated from at least 40 individual cube results each representing separate batches of similar concrete produced by the same plant under the same supervision. The current margin for the plant shall be thus established as 1.64 times the standard deviation.

- Thereafter one sample shall be taken at random for each class of concrete from every group of 25 batches made by each batching plant, and at least one sample shall be taken each day that concrete of a particular grade is made.

- In addition to the above requirements, at least one sample shall be taken from each individual structural unit, or part of a unit, when the latter is the product of a single pour.

- From each sample three cubes shall be made for testing at 28 days and one for testing at 7 days for control purposes.

- The 28-day results shall be the mean of the three cubes.

- The frequency of sampling may be required to be varied.

- The procedures shall be repeated when materials or design mixes are changed.

C) SAMPLING: FLEXURAL STRENGTH

- A sample of mass concrete for paving shall be taken at random on eight separate occasions during each of the first five days of using a mix and tested for flexural strength. The standard deviation shall be calculated from at least 40 individual results each representing separate batches of similar concrete produced by the same plant under the same supervision. The current margin for the plant shall be thus established as 1.64 times the standard deviation.

- Thereafter one sample shall be taken at random for each class of concrete from every group of 25 batches made by each batching plant, and at least one sample shall be taken each day that concrete of a particular grade is made.

- In addition to the above requirements, at least one sample shall be taken from each individual structural unit, or part of a unit, when the latter is the product of a single pour.

- From each sample three specimens shall be made for testing at 28 days and one for testing at 7 days for control purposes. The 28-day result shall be the mean of the three specimens.

- The frequency of sampling may be required to be varied.

- The procedures shall be repeated when materials or design mixes are changed.

D)  STRENGTH RESULTS

- The results will be unacceptable if:

a) The average strength determined from any four consecutive test results does not exceed the specified characteristic strength by 0.5 times the current margin, or

b) One or more results in any forty consecutives are less than 85% of the specified characteristic strength, or

c) Three or more results in any forty consecutives are less than the specified characteristic strength.

- One or more of the following actions will then be instructed:

a) Changing the mix.

b) Improving quality control.

c) Cutting and testing specimens from placed concrete.

d) Load – testing relevant structural units.

e) Non – destructive testing of placed concrete.

f) Cutting – out and replacing defective concrete.

- If the range of individual strengths of specimens made from the same sample exceeds 15% of the mean, then the method of making, curing and testing specimens shall be checked. In the event of the range exceeding 20% the KCT Engineer will order one or more of the actions outlined in (a) to (f) above.

- In the event of (c) the Sub-Contractor shall cut specimens from approved locations.

- Cores shall be tested in accordance with BS 1881 as modified by Concrete Society Report TR 11 including Addendum (1987). The values shall be reduced by 0. 69 N/mm2 per week of age in excess of 28 days.

E)  OTHER TESTS

- Durability test as per the Clause 08.1 (I, j k and l) shall be repeated for each class of concrete for every accumulative quantity of 1500m3 of concrete with a maximum of one test per day by each batching plant.

- The air content of air-entrained concrete shall be determined to BS 1881: Part 106 for each batch produced until consistency has been achieved, when fewer batches may be tested.

- Compaction factor, slump, Vebe or other workability tests shall be carried out at the site of the pour on each truck load of concrete delivered. The workability shall be as for the trial mixes; permitted tolerances shall be in accordance with BS 8500-1.

- Random monitoring of the plastic density of the concrete shall be carried out weekly, using a calibrated container.

### BATCHING

A)  MACHINERY

- Batching shall be by weigh-batching machines equipped with accuracy checks for the weighing mechanism. The machines shall be cleaned, checked and adjusted regularly as approved.

- The water supply to the concrete mixers shall have a metering system to control and record the amount.

B)  ACCURACY OF BATCHING

- Batched materials shall be measured out within the tolerances in Table 8 and discharged into the mixer without loss:

Table 8 - Batched materials measuring tolerances

| Cement | ± 2% of the weight of the cement in the batch |
| --- | --- |
| Aggregate | ± 2% of the weight of each aggregate in the batch |
| Water | ± 2% of the weight of water added to the batch |
| Admixture | 5% of the amount to be added to the batch |

2) The batched quantities shall be adjusted to compensate for variation in the moisture content of the aggregates with the approval of the Superintendent.

C) CALIBRATION OF MEASURING EQUIPMENT

- Measuring equipment shall be checked and calibrated at the start of preliminary concrete tests and at weekly intervals. The necessary test weights and the like shall be kept available on site.

- Scales shall be checked over their complete range by a specialist every three months. A calibrated container shall be used to check the accuracy of admixture dispensers once each month. The results of these checks shall be notified.

D)  MIXING

- Concrete shall be mixed in batches in machines which comply with BS 1305.

- The constituents shall be thoroughly mixed before discharge.

- The machines shall be capable of discharging while running.

E)  READY-MIXED CONCRETE

- Ready-mixed concrete shall not be used unless approved, and shall comply with the requirements specified herein and those of BS 1926 and the Quality System for Ready Mixed Concrete (QSRMC) Technical Regulation.

- Ready-mixed concrete, plants and mixer trucks shall be subject to approval.

- Truck-type concrete mixers shall comply with the requirements of BS 4251.

F)  RECORDS

1)  Daily returns shall be provided showing the quantities of cement and the total volume batched of each class of concrete for each section of the works and temporary works.

### CONTROL OF CHLORIDES AND SULPHATES

A) CHLORIDES IN CONCRETE

- The total estimated chloride ion content as a percentage by weight of the cement in the mix shall not exceed the following limits:

- For reinforced concrete:

a) 0.20% if made with cements other than SRPC;

b) 0.1% if made with SRPC.

- For mass concrete:

a) 0.50% if made with cements other than SRPC;

b) 0.12% if made with SRPC.

- The Contractor shall test the constituents of the concrete to establish these contents as provided for elsewhere in this Specification.

- In addition, regular tests to BS 1881: Part 124 for chloride content shall be made on the hardened concrete. The following values are acceptable:

a) For reinforced concrete made with cements other than SRPC:

95% of the test results less than 0.20% Cl by weight of cement and no result greater than 0.25% Cl by weight of cement.

b) For reinforced concrete made with SRPC: 95% of the test results less than 0.1% Cl by weight of cement and no result greater than 0.13%.

c) For mass concrete made with cements other than SRPC: 95% of the test results less than 0.5% Cl by weight of cement, and no result greater than 0.65% Cl by weight of cement.

d) For mass concrete made with SRPC: 95% of the test results less than 0.12% Cl by weight of cement and no result greater than 0.15% Cl by weight of cement.

- All of the above limits shall apply in addition to the limits for individual ingredients of the mix given elsewhere in this specification.

B)  SULPHATES IN CONCRETE

- The total estimated sulphate content (SO3) of the mix calculated from the ingredients including that present in the cement shall not exceed 3.7% by weight of cement in the mix.

- In addition, regular tests to BS 1881 Part 124 shall be made on the hardened concrete to determine the total sulphate content, which shall not exceed 4% by weight of cement in the mix.

- The above limits shall apply in addition to the limits for individual ingredients of the mix given elsewhere in this specification.

C)  PERMISSIBLE LEVEL OF CHLORIDES AND SULPHATES

- The permissible level of chlorides and sulphates quoted in the above clauses shall not be considered as mean values for the whole of the Works, but shall apply to any concrete.

### PLACING

A)  PLACING GENERAL

- Concrete shall be placed in the position and sequence indicated on the Drawings, or as directed.

- Placing shall not be commenced until the fixing and condition of reinforcement and items to be embedded and the condition of the containing surfaces or formwork has been approved.

- 24 hours notification shall be given of the intention to place concrete.

- Concrete shall be transported by means which prevent contamination (by dust, rain etc.) segregation or loss of ingredients, and shall be transported and placed without delay.

- Where the time to compacting the concrete in its final position is likely to be greater than 1 hour after introduction of cement to the mix, retarding admixtures shall be used. Retarding admixtures shall be chloride free and shall only be used with prior approval of the Superintendent. Dosage rates of admixtures shall be determined following trial mixes, and consider transit time and ambient temperature.

- The Sub-Contractor shall take all necessary measures to prevent cracking in concrete especially due to autogenous, plastic shrinkage, thermal cracking, etc. These measures shall include avoiding placing concrete under environmental conditions when plastics shrinkage is likely to occur.

- Concrete shall be placed directly in its final position without segregation or displacement of the reinforcement, embedded items and formwork. Concrete shall not be placed in water, except as specified.

- Peak temperature of the concrete during hydration should not exceed 70ºC.

B)  EXTENT OF POURS

- The limit of individual pours and the height of lifts shall be as approved.

- For walls, the length of panel placed at one time shall not exceed 6m; adjacent panels shall not be placed within 2 days, but shall be placed as soon as practicable thereafter. Subsequent vertical lifts shall not be poured within 2 days.

- Floors, roofs and ground slabs shall be placed in an approved sequence of panels. 'Chequerboard' placing shall be avoided. Adjacent panels shall not be placed within 2 days, but shall be placed as soon as practicable thereafter. Panels shall not exceed 25m2.

- If the use of slip-forms or paving trains is permitted, these limits will be revised.

- The sequence of pours shall be arranged to minimize thermal and shrinkage strains.

C)  PLACING EQUIPMENT

- Concrete shall generally be placed without segregation by pumping or bottom-opening skips. If chutes are used their slopes shall not cause segregation and spouts or baffles shall be provided.

D)  TIME FOR PLACING

- Concrete and mortar must be placed and compacted within 45 minutes of water being added to the mix or otherwise included via damp aggregates, unless admixtures are in use. Partially-set concrete shall not be used in the works.

E)  COMPACTION

- Concrete shall be compacted during placing by approved internal vibrators.

- 2)  The vibrators shall operate at a frequency of not less than 10,000 cycles per minute, and shall be designed for continuous operation.

- The performance of vibrators shall suit the working conditions, and they shall generally not be less than 75mm diameter.

- The radius of influence shall ensure that the mass under treatment is compacted at a speed commensurate with the rate of supply of concrete.

F)  VIBRATORS

- Vibrators shall penetrate the full depth of the layer of concrete placed and just into the layer below, and be withdrawn slowly to avoid the formation of voids.

- Vibration shall not to be applied directly or indirectly to concrete after the initial set has taken place, nor shall it be used to make concrete flow in formwork.

G)  CONTINUITY OF PLACING

- Placing in each section of work shall be continuous between construction joints. The Sub-Contractor shall make provision for standby equipment.

- If the placing of concrete is delayed due to breakdown, then the Sub- Contractor shall erect vertical stop-ends and form a construction joint or remove the concrete already placed and restart after repair of the breakdown, as directed.

H)  PLACING IN INCLEMENT WEATHER

- Placing shall not take place in the open during storms or heavy rains. If such conditions are likely to occur the Sub-Contractor shall provide protection for the materials, plant and formwork so that work may proceed. If strong winds are prevalent protection from driving rain and dust shall be provided.

I)   PLACING IN HIGH TEMPERATURE

- The temperature of concrete shall not exceed 30°C at the time of placing and the temperature differential shall not exceed 25ºC for concrete containing carbonates aggregates or 20ºC for concretes containing siliceous aggregates.

- Temperature differential is defined as the difference in temperature between the form or face of a concrete pour and the center of the pour, or at a distance of 1.5m, whichever is the shortest).

- Recommendations for Hot Weather Concreting' in the 1991 edition of ACI 305 shall be complied with. The following measures inter alia may be required to control the placing temperature:

a)  Shade aggregate, cement silos, water tanks and concrete plant

b)  Paint concrete plant white

c)  Run concrete plant with flake ice before mixing or transporting concrete

d)  Use chillers to cool the mixing water

e)  Add flake ice as a proportion of the mixing water

f)  Place concrete at night

g)  Use of liquid nitrogen

- The Sub-Contractor shall supply suitable maximum/minimum thermometers and record the shade and sun temperatures at locations where concrete is being placed. In addition, the Sub-Contractor shall also record wind speed and relative humidity at the place of concreting.

- Concreting shall not be permitted where the rate of evaporation exceeds 1.0kg/m2/hr.

J)   CONTROL OF THERMAL CRACKING IN LARGE POURS

- For large pours, i.e. where the thickness of the concrete element exceeds 600mm, the Sub-Contractor shall adopt special precautions, to avoid thermal cracking due to temperature differential.

- In no case shall this temperature differential exceed 25ºC for concrete containing carbonate aggregates or 20ºC for concretes containing siliceous aggregates.

- The difference in the mean temperature between the hardening concrete and adjacent hardened concrete, which will restrain the movement of the hardening concrete, should not exceed 12ºC unless specifically approved on the basis of the Sub-Contractor’s detailed documentation. Peak temperature of the concrete during hydration should not exceed 70ºC.

- The Contractor shall submit a detailed method statement to the Superintendent, for approval, defining his proposed arrangement to avoid the effects of thermal cracking and temperature differential. The method statement shall include but not be limited to, the size and sequence of pours, additional curing regime, concrete temperature- monitoring system of pours, formwork type and removal time, and calculations for temperature and strain development at internal and surface locations, taking into account heat of hydration, ambient radiation and temperature, and physical restraints.

- In case early age cracks are observed, the Contractor shall replace/remediate the defects to the satisfaction of the Superintendent.

K)  PLACING AT NIGHT

- If approval has been given for placing at night or in dark interiors, adequate lighting shall be provided where mixing, transportation and placing are in progress.

L)   PLACING UNDER WATER

- Underwater concrete shall be placed with minimum disturbance of the water. Running water and wave wash shall be controlled. The specified concrete grade shall be used and the mix design shall provide for good flowing ability.

Tremie pipes, bottom-dump skips or other approved placing equipment shall be used. Segregation shall be avoided.

- Placing shall be commenced in approved sections and continued to completion.

- The tremie pipe shall be buried in the concrete and the pipe must not be emptied until the pour is complete.

If a bottom-dump skip is used, the contents shall be covered by canvas or similar before lowering into the water.

The doors shall be opened when the skip is resting on the bottom with no tension in the support cable, and the skip shall be lifted gradually so that the concrete flows out steadily.

M)  Preparation before Placing

- Before placing concrete for reinforced work on the ground, the formation shall be compacted as specified and a screed of blinding concrete shall be applied to form a surface for construction.

- Before placing concrete on or against rock, masonry, brickwork or old concrete, loose material shall be removed and the surface washed down;

Water seepage shall be stopped or channeled away from the work.

- For mass concrete placed against masonry or brickwork the following shall apply:

a) The mortar joints in the face work shall have fully hardened.

b) The surface shall be soaked prior to placing.

c) The concrete shall be worked around ties and bond stones and into open joints.

### FORMWORK

A)  FORMWORK GENERAL

- The Contractor shall obtain approval of the methods and materials proposed. Details of formwork for special finishes shall be approved before materials are ordered. Formwork shall provide concrete of the shape, lines and dimensions shown on the Drawings.

- Formwork shall be constructed from materials of sufficient strength, supported to provide rigidity during placing and compacting concrete without discernible deflection and shall be removable without disturbing the concrete.

Internal ties shall be metal.

Removable ties shall be located so that the specified cover to reinforcement is maintained to all surfaces including that of the tie-holes.

If ties are left in the cover shall be as specified for the reinforcement or as approved.

Tie cavities shall be roughened and filled with approved non-shrink concrete or epoxy mortar.

- Formwork panels shall have true edges for accurate alignment and shall be fixed with either vertical or horizontal joints.

Where chamfers are required, the fillets shall be cut to provide an even line.

Joints shall not permit leakage of grout, nor steps and ridges in exposed surfaces.

B)  ROUGH FORMWORK

- Rough formwork shall be butt-jointed, seasoned, sawn timber.

C)  FINE FINISH FORMWORK

- Fine finish formwork shall be used for all concrete surfaces unless detailed otherwise on the Drawings. This finish shall be obtained from forms designed to produce a hard smooth surface with true, clean arises.

D)  PREPARATION OF FORMWORK FOR CONCRETING

- Formwork and supports shall be cleaned; temporary openings shall be provided for the removal of rubbish.

The formwork shall be coated with an approved release agent and the excess removed. Release agent shall not be allowed to come into contact with concrete already placed or with reinforcement.

- Not less than 4 working hours’ notice shall be given for the inspection and approval of the formwork and reinforcement, prior to which concrete shall not be placed.

E)  REMOVAL OF FORMWORK

- Formwork shall be removed without damage to the concrete, but not until the concrete has sufficient strength to support itself.

Centers and props may be removed when the member has sufficient strength to carry itself and any loading. External loading shall not be applied until the concrete has reached the 28 days CCS.

- Table 9 is a guide to the minimum periods between placing and the removal of formwork for concrete made with PC, MSRPC and SRPC:

Table 9 - Minimum periods between placing and removal of formwork

| Vertical sides of beams, walls, Columns |  |  |
| --- | --- | --- |
| lift not exceeding 1.2m | 12 hours | but see Curing and Protection Clause |
| lift exceeding 1.2m | 36 hours |  |
| Soffits of main slabs and beams Props left under | 5 days |  |
| Beams and main slabs Removal of props | 18 days |  |
| Vertical sides of beams, walls, Columns |  |  |
| lift not exceeding 1.2m | 12 hours | but see Curing and Protection Clause |
| lift exceeding 1.2m | 36 hours |  |
| Soffits of main slabs and beams Props left under | 5 days |  |
| Beams and main slabs Removal of props | 18 days |  |

- For mixes containing other types of cement the formwork striking times shall be determined in accordance with CIRIA Report No 136

Formwork striking times - Criteria prediction and methods of assessment.

- After removal remedial work shall not be undertaken until the concrete has been inspected and approved.

F)  PERMANENT FORMWORK

- Permanent formers of GRP pipe are to be used where indicated on the Drawings. The minimum wall thickness of the GRP shall be 5mm.

### REINFORCEMENT

A)  REINFORCEMENT GENERAL

- Steel rod reinforcement shall be cut, bent and fixed to BS 8110: Part 1 / B.S. 8666.

- Cutting or bending by the application of heat is not permitted.

- 3) Welding of reinforcement shall only be permitted when approved in writing by the Superintendent. If such approval is given then the workmanship shall be in accordance with BS EN 1011.

The Contractor shall submit full technical details of his proposed procedures prior to seeking approval.

B)  FIXING

- The number, size, form and position of pieces of reinforcement shall be as shown on the Drawings. They shall be held in position in the formwork during the placing of concrete by use of distance pieces and spacer bars.

- Links shall be taut so that bars are braced and the inside of their curved parts shall be in contact with the bars being connected. Tying wire shall be twisted tight with pliers and the free ends shall be bent inwards.

- Reinforcement shall be grit-blasted before use to remove rust, oil, grease, salt and other deleterious matter, and where pitting has occurred the causes and products thereof. Repeated blasting may be required when the reinforcement is in position, or partially cast in. Partially-set concrete adhering to exposed bars during concreting operations shall be removed.

- The use of mechanical couplers of any type or form shall only be as detailed on the Drawings or as instructed by the Superintendent. Couplers shall conform to the requirements in BS 5400.

- Reinforcement temporarily projecting from the concrete at joints shall not be bent out of position without approval, in which event the reinforcement shall be bent over a suitably sized former to prevent any damage or over-stressing.

C)  BENDING SCHEDULES

- The Contractor will be supplied with drawings detailing the reinforcement required and shall prepare for approval bending schedules in accordance with BS 4466/BS 8666. Laps and anchorages shall be 45 bar diameters in length unless detailed otherwise.

D)  WELDING

- Electric arc welding may be used, if approved, for joining bars. Covered-alloy or shielded-arc electrodes shall conform to BS EN 499.

- Workmanship shall be to BS 5135.

Joints shall be butt-welded with standard double-V or double-U welds.

E)  COVER TO REINFORCEMENT

- The location of reinforcement and nominal dimensions of concrete cover shall be as shown on the Drawings.

- Tolerance for the concrete cover shall be ±5 mm for superstructure. However, reinforcement for substructure shall be placed within a tolerance of plus 10mm (towards increasing the concrete cover) and minus 5mm (towards reducing the concrete cover). The achievement of cover shall be confirmed by checking by cover meter after concreting.

- Nominal cover shall be measured to the reinforcement nearest to the concrete face concerned.

- Only approved spacers may be used in permanent works. Before spacers are approved for use in the works, their capability to securely hold the reinforcement in position during concreting without detriment to concrete placement, compaction or durability shall be fully demonstrated.

- Concrete spacers shall be of similar concrete grade and durability to the parent concrete and shall have non-metallic ties. The spacers shall comply with the specified requirements for water absorption. The materials and installation or spacer shall comply in accordance with BS 7973 Part 1 & 2:2001.

- Plastic spacer shall be allowed for piles only subject to the approval of Superintendent.

### JOINTS

A)  CONSTRUCTION JOINTS

- Construction joints shall be located and the sequence of placing arranged as approved, or as shown on the Drawings, to minimize shrinkage and thermal strains in the concrete.

- Concrete placing shall not be interrupted except where joints occur, and shall continue after normal hours if necessary to achieve this.

- Joints shall be formed square to the work with keyways included.

- Horizontal joints shall be generally at least 500mm above ground level if Design Sulphate Classes DS-4, DS-4m, DS-5 or DS-5m of BRE Special Digest 1, Part 1, Table 2 apply, and 500mm above high-water level in marine locations, or as shown on the Drawings.

- Upon removal of the formwork the joint face shall be inspected, and if the soundness of the concrete is not approved the Sub-Contractor shall investigate and remedy defects.

- Before placing is resumed at a joint the set surface shall be roughened to remove laitance and expose the aggregate; the concrete shall have gained sufficient strength to ensure that aggregate is not in any way damaged or loosened within the matrix.

- If damaging materials have come into contact with the surface of the joint the concrete shall be cut back and the roughened surface cleaned by compressed air or water jets and brushed and watered immediately before placing.

- Chemical surface-retarders shall not be used.

- Construction joints shall be sealed with an approved sealant on debonding tape at external and liquid-contact faces.

- Construction joints in water-retaining structures shall incorporate an approved waterbar and construction joint details shall be submitted to the Superintendent for approval.

B)  DESIGN JOINTS

- Expansion and contraction joints shall be as shown on the Drawings.

- A contraction joint in a non-water-retaining structure shall form a plane of discontinuity in the member. The concrete face first cast shall be painted with two coats of approved rubberized bitumen paint before the adjacent concrete is placed. The adjacent concrete shall include a groove against the joint for sealant. The exposed edges shall be sealed with an approved sealant on debonding tape.

- If a contraction joint is likely to be contaminated, the joint shall be sealed immediately with an approved free-flowing sealing fluid as soon as the formwork has been removed.

- An expansion joint in a non-water-retaining structure shall be formed as for a contraction joint, but non-absorbent closed-cell polyethylene joint filler shall be included so that the adjacent concrete members can expand.

- A design joint in a water-retaining structure shall include a continuous waterstop strip of copper, rubber, rubber and steel or PVC fixed across the joint as shown on the Drawings. The concrete shall be free from honeycombing and worked against the embedded part of the strip. Projecting portions of the strip shall be protected from damage during operations and, in the case of rubber and plastic, from light and heat.

- Bituminous paint shall be applied to the lips of the loop of copper water stop and the loop filled with bituminous compound before embedding in the concrete.

- The method of joining water stops shall be in accordance with the manufacturer's instructions.

### CURING AND PROTECTION

A)  Curing and Protection

- Concrete shall be protected from sunshine and drying winds by approved shading and wind breaks, and from cold, rain or running water, for a period of 14 days after placing. Additional measures for protection and curing must be taken while using concrete containing GGBS / Fly ash or micro silica. During this period the following measures shall be taken to prevent the loss of moisture and to minimize thermal stresses caused by the difference in temperature between the surface of the concrete and the core of the concrete mass:

- Horizontal surfaces:

a)  Polythene sheeting shall be placed immediately after finishing.

b) After final set has taken place, the polythene shall be replaced by wet hessian covered with polythene; the hessian shall be kept permanently damp.

c) After 14 days the hessian and polythene shall be removed and an approved aluminized or white resin-based curing compound applied. The rate of application shall be as recommended by the manufacturer.

d) Alternative methods of curing must be approved before use where special finishes are required.

- Vertical surfaces:

a) Polythene over wet hessian shall be secured to the surfaces immediately after removal of the formwork. The hessian shall be kept permanently damp.

b) After 14 days the hessian and polythene shall be removed and an approved aluminized or white resin-based curing compound applied.

- Water used during curing operations shall be fresh water. Curing membranes shall be compatible with waterproofing or other materials that may subsequently be applied to the surface of the concrete.

- In case of using approved curing compound, products with at least 90% efficiency shall be applied. The rate of application shall be as per the manufacturer’s recommendations. Application of curing compound shall be allowed only after 7 days of wet curing.

B)  CONTAMINATION

- Concrete shall be protected from contamination by sea or brackish water, oil, fuel and other deleterious materials for a minimum period of 30 days after placing.

C)  INSULATING FORMWORK

- Insulating formwork shall be left in place for 72 hours after placing or until the temperature peak of the concrete is reached. The initial curing period in (B)(1) above may then be reduced in proportion.

D)  PROTECTION OF JOINTS

- Rebates formed to receive sealant and the surfaces of construction joints shall be protected from curing compound by wet hessian to ensure proper curing of the joint surface and adjacent concrete. The protection shall remain in place until the joint surface is sealed.

### FINISHES

A)  FINISHES GENERAL

- The finished faces of concrete shall be sound, even colored, even- textured and free from defects. Arises shall have a 20 x 20mm chamfer unless detailed otherwise on the Drawings. A fine finish shall be provided unless detailed otherwise on the Drawings.

B)  CONCRETE SURFACES WITHOUT FORMWORK

- On upward-facing surfaces which do not require formwork or special finish the finish shall be produced by proper placing and compacting operations alone.

- For a fair finish screening shall be used, carried out by sliding and tamping a screed board running on the top edges of the formwork, or on screening guides, to give a dense concrete skin.

- For a fine finish screening shall be used as described, then left until the concrete has stiffened and the film of moisture has disappeared. A steel or wooden float shall then be used for a glossy or sandpaper surface as required. Working shall be the minimum compatible with a good finish. The surface shall be protected from waterdrops.

C)  WIRE-BRUSHED FINISH

- After removal of the formwork the surface of the concrete shall be abraded by stiff wire brushes and water to remove the cement laitance and expose the aggregate.

D)  BUSH-HAMMERED FINISH

- The surface shall be abraded by carborundum stones to remove irregularities.

- Within 3 weeks, the surface shall be bush hammered to remove the cement laitance and expose the aggregate. Approved bush hammers shall be worked to within 12mm of corners and arises; the remaining 12mm shall be hand-chiseled to match.

- Bush hammers shall be operated perpendicularly to the surface, and the remaining exposed aggregates shall not be loose or fractured.

- The treated surface shall be washed with water and stiffly brushed. The exposed aggregate shall be clean and free from film.

E)  CHEMICAL RETARDERS

- Chemical surface retarders, if approved, may be used to produce an exposed aggregate finish, and the Sub-Contractor shall demonstrate that the durability of the concrete surface is not reduced.

F)  CARBORUNDUM FINISH

- Carborundum finish shall be achieved by sprinkling carborundum grit on the unset surface and working-in by wooden float.

- The grit shall vary in size between BS 1.18mm mesh and BS 0.60mm mesh and shall be distributed from a BS 1.18mm hand-screen at the rate of 2.15kg per m2.

G)  SPECIMEN PANELS OF CONCRETE

- The Sub-Contractor shall produce specimen panels of finished concrete for approval. The approved panels shall be retained by the Superintendent and used to determine the acceptability of concrete finishes in the Works.

### SPECIAL CONCRETE

A)  PRECAST CONCRETE

- The Sub-Contractor shall submit for approval details of arrangements for casting, handling and placing precast units.

- The Sub-Contractor shall cast sample panels for approval, and approved panels shall be retained on site as the control standard for subsequent panel production.

- Precast concrete units shall be cast on manufactured beds. The beds shall not be liable to settlement and shall have smooth, hard and level surfaces. Each unit shall be marked with a serial number and date of casting. Steel bars shall not be embedded in the concrete for lifting.

- Units shall not be removed from the beds until the representative flexure-test beams reach adequate strength for handling and shall not be placed until the cubes representing them reach the appropriate 28- day Characteristic Strength.

B)  NO-FINES CONCRETE

- The aggregate for no-fines concrete shall be coarse graded from 10mm to 20mm. A small percentage of fines from 10mm to 5mm may be added to improve the strength if approved. Cement shall be mixed with the aggregate in the proportion of 1 to 8 by volume. Segregation of the cement grout shall be prevented.

C)  GRANOLITHIC CONCRETE

- Granolithic concrete shall consist of one part by weight cement to three parts of combined coarse and fine aggregate.  The combined aggregate grading shall be as in Table 10:

Table 10 - Combined aggregate grading for granolithic concrete

| BS sieve | Percentage passing |
| --- | --- |
| 14 mm | 100 |
| 10 mm | 95 - 100 |
| 5 mm | 30 - 45 |
| 2.36 mm | 30 - 35 |
| 1.18 mm | 15 - 25 |
| 0.60 mm | 10 - 20 |
| 0.30 mm | 5 - 10 |
| 0.15 mm | 0 - 5 |

- Granolithic concrete shall be laid on top of the unset base concrete, and compacted and worked to the correct levels.

The surface shall be floated with a steel float after hardening until water sheen has disappeared. Cement or cement-sand shall not be sprinkled onto the surface. The layer shall be 12 to 18mm thick.

- If a granolithic layer is required to be placed on set concrete, the latter shall be scrabbled and cleaned to expose the aggregate, and an approved bonding agent applied. The layer shall not be less than 50mm thick.

- If required, compounds shall be added or applied to give a concrete with improved dust proof and oil-proof qualities. The compounds shall be used in accordance with the manufacturer's instructions.

- Granolithic concrete paving shall be placed in panels not exceeding 3m2. Approved contraction joints shall be provided around the perimeter of each panel.

D)  CEMENT-MORTAR, GROUT AND RENDERING

- Cement-mortar shall consist of one part cement and four parts fine sand by volume with just enough water to achieve workability.

- Grout shall consist of cement mixed with water in approved proportions. Fine sand may be included in approved quantities.

- Rendering shall consist of three parts fine, sharp sand to one part cement applied in two 10mm coats and one 5mm finishing coat. The color of the finishing coat shall be as approved.

- Acid-resistant epoxy mortar shall be obtained from an approved manufacturer and shall be applied in accordance with the manufacturer's instructions.

- Mortar, render and grout shall be used freshly mixed.

### PROTECTIVE COATINGS

A)  EXTERNAL SHEET TANKING MEMBRANE

- External sheet tanking membrane to concrete substructures shall be an impervious, cold applied flexible laminated sheet, consisting of multi - layer high density cross laminated polyethylene film with a backing of self-adhesive rubber bitumen compound to give a combined thickness of 1.5mm and protected with silicone coated release paper.

- The mass of the membrane shall be not less than 1.6 kg/m2 gross.

- A special grade of compound formulated for hot climates shall be used, which has in excess of 10 years of successful usage in the Middle East.

- The laminate shall withstand cracking of the substrate up to a crack width of 0.6mm.

Minimum test performance data shall be as follows:

Table 11 – Minimum test performance protective coating.

| Property | Test Method | Results |
| --- | --- | --- |
| Tape Strength | ASTM D638 | Long 4.2 N/mm Trans 4.8N/mm |
| Tensile Strength | ASTM D638 | Long 42 N/mm2 Trans 48N/mm2 |
| Elongation Film | ASTM D638 | Long 210 % Trans 160% |
| Tear Resistance | ASTM D1004 | Long 270N/mm Trans 270N/mm |
| Adhesion to Primed Concrete | ASTM D1000 | 1.8 N/mm |
| Adhesion to Self | ASTM D1000 | 1.8 N/mm |
| Puncture Resistance | ASTM E154 | 290N 65mm |
| Water Resistance | ASTM D570 | ---- |
|  | After 24 hours | 0.14% |
|  | After 35 days | 0.95% |
| Environmental Resistance | ASTM D543 | Conforms |
| Moisture Vapor Transmission Rate | ASTM E96 | <0.3g/m2/24 hrs |
| Adhesive Softening Point | ASTM D36 | Not lower than 103°C |
| Adhesion to Primed Concrete | ASTM D1000 | 1.8 N/mm |
| Adhesion to Self | ASTM D1000 | 1.8 N/mm |

- Generally, for floor slabs, the membrane shall be applied to the blinding concrete. The protective layer of concrete or screed shall be placed to extend 200mm outside the lines of the floor slab.

- For vertical faces the dry surfaces shall be cleaned and primed.

- A fillet 25 x 25mm of bitumen putty shall be formed at the corner between the wall and the protruding membrane covered blinding layer.

- The membrane shall be backed to the wall, brought down over the fillet, and extended over the protruding margin of the bottom membrane to form a watertight seal.

- A layer of concrete or mortar shall be placed over the jointed section. Minimum 150mm overlaps shall be provided at joints.

- Sheet tanking membrane shall be Bituthene 1000X HC, or equal and approved.

- Prior to the application of the membrane to vertical surfaces, the concrete shall be primed with one brush coat of compatible primer specially formulated for hot climates containing 50% aromatic hydrocarbon solvents and 50% bitumen solids, giving an average coverage of approximately 10 to 12m2 per liter, dependent on texture and porosity of concrete surface. The primer shall be compatible with the waterproofing, recommended by the waterproofing manufacturer, and specially formulated for hot climates.

- Cold applied rubber/bitumen mastic compound, or bitumen putty for trowel application shall be used for molding into fillets and collars. Tapes shall be used for sealing around pipes and irregularities. The grade of materials shall be formulated for hot climates.

- Waterproofing membrane shall be protected from mechanical damage by a bitumen-impregnated board of minimum thickness 3mm. The board shall be weather, water and rot-proof. The board shall be nearly cut to closely abut at joints which shall be continuously taped with 75mm wide strips of heavy-duty self-adhesive tape.

- The membrane shall extend to 75mm below finished ground level where it shall be tucked into a 20x20mm continuous rebate, secured by tantalized hardwood battens fixed at 300mm centers by shot-fired masonry nails, and pointed with approved sealant.

- Above membrane tuck-in level, the vertical edges of concrete structures shall be coated to 200mm above finished ground level, or other level as detailed on the Drawings, with an approved pitch extended epoxy resin coating to a minimum thickness of 250 microns.

B)  EXTERNAL BRUSH APPLIED TANKING MEMBRANE

- Substructures shall be protected externally, where required, with a two-coat brush-applied tanking membrane applied to the top of blinding concrete and to the outside surfaces of all buried concrete.

- Coatings shall be solvent-based bituminous compounds complying with the UK Building, Research Station recommendations for damp-proof membranes, Digest No 54, Series 2.

- In addition, the first coat shall comply with BS 3416 Types 1 and 2.

- The second coat shall incorporate non-asbestos fiber reinforcement and shall be applied to a minimum thickness of 1.5mm.

- The surfaces shall be cleaned and brought to a fine finish before coating. Each coat shall be applied at the rate specified by the manufacturer.

- Coating shall be protected by protective board during backfilling.

### TOLERANCES

A)  TOLERANCES OF CONCRETE SURFACES

- The tolerances of concrete surfaces shall be in accordance with the following:

a)  Foundations and other in – situ buried concrete: BS 5606

b) Exposed concrete: BS 5606

c)  Floor slabs to receive self-levelling floor compound: ± 8mm

d) Floor slabs with concrete wearing surface: BS 8204: Part 2, Class SR1 with level tolerance of ± 8mm

### CONCRETE SURFACE TREATMENT

A)  ABOVE GROUND CONCRETE COATINGS – GENERAL

- All exposed surfaces of reinforced concrete elements shall be protected by a water repellent, chloride resistant coating. Coatings for specific applications are to be as shown on the drawings. All other surfaces shall receive the protective and decorative coating specified in related clause.

- All concrete coatings shall be supplied by an approved manufacturer with technical representation in the Republic of Lebanon, and a track record of product applications in the Republic of Lebanon. All materials shall have a proven 10 years satisfactory performance in climatic conditions similar to the Republic of Lebanon.

The complete coating system including primers shall be applied in accordance with Manufacturer's instructions.

- Coatings shall be applied by a specialist applicator approved by the Manufacturer and the KCT Engineer, and shall have at least 5 years proven successful experience.

- A method statement for application shall be submitted giving full details of all equipment and application methods proposed and safe access provisions. The method shall include wet and dry film thickness tests, pull off tests and any other quality control tests appropriate to the coating performance.

- Sample Panels of each coating type shall be prepared, before approval of material and applicator, on L shaped panels comprising vertical and horizontal surfaces of at least 1 sqm each.

- Full records of areas coated, quantity of material applied, ambient and substrate temperature, and humidity shall be kept on a daily basis and submitted to the KCT Engineer.

B)  SURFACE PREPARATION

- Surfaces shall be lightly grit blasted to remove all contamination such as oil, grease, loose particles, decayed matter, laitance, mold release oils and curing compounds.

- Any surface defects and blow holes shall be filled to produce a fine finish using a proprietary product such as an acrylic modified cementitious repair fairing coat (or mortar for larger defects). The repaired shall be completed at least 48 hours before application of coatings.

C)  COATING FOR GENERAL EXPOSURE

- Unless a particular finish is specified on the drawings, all exposed concrete surfaces shall be coated with the following system.

- The coating system shall comprise a penetrating silane-siloxane primer and subsequent pigmented coating of minimum total thickness 150 microns DFT applied in two coats.

- Minimum test performance for the complete coating system shall be:

Table 12 – Minimum test performance coating system

| Carbon dioxide diffusion resistances (Taywood method). Equivalent thickness of air: Initial: 2000 hours QUV weathered | More than 140m 100m |
| --- | --- |
| Reduction in chloride ion penetration (diffusion cell method) | More than 98% |
| Able to bridge crack of at least | 0.3mm |
| Water vapor transmission (SD) Taywood method, of not more than | 1.0m |
| Fire Testing to BS 476 Pt 7 1987 Spread of Flame: | Class 1 |

- The coating shall be UV resistant, resistant to alkalis and water, and available as standard in a range of pale colors including white, grey, beige, green and blue.

D)  SILANE

- Exposed concrete surfaces shall be given two coats of silane protective treatment applied according to the manufacturer’s instructions.

- The silane material shall comply with United Kingdom Highway Agency BD43/90.

### LIST OF TEST REQUIREMENT

TESTS FOR CEMENTITIOUS MATERIALS, AGGREGATE, WATER AND STEEL

Table 13 - Tests for cementitious materials, aggregate, water and steel

| Sr | Particular | Test | Applicable Standard |  | Limit (s) | Minimum Frequency |
| --- | --- | --- | --- | --- | --- | --- |
| A | Cement | Test for compliance with appropriate standard indicated in PART 2 including the following specific requirements: |  |  |  |  |
|  |  | Alkali level |  | BS EN196 | Max 0.6% | Each source change in material* |
|  |  | Heat of hydration |  | BS 4550 : Part 3 | 315kJ/kg @ 7 days | Each source change in material* |
|  |  | Specific surface fineness |  | BSEN 196: Part 6: 1992 | Shall not be greater than 350m2/kg & not less than 225m2/kg | Each source change in material* |
|  |  | C3A content (tri- calcium aluminates content) |  | BSEN 196: Part2:1995 | OPC- 4-13% SRC- 3.5% | Each source change in material* |
|  |  | Chloride content |  | BSEN 196-21 | Max 0.05% | Each source change in material* |
|  |  | Early strength Standard Strength |  | BSEN 196-1 | As per BS EN197 | Each source change in material* |
|  |  | Initial setting time |  | BS EN 196-3 | As per BS EN197 | Each source change in material* |
|  |  | Soundness (expansion) |  | BS EN 196-3 | As per BS EN197 | Each source change in material* |
|  |  | Loss of ignition |  | BS EN 196-2 | As per BS EN197 | Each source change in material* |
|  |  | Insoluble residue |  | BS EN 196-2 | As per BS EN197 | Each source change in material* |
|  |  | Sulphate content |  | BS EN 196-2 | 3.0% by mass | Each source change in material* |
|  |  | Pozzolanity |  | BS EN 196-5 | As per BS EN197 | Each source change in material* |
| B | GGBS (Slag) | Testing to confirm compliance with requirements of BS 6699 |  |  |  | Each source change in material* |
| C | PFA (Fly ash) | Testing to confirm compliance with requirements of BS EN 450 |  |  |  | Each source change in material* |
| D | Micro Silica (Silica Fume) | Testing to confirm compliance with requirements of EN 13263 / ASTM C1240-95 / NS 3045 |  |  |  | Each source change in material* |
| *After acquiring approval for each source from the Superintendent each consignment of cement and other cementitious materials shall be accompanied by a compliance certificate from the manufacturer. Further test as may be required by the Superintendent in case of doubt. |  |  |  |  |  |  |
| E | Aggregates | Grading |  | BS 812: 1989 P103, BS EN933-1 | Standard | ▪1 test per 1000m3 |
|  |  | Material finer than 0.075mm |  | BS 812: 1989 P103 | 3 % Max. | ▪1 test per 1000m3 |
|  |  | Clay lumps and friable particles |  | ASTM C 142 | 1% Max. | ▪1 test per 4000m3 |
|  |  | Organic impurities |  | ASTM C 40 | Lighter than std. color | ▪1 test per 2000m3 |
|  |  | Water absorption |  | ASTM C128:2001/C127:2001 | 2 % Max | ▪1 test per 2000m3 |
|  |  | Specific gravity |  | ASTMC128;2001/C127:2001 | 2.6 % Max | ▪1 test per 2000m3 |
|  |  | Shell content |  | BS 812:1995 P106 | 5 % Max | ▪1 test per 2000m3 |
|  |  | Particle shape |  | BS 812:1990 P105.1, 105.2 | 30 % max | ▪1 test per 2000m3 |
|  |  | Acid soluble |  | Qualitative BS812:1988 Part |  | ▪1 test per 1000m3 |
|  |  | chlorides, Cl |  | 117 Appendices A/B |  | ▪Each 12 concrete day if result is more than 75% of the limit & each month if results are less than 75% of the limit. |
|  |  |  |  | Qualitative BS 812:1988 Part 117 Appendices C, BS 1377-3 | 0.01-0.03% |  |
|  |  | Acid Soluble Sulphates, SO3 |  | BS 812: 1988 Part 118, BS EN 1744-1 | 0.3% Max. | ▪Per 1000m3 or each 24 concrete days if result is more than 75% of the limit & each 2 mo. If result is less than 75% of the limit whichever is less. |
|  |  | Soundness, MgSO4 (5 cycles) |  | ASTM C88:1999A, BS EN1367-2 | 12 % Max. | ▪Each 48 concrete days or 4,000m3 |
|  |  | Mechanical strength (10% fines or Impact Value) |  | BS 812 Parts 111,112 | Min 100kN /Max 30% | ▪Each 72 concrete days or 6,000m3 |
|  |  | Los Angeles Abrasion |  | ASTM C131/ C 535 | Max 30% | ▪Each 72 concrete days or 6,000m3 |
|  |  | Moisture variation in sand-by moisture meters |  |  |  | ▪Twice daily |
|  |  | Drying shrinkage |  | BS 812 Part 120 | 0.05 % Max. | ▪At the start of the project and whenever there is a change in the source of supply. |
|  |  | Potential reactivity: aggregates |  | ASTM C295, C289 | Innocuous | ▪At the start of the project and whenever there is a change in the source of supply. |
|  |  | Cement aggregate combination |  | ASTM P214 and C227 | Max 0.1% (6 month) |  |
|  | Water | Max. chloride content |  | BS EN 1008 |  | ▪Each source and once every week for quality testing. ▪At start of the project ▪Change in the source ▪When performing initial test. ▪In case of an increase in the chloride content of the constituents or in case of doubt. |
|  |  | Prestressed concrete or grout |  |  | Max. 500mg/l |  |
|  |  | Concrete with reinforcement or embedded metal |  |  | Max. 1000mg/l |  |
|  |  | Concrete without reinforcement or embedded metal |  |  | Max. 4,500mg/l |  |
|  |  | Sulphate content |  | ASTM D512:2002 | Max 350ppm | ▪At the start of the project. ▪Whenever there is change in the source of supply. ▪When performing initial test. |
|  |  | Alkali carbonates and bicarbonates |  | ASTM D 513 | Max 500ppm |  |
|  |  | Total dissolved solids |  | BS 1377: Part3:1990 | Max 2000ppm |  |
|  |  | pH |  | ASTM D1293:1999 | min7/max9 |  |
| G | Reinforcement | Testing to confirm compliance with requirements of BS4449 |  |  |  |  |

B)  TEST ON CONCRETE (INITIAL AND PRODUCTION STAGES)

Table 14 - Test to be carried out on concrete during the initial and production stages

| Type of Tests | Applicable Standard | Limit(s) | Minimum Frequency |
| --- | --- | --- | --- |
| 1.Bleeding | ASTM C232:1999 | Max. 0.5% | ▪1 test routine representative sample during the trial mix. ▪1 test routine during every production (1st batch or load of each production day). |
| 2.Drying shrinkage and wetting expansion | BS 1881 Part 5/BS 6073 Part 1 | Max.0.05%/ Max.0.03% | ▪ 1 test routine representative sample during the trial mix only. ▪ as required by Superintendent. |
| 3.Air content | BS EN12350: P7:2000 / ASTM C231 | Max 2% | ▪1 test routine representative sample during the trial mix. Note: concrete containing entrained air: 1st batch or load of each production day until values stabilize. |
| 4.Free water/cement ratio | Ref. BS EN 206-1:2000 P.30 | As per PART 8.2 | ▪1 test routine representative sample during the trial mix. |
| 5. Workability (slump & concrete temperature, concrete flow tests) | BS 1881 Part 102:1983/ BS 1881-2 | As required by Superintendent/ BS EN 206 | ▪ 1 test routine representative sample each during the trial mix. ▪ Every truck load or Batch. |
| 6. Fresh and hardened concrete density | BS 1881 Part 107 & 114:1983 | Standard | ▪ 1 test routine representative sample during the trial mix. ▪ Same routine sampling of compressive strength of cubes during the production stage. |
| 7. ISAT (Initial Surface Absorption Test) | BS1881-208:1996/ BS 1881-5: 1970 | Refer to PART 9.1 | ▪ 1 test routine representative sample during the trial mix. |
| 8. Water absorption | BS 1881-122 | 1.6 % after 30 min. | ▪ 1 test routine representative sample during the trial mix. ▪ For every 1500m3 on 28-day old sample during the production. |
| 9. RCPT (Rapid Chloride Penetration Tests) | ASTM C1202 | 1000 coulombs Refer to PART 9.1 | ▪ 1 test routine representative sample during the trial mix. ▪ For every 1500m3 on 28-day old sample during the production. |
| 10. Water penetration | BS EN12390; Part 8: 2000 | Max. 10mm at 28 days Max. 15mm at 7 days | ▪ 1 test routine representative sample during the trial mix. ▪ For every 1500m3 on 28-day old sample during the production. |
| 11. Concrete compression tests | BS 1881 Part 108,111,116 | As per PART 10 | ▪1 sample representative at random on 8 separate occasions during the 1st five days to established standard deviation. ▪1 sample representative at random for each class of concrete in |
|  |  |  | accordance to PART 10 of the specification. ▪At least 1 sample representative from individual structural unit or part of a unit when the later is the product of a single pour. |
| 12. Splitting tensile strength | BS 1881 Part116/ ASTM C496:1996/ BS EN 12390-6 |  | ▪At least 6-cylinder samples representative for 3 at 7days, 3 at 28days at every 100 batches. ▪But at least once a week during full scale concreting operations. |
| 13. Chloride content | BS 1881: Part 124 | 0.1 -0.2 % (refer PART 12.1) | ▪ At the trial mix. ▪ Every accumulative quantity of 1500m3 of concrete with a maximum of one test per day. |
| 14. Sulphate content | BS 1881: Part 124 | 3.7 % (refer PART 12.2) | ▪ At the trial mix. ▪ Every accumulative quantity of 1500m3 of concrete with a maximum of one test per day. |
| 15. Wetting expansion | BS 6073:1981 Part1 | max 0.03% | If required by the Superintendent. |
| 16. Chloride diffusion test NT Build 492 | BS 1881-122 | Max diffusion coefficient 1.75 x 10-12 m2/s at 90 days | Min. 1 test for qualification test for each mix and as required by the Superintendents |
| 17. Tensile strength | BS 1881: Parts 117 & 118 |  | ▪ 1 test during the trial mix and at every 100 batches/once a week (ref: PART 10.3) |
| 18. Flexural Strength of concrete prism | BS EN 12390: P5 |  | ▪ 1 test routine representative during the trial mix or as required by the Superintendent |

In the case where range exceeds 15% of average for a single batch or range of 3 No. batches average exceeds 20% of overall the following additional tests shall be carried out: Tests no. 1,4,12,16,17,18.

C)  QUALITY CONTROL (DURING CONSTRUCTION)

- Additional tests shall be carried out at change of source and as required by the Superintendent.

Table 15 - Quality control test to be done during construction

| Type of Tests | Applicable Standard | Limit(s) | Minimum Frequency |
| --- | --- | --- | --- |
| Compression tests | BS EN 12390: P3 |  | refer to PART 8 and PART 10 |
| Splitting tensile strength | BS EN 12390: P6 |  | refer to PART 10 |
| Water absorption | BS 1881-122 | refer to PART 9 | For every 1500m3 on 28- day old sample |
| Water penetration | BS EN12390 | refer to PART 9 | For every 1500m3 on 28- day old sample |
| RCP test | ASTM C1202 | refer to PART 9 | For every 1500m3 on 28- day old sample |
| Drying shrinkage | BS 1881 Part 5 | max 0.05% |  |
| Wetting expansion | BS 6073:1981 part1 | max 0.03% |  |
| Air content |  |  | For concretes containing entrained air: first batch or load of each production day until values stabilize. |
| Workability (slump & concrete temperature) | BS 1881-2 |  | Per batch or truck load |
| Bleeding |  |  | Trial mix: 1 test / Prodt’n: 1 test* |
| Chloride content | BS 1881-124 |  | When performing initial test or in case of an increase in the chloride content of the constituent. |
| Sulphate content | BS 1881-124 |  | When performing initial test or in case of an increase in the chloride content of the constituent. |
| Chloride diffusion test NT Build 492 | BS 1881-122 | refer to PART 9 | Min. 1 test. Where a new cement/constituent source is used. |

* Additional tests shall be carried out at change of source and as required by the Engineer when in doubt.

# APPENDICES

Appendix 1: Drawings

- ST-HP-2030-MAR-TD-001-A July 2024

- ST-HP-2030-MAR-TD-002-A July 2024