# Managed SD-WAN Infrastructure

# RFP

Contents

1.	Introduction	1

2.	Scope of Work	2

3.	Deployment Model	4

4.	Licensing Model	4

5.	CPE Requirements	5

6.	Commercial Strategy/ Procurement PLAN	6

7.	Customer Segments and Use Cases	6

8.	Reporting and Analytics	6

9.	Support, SLA, and Maintenance	7

10.	Training and Documentation	8

11.	Vendor and Bidder qualification	8

12.	Bill of Quantity	9

13.	Killing Factors	9

14.	Pricing	10

# Introduction

MIC2 the leading Mobile Network Operator in Lebanon intends to offer SD-WAN to its B2B customers (SMBs and enterprise), and to use it to connect MIC2 branches, with a centralized orchestrator hosted in its data centers. This RFP seeks proposals from SD-WAN solution vendors to supply, install, and support all required infrastructure, including orchestrator, CPEs, licenses and all necessary integration components.  Solution need to provide flexibility and scalability to allow different offerings.

# Scope of Work

- Provide an on-premise SD-WAN orchestrator hosted in MIC2 data centers

- Offer multi-tenant architecture

- Provide all needed licenses (or license models) for SD-WAN controller, CPEs, and analytics

- Deliver hardware and software (routers, cabling, switches, racks, etc..)

- Ensure support for:

  - Multi-WAN connectivity (MPLS, broadband, LTE/5G,..) to ensure reliable and flexible data transmission

  - SLA-based traffic routing

  - VPN, firewall, web filtering

  - Zero-touch provisioning

  - Multi-tenancy and customer self-service portal

- Provide centralized management and analytics dashboard for managing network policies, configurations, and monitoring

- Centralized management using a graphical interface

- Solution must support (API interface

- Unified communications and voice support: Seamless integration and optimization for unified communication and voice applications directly on the SD-WAN routers

- Secure Web Gateway: Advanced web traffic filtering

- Firewall as a Service: Cloud-based firewall capabilities

- Intrusion Detection and Prevention (IDPS): comprehensive threat monitoring and protection

- Security IPS and URL filtering

- Support flexible application SLA (packet loss, jitter, latency,..)

- Application awareness with updated library of 3,000 + applications

- Application library should include both business applications (Salesforce, office 365, Teams, social media applications such as whatsapp, facebook,..)

- Traffic shaping:  Capability to enable or disable traffic shaping

- Threat Prevention: Device should provide next generation firewall capabilities in order to secure and filter user traffic

- Support local and cloud sandbox analysis support

- Data Loss prevention features

- SSL Deep inspection that is hardware accelerated

- Antispam capabilities over SMTP, POP3 & IMAP without external solution

- IPS Detection capabilities

  - Signature based detection using real time updated database, at least 6,000 signatures

  - Anomaly based detection that is based on thresholds

- Application control

- Solution should be able to manage local same vendor access points to provide a full secure SD-WAN and access functions for the branch offices

- Solution should provide SSL VPN tunnel mode for different operating systems

- Proposed appliance shall support built-in 2 factor authentications services and database using tokens, email and SMS

- Support dynamic IPSEC VPN

- Solution should support IPSEC site-to-site VPN and remote user VPN in transparent mode

- Solution should provide IPV6 IPSEC

- Solution should provide web filtering content without external solution, devices, or hardware.

- Solution should be able to enable or disable Web filtering per firewall policy or based on firewall authenticated user groups for both HTTP and HTTPS traffic.

- Solution should be scalable to support connecting up to 10,000 CPE devices

# Deployment Model

- Redundant deployment

- Hardware requirements for orchestrator nodes

- Network Connectivity (redundant 10G interface,..)

- Security Requirements: hardening, access control, logs

- Backup and restore capabilities

- Support horizontal scaling (multi-controller clusters) to handle increasing customer demand without downtime.

- Vendor should detail how new clusters can be added and synchronized without downtime.

- High availability across two MIC2 data center sites

- Solution must support centralized configuration and policy deployment, and monitoring of all edge devices from the orchestrator

- Solution must support scheduled updates and automated provisioning workflows

- Diagrams of deployment architecture

- Redundancy and failover capabilities

- Resilient SD-WAN architecture:

  - Branches should keep running based on their current settings even if the central controller/ management console is unreachable or offline.

  - Ability to make configuration changes on individual SD-WAN branch sites if needed

  - Centralized controller status shouldn’t affect the SD-WAN infrastructure availability , routing, security and user data traffic

  - The Management component should allow segregation of different Tenants with their respective devices with the ability of providing them with full access to manage their devices or read-only access in order to have detailed visibility.

# Licensing Model

- Price licenses per CPE, per throughput tier

- Perpetual model

- Include cost over 3 years (including support and maintenance)

# CPE Requirements

- Multiple CPE models (small, medium, high throughput)

- Indoor and ruggedized versions

- CPE technical specs (throughput, number of WAN ports, LTE failover support, Wi-Fi, etc,..)

- Support for Zero-Touch provisioning

- Should support (SIM slot and/or eSIM)

- Failover to LTE/5G via MIC2 sims

- APN provisioning

| Criteria | Basic Package | Silver Package | Premium Package |
| --- | --- | --- | --- |
| Routing & VPN | IPsec VPN (site-to-site), NAT, static routing | IPsec & SSL VPN, basic dynamic routing (RIP/OSPF) | IPsec & SSL VPN, dynamic routing (OSPF/BGP), basic path selection |
| Minimum Throughput | 100 Mbps | 200 Mbps | 500 Mbps |
| WAN Interfaces | 1× GE RJ45 | 2× GE RJ45 | 2× GE RJ45 + 1× SFP |
| LAN Interfaces | 2× GE | 4× GE | 4–6× GE |
| Security Features | Basic firewall, IP filtering | Firewall + basic antivirus, application control | Firewall + antivirus, web filtering, basic IPS |
| Zero-Touch Provisioning | Yes | Yes | Yes |
| Application Awareness | None | Basic (Layer 7 visibility for top apps) | Application identification + basic QoS |
| SD-WAN Capabilities | Static routing, manual failover | Basic dynamic path selection | Dynamic path selection + SLA monitoring |
| Management | Centralized (read-only) | Centralized with basic tenant view | Centralized with tenant access + limited API |
| Security Compliance | Basic encryption & logging | Encryption + signature updates | Encryption + threat feed updates |

# Commercial Strategy/ Procurement PLAN

MIC2 is not committing to a bulk CPE purchase upfront. Vendors must offer a flexible procurement model allowing MIC2 to purchase CPEs progressively as customers are on-boarded – it will be back-to-back (after MIC2 gets order from the B2B customers it will put the order with the bidder and payment will be done accordingly)

Vendor must provide:

- Price commitment over 3 years after PO issuance

- Stocking recommendation for startup phase

- Lead time estimates for each CPE type

- Packages recommendation and selling prices

# Customer Segments and Use Cases

MIC2 intends to address various B2B segments including but not limited:

- Retail: Branch connectivity with LTE failover

- Banking: SLA-guaranteed app performance secure data routing

- SMBs: Plug-and-play routers with bundled LTE

- Enterprises: Centralized management with API access

MIC2 will have the option to provide its customers with different packages.

Example:

- Basic Package: VPN IPSec

- Silver Package: Anti-Malware, ..

- Premium: Web filtering, IPS,  Guaranteed SLA,

# Reporting and Analytics

- Management and Monitoring: Solution must include a management and reporting module to support centralized management.

- SD-WAN solution must easily connect new SD-WAN gateways and CPE devices to the monitoring system

- Solution must allow configuration of monitoring parameters and set the threshold values

- Monitoring and troubleshooting with Multi-Tenancy support.

- Solution should include detailed logging, analytics, reporting device that parse and archive logs.

- Scalable management solutions, which can scale to manage thousands of devices from a single console.

- Visibility and Monitoring capabilities, Dashboard and Map view

- Support Zero touch provisioning.

- The proposed system shall provide high availability clustering features, enabling enhanced reliability.

- Role-based dashboards for MIC2 and enterprise admins

- Application visibility reports (usage, violations, QoS drops,..)

- Alerts (link failures, SLA breaches, threat detection,..)

- Usage stats per customer, branch, CPE, WAN link,..

- Solution must provide full zero-touch provisioning capabilities

- Devices to be deployed without on-site configuration

- Automated certificate enrollment and profile assignment

# Support, SLA, and Maintenance

- Bidder must provide 24/7 TAC and local support presence

- SLA: uptime, response, replacement timelines

- Software updates and security patches

- Quarterly health checks and roadmap sessions

- Escalation matrix and support contact points

- Proactive monitoring and rapid issue resolution

- Continuous analysis and optimization of network performance

| Crucial Priority (Severity A) (system down, threat or malware) | Immediate response by phone or email as of the escalation | Resolution time within maximum 4 hours as of the response / 24 hours a day - 7 days a week |
| --- | --- | --- |
| Average Priority (System Urgent or Severity B) | Response by phone or email within maximum 3 hours as of escalation | Resolution time within maximum 6 hours as of the response / 24 hours a day - 7 days a week |
| Not Critical (System is running with no threat) | Response by phone or email within maximum 5 hours as of escalation | Resolution by email or phone within maximum 48 hours as of the response - Site visit upon MIC2’s request |

# Training and Documentation

- Detail training plan for network and administrators of the solution

  - Include travel and accommodation for 4 engineers

- Detailed training for the enterprise B2B team

- System admin, troubleshooting, provisioning guides

- Customers onboarding playbook

# Vendor and Bidder qualification

- Bidder must have the necessary experience in deploying, implementing and managing large-scale solution. To provide at least 3 references (project details and size)

- Bidder must have local support team to answer all the complaints, queries in addition to replace the CPEs if needed

- Vendor must provide 3 years roadmap for the end-to-end solution

- Vendor must be in Gartner magic quadrant

# Bill of Quantity

- Bidder must provide detailed breakdown of the prices for the orchestrator at MIC2 premises with all the necessary integration elements and licenses

- Bidder must provide the prices for the CPEs for the requested 3 models that include all the basic requirements:

  - Small  (0-500 CPE)

  - Medium (0-200 CPE)

  - High throughput (0-100 CPE)

- Bidder need to provide license cost per feature example but not limited:

  - Firewall

  - Antimalware

  - Security

  - App Control

  - Web filtering

  - Traffic Shaping

  - Cloud Storage,..

- Bidder must provide breakdown for installation fees for the CPEs per CPE

- Bidder need to provide the configuration fees for the CPEs per CPE

- Vendor to specify if features like NGFW, IPS , Anti-Malware, Web Filtering, SSL inspection, Cloud Sandbox, and others are:

  - Licensed per CPE model

  - Licensed per throughput or per tenant

  - Available as bundles or per CPE purchase

  - Require cloud connectivity or hosted locally

# Killing Factors

- Vendor must ensure seamless integration

- At least 3 similar deployment for the solution in similar environment

- Bidder must have local support team to answer all the complaints, queries in addition to replace the CPEs if needed

- Vendor must provide 3 years roadmap for the end-to-end solution

# Pricing

- Break Down of all the prices must be provided with all features and licenses

- Unbilled price list to be provided in the technical offer

- Vendors are required to provide a detailed cost breakdown, including hardware, software, license, installation, maintenance, and support services. Latest Software version for all modules should be available during acceptance.

- MIC2 can chose any or all of the features provided.

- Any license or feature mentioned in the compliancy sheet and/or in the technical response will be considered as part of the solution/ additional features that are not included in the solution should be clearly mentioned